Themis Software meeting - Items for Scientists

| 3099f | Revise the calibration code for the particle data types (ESA, perhaps also SST or MOM?) to look for spacecraft potential in L1 FIT, if the moments are decimated | 3/2/2010 | Jim L | 1 | Jim McFadden | pre-requisite 3099c test data (Jim L) and awaiting FSW patch from PRH to create test data for 3099-d - in progress; Jim McFadden needs to vet and make a change in one of his programs |
| --- | --- | --- | --- | --- | --- | --- |
| 3090 | Then one needs to correct the particle distributions to be introduced in the same orientation. I.e., should use the corrected spinphase upon being introduced (on the fly). This means the particle loading software needs to read the above difference between corrected and uncorrected spinphase, and apply a rotation to the distribution functions upon reading. This should be done with a keyword first, and routinely when we get into orbit (by which time the shadow spinphase determinations will hopefully be automated). I think McFadden is the one to do this but he can also assign it. | 2/22/2010 | Vassilis | 1 | Jim McFadden | 3089 is prerequisite - Vassilis 4/23: un-rotate the V’s from the dphi and the onboard spinphase) and compare with the ground-processed V from the distribution functions. ESA and MOM code to use new features of spin model. In progress and being done in conjunction with 3099f. Jim McFadden needs to review. |

| 3025a | Determine scales and offsets for correcting on-board moments to match ground based moments during solar wind mode. |  |  |  | Pat | scaling factors locked down for solar wind mode, but it turns out that we also need a different set of scales and offsets when not in solar wind mode. Found scaling factors during non-solar wind mode that were not expected. Trying to contact |
| --- | --- | --- | --- | --- | --- | --- |

3055c - 3055j Davin to review and see if further changes needed					Pat

3055k - Davin  SST calibration parameters (meeting 7/26 with Davin, Vassilis and Pat)

Davin:

| 4070 | "Add support to thm_cotrans, ssl2gse, and ssl2dsl for /pseudo_dsl keyword. If keyword is present, look for and apply eclipse delta_phi corrections as appropriate." | 6/1/2010 | Jim L | 1 | Jim L | thm_load_fit and thm_load_mom, add keyword. In progress - send email waiting for Davin to describe how the tensor quantities in thm_load_mom need to be handled. Will ping him today. Might be able to finish thm_load_fit changes today (while waiting on SPDF feedback for 4074c1) and start testing. |
| --- | --- | --- | --- | --- | --- | --- |