THEMIS

PERFORMANCE ASSURANCE

IMPLEMENTATION PLAN

(PAIP)

Contract Number: NAS5-02099

UCB Approvals:	____________________________________

Dr. Vassilis Angelopoulos, THEMIS PI

____________________________________

Peter R Harvey, THEMIS Project Manager

____________________________________

Ron Jackson, THEMIS Mission Assurance Mgr

GSFC Approvals:	____________________________________

Frank Snow, THEMIS Mission Manager

# Document Revision Record

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| - | 6/09/2003 | Initial Draft | - |
| A | 9/30/2003 | Removed Appendix B Added References to FGM, SCM & Probe PAIPs | - |

# Distribution List

Ron Jackson, UCB

Peter Harvey, UCB

Vassilis Angelopoulos, UCB

Frank Snow, GSFC

Table of Contents

Document Revision Record	2

Distribution List	2

1.	GENERAL	6

1.1	Basis and Scope of the Plan	6

1.2	General Requirements	6

1.3	Use of Previously Designed, Fabricated, or Flown Hardware	6

1.4	Flow-Down of PA Requirements	7

1.5	Surveillance	7

1.6	SR&QA Verification	7

1.7	Applicable Documents (Appendix A)	7

2.	ASSURANCE REVIEW REQUIREMENTS	7

2.1	General Requirements	7

2.2	GSFC Flight Assurance Review Requirements	7

2.3	Flight Assurance Review Program	8

3.	PERFORMANCE VERIFICATION REQUIREMENTS	8

3.1	General Requirements	8

3.2	Documentation Requirements	8

3.2.1	Verification Matrix	8

3.2.2	Environmental Test Plan	8

3.2.3	Verification Test Procedures	8

3.2.4	Verification Test Report	9

3.3	Demonstration of Failure-Free Operation	9

3.4	Comprehensive Performance Test	9

4.	SAFETY	9

4.1	General	9

4.2	System Description and Safety Assessment Report	9

4.2.1	Preliminary Safety Assessment	9

4.3	Procedure Approval	10

4.4	Safety Noncompliance Requests	10

4.5	Support for Safety Working Group Meetings	10

4.6	Safety Data Package, Launch Site Safety Plan, and Orbital Debris Assessment	10

5.	EEE PARTS REQUIREMENTS	10

5.1	General Requirements	10

5.2	Parts Selection	11

5.2.1	EEE Parts Identification List	12

5.3	Other Parts	12

5.3.1	Magnetic Devices	12

5.3.2	Plastic Encapsulated Microcircuits (PEMs)	13

5.3.3	Units and Subassemblies	13

5.3.4	Field Programmable Devices	13

5.3.5	PIND Testing	14

5.3.6	Destructive Physical Analyses (DPA)	14

5.4	Derating	14

5.5	Radiation Tolerance	14

5.6	Alerts	14

5.7	Parts Age Control	14

5.8	Parts Control Board	14

6.	MATERIALS AND PROCESSES CONTROL REQUIREMENTS	15

6.1	Selection Requirements	15

6.1.1	Compliant Materials	15

6.1.2	Noncompliant Materials	15

6.1.3	Conventional Applications	15

6.1.4	Nonconventional Applications	15

6.1.5	Inorganic and Metallic Materials	15

6.1.6	Non-metallic Materials	15

6.1.7	Fasteners	15

6.1.8	Lubricants	16

6.1.9	Consideration in Process Selection	16

6.1.10	Shelf Life Controlled Items	16

6.1.11	Magnetics Compatibility	16

6.2	Documentation	16

6.3	GIDEP Alerts	16

7.	DESIGN ASSURANCE AND RELIABILITY	16

7.1	Requirements	16

7.2	Implementation	16

7.3	Failure Modes and Effects Analysis	17

7.4	Limited Life Items	17

7.5	Trending	17

8.	QUALITY ASSURANCE REQUIREMENTS	17

8.1	Support of Design Reviews	17

8.2	Configuration Management	17

8.3	Identification and Traceability	17

8.4	Procurement Controls	17

8.4.1	Purchased Raw Materials	18

8.4.2	Age Control and Limited-Life Products	18

8.4.3	Inspection and Test Records	18

8.4.4	Purchase Order Review	18

8.4.5	Re-submission of Non-conforming Articles or Materials	18

8.5	Receiving Inspection	18

8.6	Fabrication Control	18

8.6.1	Manufacturing Certification Log	18

8.6.2	Workmanship	18

8.6.3	Process Control	19

8.7	ESD Control	19

8.8	Non-conformance Control	19

8.8.1	Discrepancies	19

8.8.2	Failures	20

8.8.3	Alert Information	22

8.9	Inspections and Tests	22

8.9.1	Inspection and Test Records	22

8.9.2	Printed Wiring Boards Inspections and Tests	22

8.10	Metrology	22

8.11	Handling, Storage, Marking, Shipping, Preservation, Labeling, and Packaging	23

8.11.1	Handling	23

8.11.2	Shipping	23

8.12	Government Property Control	23

8.13	End Item Acceptance	23

8.14	Ground Support Equipment	23

9.	CONTAMINATION CONTROL	23

9.1	Project Requirements	23

9.2	THEMIS Concerns	23

9.3	Control Plan	23

9.4	THEMIS Requirements on Spacecraft I&T and Operations	24

10.	SOFTWARE ASSURANCE	24

10.1	General	24

10.2	Software Development	25

10.3	Documentation	25

10.4	Software Design Reviews	25

10.5	Configuration Management	26

Appendix A - APPLICABLE DOCUMENTS	26

# GENERAL

## Basis and Scope of the Plan

This document has been written in response to the MIDEX Safety, Reliability & Quality Assurance Requirements document (GSFC-410-MIDEX-001).

The THEMIS project is a NASA Explorer mission which will launch a constellation of five micro-satellites in mid-2006. Flying in synchronous orbits within the earth’s magnetosphere, the satellites will measure the particle processes responsible for eruptions of the aurora. As the prime contractor for THEMIS, the University of California at Berkeley will provide the systems engineering, flight instrumentation, ground-based imagers, mission operations, and Performance Assurance. Swales Aerospace will provide spacecraft buses. Key international partners include teams from Canada, France, Germany, Austria and the Netherlands.

As a PI mode mission, the Explorer’s Office is responsible for oversight of UCB in the proper development of the spacecraft and ground systems. The Explorer’s Office is responsible for arranging a suitable Launch Vehicle and launch site support through KSC. These efforts are coordinated by the Mission Manager.

UCB will provide a Project Office which contains at least the following roles and responsibilities: Project Manager (PM), Mission Systems Engineer (MSE), Lead Mechanical Engineer (LME), Mission Assurance Manager (MAM), Financial Manager (FM), Contracting Officer (CO). In this document, unless otherwise stated, the term Project refers to the THEMIS Project office at UCB, and the terms GSFC and EO refer to the GSFC Explorers Office.

UCB and all US institutions funded by the THEMIS project will meet the plan described below. Non-NASA-funded institutions (foreign Co-Investigators) shall meet the Performance Assurance requirements called out by their funding agency.

## General Requirements

The PI for THEMIS will establish an organized program which will demonstrate that the design meets the functional requirements, including margins, has been manufactured properly and that it will operate properly in association with other project components. This will be accomplished by conducting analyses, tests and inspections.

The performance assurance program will encompass flight equipment, critical GSE, Flight Software and spare flight equipment. This plan will be used by the PI and all Co-investigators who fabricate or test such equipment. This plan does not apply to ground support, mission operation; data analysis equipment or software except where specifically called out.

Critical Ground Support Equipment (GSE) includes that parts of the THEMIS GSE that interfaces directly to the flight hardware in such a way that failure or incorrect operation could damage the flight hardware.  The requirements of this PAIP only apply to such GSE as required to ensure the safety of the flight hardware.

## Use of Previously Designed, Fabricated, or Flown Hardware

Any previously designed or fabricated section of the hardware used on THEMIS will be subject to the PA requirements of this PAIP.

## Flow-Down of PA Requirements

The PI will ensure that all vendors and subcontractors who supply hardware for the THEMIS instrument suite will meet applicable PA/QA requirements.

The following documents are included as part of this PAIP:

[1] THM_PA_002	FGM Performance Assurance Implementation Plan

[2] THM_PA_003	SCM Performance Assurance Implementation Plan

[3] THM_PA_004	THEMIS Probe, Probe Carrier and Probe Carrier Assembly Performance Assurance Implementation Plan and EEE Parts Program

## Surveillance

The work activities, operations, and documentation performed by NASA-funded THEMIS institutions and their suppliers are subject to evaluation, review, audit, and inspection by government-designated representatives from GSFC, the Government Inspection Agency (GIA), or an independent assurance contractor (IAC).  GSFC will delegate in-plant responsibilities and authority to those agencies via a letter of delegation, or the GSFC contract with the IAC.

The PI and/or subcontractor, upon request, shall provide government assurance representatives with documents, records, and equipment required performing their assurance and safety activities.  The PI shall also provide the government assurance representative(s) with an acceptable work area within developer facilities.

Work activities performed at foreign co-I institutions are evaluated regularly with internal inspections and on an as-needed basis by external inspectors.

## SR&QA Verification

The PI will provide GSFC or their representative with any documents and records outlined in the PAIP upon request.

## Applicable Documents (Appendix A)

To the extent referenced herein, applicable portions of the documents and revision levels listed in Appendix A form a part of this document.

# ASSURANCE REVIEW REQUIREMENTS

## General Requirements

The PI will support a series of system-level design reviews that are conducted by an independent review team. The reviews will cover all aspects of the flight hardware, critical ground support hardware, flight software, and operations.

In addition, the PI will support informal subsystem-level engineering peer reviews.  These peer reviews will be coordinated with GSFC such that GSFC may provide addition technical expertise.  Resulting action items shall be tracked by the Project office.

## GSFC Flight Assurance Review Requirements

For each System-level review, the Project will :

- Provide oral presentations of materials from each sub-system team and operations;

- Provide preliminary copies of the viewing material one week before the review;

- Provide a final version at the time of the review;

- Support splinter review meetings resulting from the major review.

- Produce written responses to recommendations and action items resulting from the review.

## Flight Assurance Review Program

The Project will perform the following System-Level design reviews:

- A Preliminary Design Review (PDR) which is to occur when the preliminary design is completed;

- A Critical Design Review (CDR) which occurs before the bulk of the flight fabrication begins;

- Pre-Environmental Review (PER) which occurs after the space probe is complete and before the full environmental tests are performed;

- A Pre-Shipment Review (PSR) which occurs prior to shipping the probe carrier assembly (PCA) to the launch site for integration to the launch vehicle.

Informal delta-PERs will be held for probes 2-5 to determine readiness for environments, and discuss changes to the environmental flows if needed.

# PERFORMANCE VERIFICATION REQUIREMENTS

## General Requirements

A Performance Verification program will be conducted to ensure that the flight hardware meets the mission requirements. The program consists of a series of functional demonstrations, calibrations, analyses, physical measurements, and environmental tests which simulate the environments encountered during handling and transportation, pre-launch, launch and flight.  All flight hardware will comply with the requirements of this PAIP. In the event that spare instrumentation is used, it will be verified prior to flight.

The applicable environmental verification program is described in GEVS-SE, as modified by environmental test specifications provided by GSFC based on Launch Vehicle Contractor provided information.

## Documentation Requirements

The THEMIS Project Manager will be responsible for managing the collection and distribution of verification documentation.  This documentation will include a Verification Matrix, Environmental Test matrix, Verification Procedures, and Verification Reports.  Verification documentation shall be available on request, and shall be summarized at design reviews.

### Verification Matrix

The Verification Matrix shall show the flow-down of science and mission requirements and the method of verification for each requirement.

### Environmental Test Plan

The Environmental Test Plan shall summarize the environmental tests to be performed at each level of assembly.  Test levels, cycles, and special provisions will be called out.

### Verification Test Procedures

Verification Test Procedures will be developed for all tests conducted at the component level and above. Such procedures will be at least a lab notebook level of formality.

### Verification Test Report

A test report will be generated for each test at the component level and above. This report will show the degree to which the test objectives were met how well the data correspond to the expected results, and any other significant findings.  They will include as-run procedures and test data.  Such reports shall be at least a logbook level of formality.

## Demonstration of Failure-Free Operation

Prior to launch, all spacecraft probes shall have demonstrated trouble-free operation for at least 100 hours without significant change to the hardware.  A “Significant Change” generally means any work on the item that calls into question the validity of the previous test results, such as a repair or part replacement.

## Comprehensive Performance Test

A set of Comprehensive Performance Tests (CPT) shall be used before, in some cases during, and after environmental tests to verify the end-to-end performance and functionality of the subsystem.   These tests shall also be used at spacecraft integration and test as a validation of instrument functionality.  Some instrument performance characteristics can only be verified in vacuum or at a special facility, but the CPT will be designed to verify as much as possible of the system, and provide a baseline against which to compare subsequent test results.

# SAFETY

## General

The PI shall plan and implement a system safety program that accomplishes the following:

- Identifies and controls hazards to personnel, facilities, support equipment, and the flight system during all stages of project development.  The program shall address hazards in the flight hardware, associated software, ground support equipment, and support facilities.

- Ensure the design meets the system safety requirements of EWR 127-1 "Range Safety Requirements Eastern and Western Range". For the Eastern launch site (KSC) and KHB 1710.2D, "Kennedy Space Center Safety Practices Handbook".

- Ensure the design meets the baseline industrial safety requirements of the institution, EWR 127-1, as well as any special contractually imposed mission unique obligations.

## System Description and Safety Assessment Report

The THEMIS Concept Study Report includes a detailed description of the system down to the subsystem level.  This document shall serve as a baseline for the THEMIS Mission Assurance Manager (MAM).  A preliminary assessment of the THEMIS instrument's compliance with the requirements of section 4.1 follows.  The PI shall continue to identify, analyze, and minimize hazards throughout the development effort.  All hazards affecting personnel, launch vehicle hardware, or the spacecraft shall be identified and brought to the attention of the MAM.  A synopsis of the on-going safety analysis, consistent with the maturity of the subsystem design, shall be part of each subsystem presentation at peer level and system level independent design reviews.

### Preliminary Safety Assessment

The only unusual identified hazards related to the THEMIS spacecraft development and test are:

- High Voltage: The instrument contains a number of high voltages supplies, as high as 3000V.  There shall be no exposed high voltage.  The supplies shall be resistively current limited on the output.  The instrument can be damaged by inadvertent operation of the supplies in air.  This risk is mitigated by the use of red tag disable plugs and/or green tag enable plugs, multiple series commanding interlocks, plus appropriate hazardous procedure interlocks.

- Radiation Sources: Low level radiation sources shall be used during the calibration and test of the instruments.  These sources do not fly with the instrument, but will be used during instrument and spacecraft functional tests.  Qualified personnel using appropriate handling procedures will handle these sources.

- Non-Explosive Actuators (NEA): The instruments shall contain in-flight deployable covers using non-explosive actuators.  The THEMIS boom deployment is also planned to use NEAs.  None of these actuations are expected to present a personnel hazard.  Enable or disable plugs will be used to prevent unintended actuation that might expose a detector to contamination or damage the boom by deployment in 1G.

## Procedure Approval

The PI shall submit, in accordance with the contract schedule, all ground operations procedures to be used at GSFC facilities, other integration facility, or the launch site.  All hazardous operations as well as the procedures to control them shall be identified and highlighted.  All launch site procedures shall comply with the applicable launch site safety regulation.

## Safety Noncompliance Requests

When a specific safety requirement cannot be met, the PI shall submit to GSFC an associated safety noncompliance request that identifies the hazard and shows the rationale for approval of a noncompliance, as defined in the applicable launch site safety regulation.

## Support for Safety Working Group Meetings

The PI shall provide technical support to the THEMIS Mission Assurance Manager for safety working group meetings, when necessary.

## Safety Data Package, Launch Site Safety Plan, and Orbital Debris Assessment

The spacecraft contractor shall develop the Safety Data Package, the Launch Site Safety Plan, and the Orbital Debris Assessment documents under contract to the PI.  The PI shall provide input as required concerning the THEMIS instruments and its related ground activities that effect safety.

# EEE PARTS REQUIREMENTS

## General Requirements

UCB will conduct a parts control program covering the selection, procurement, and acceptance of EEE parts used on the THEMIS flight subsystems and critical GSE.  This section of the PAIP shall serve as the THEMIS Parts Control Plan.

The UCB Mission Assurance Manager is responsible for implementation of the parts control program.  Parts selection and screening plans will be done by various engineers working on the project, with final approval by the Parts Control Board. Parts testing, when required, will be performed by engineers assigned to the project, and/or outside vendors.

Note that non-NASA funded institutions providing hardware to the THEMIS instrument suite will use the parts control program mandated by their funding institution.  It is assumed that the parts quality level shall be similar to that imposed in this section on the NASA-funded institutions.  At a minimum, sections 5.2.1 and 5.5 shall apply to all flight hardware.

## Parts Selection

Parts will be selected and processed in accordance with GSFC Specification GSFC-311-INST-001 for level 3 quality level.  Parts will be preferably selected from the following sources:

- Parts listed in the GSFC Preferred Parts List (PPL), or the NASA Standard Electrical, Electronic, and Electromechanical (EEE) Parts List (NSPL), MIL-STD-975.  Where differences in requirements exist between the PPL and the NSPL, the PPL should take precedence.  Parts should be procured in accordance with the appropriate specification designated for that part.

- MIL-M-38510, Class B or better microcircuits procured from a Qualified Products List (QPL) supplier. Additional PIND testing is required for class B parts.

- MIL-I-38535, Class Q or better microcircuits procured from a Qualified Manufacturers' List (QML) supplier. Additional PIND testing is required for class Q parts.

- MIL-H-38534, Class H or better hybrid microcircuits procured from a Qualified Manufacturers' List (QML) supplier. Additional PIND testing is required for class H parts, plus precap visual or DPA.

- Standard Military Drawing (SMD) microcircuits procured from an authorized supplier as listed in the SMD.

- Microcircuits compliant with paragraph 1.2.1 of MIL-STD-883 and procured from manufacturers having QPL or QML status for parts of the same technology.  Parts procured from manufacturers without QPL or QML status should be procured with lot specific or generic Group C Quality Conformance Inspection (QCI) data within one year of the lot date code of the parts being procured.  MIL-STD-883 compliant microcircuits shall be subjected to PIND testing and DPA.  Screening data shall be obtained with the parts.

- Manufacturers' in-house high reliability processed parts provided all screening tests listed in Appendix C of the PPL have been satisfied.  The high reliability processes flow should be that formally documented by the manufacturer in cases where changes would require a revision to the flow documentation.  Tests not included in the manufacturer's high reliability flow must be performed by the manufacturer, an independent test facility, or by the developer.  Parts procured in this section should be procured with lot specific or generic Group C Quality Conformance Inspection (QCI) data within one year of the lot date code of the parts being procured.  If not included in the manufacturer's high reliability test flow, the parts should be subjected to PIND testing and DPA.  Screening data shall be obtained with the parts.

- MIL-S-19500, JANTX, JANTXV and JANS semiconductors procured from a QPL listed supplier.  It is preferred that semiconductors be procured to JANTXV level or better.  Any semiconductor that has an internal cavity should be subjected to PIND.

- Established Reliability (ER) passive components procured from a QPL listed supplier for the appropriate military specification.  Only ER parts within the minimum and maximum value ranges specified in the PPL should be considered acceptable.  50V ceramic capacitors used in applications <10VDC may require steady state humidity low voltage testing per 311-Inst-001A; refer to PCB.  CSR09 and CSR13 capacitors require surge current testing per 311-Inst-001A and PPL.

- Parts procured to a GSFC S-311 specification from a GSFC approved source.

- Relays procured to MIL-R-39016, MIL-R-83536, MIL-R-6106, or MIL-R-83536 procured to failure rate M or better.

### EEE Parts Identification List

A list of all parts used in the THEMIS instrument suite flight hardware will be maintained in electronic format by UCB.  The list shall be available to THEMIS Project.  This list will include, as applicable:

- Generic part type

- Control specification

- Part number

- Manufacturer

- Lot # and/or lot date code

- Where used

- Total quantity used

- Screening required, status

An as-built parts list shall also be maintained, typically in the form of as-built PWB loading documentation.  Documentation shall include lot date code and, where applicable, serial number for each part is loaded on the board.  Photographic documentation may also be used (see section 8.3).  The as-built documentation shall also include lot/serial information on any replacement parts.

## Other Parts

Other parts, not on any of the documents listed in section 5.2, and including Custom circuits (ASICs, MCMs etc.) will be purchased or screened in accordance with GSFC Specification GSFC-311-INST-001 for Level 3 quality level.  The Parts Control Board shall review and approve use of such parts and their application, procurement and screening plans/specifications.

For devices not covered under GSFC-311-INST-001, they shall be screened in accordance with the nearest applicable GSFC specification or military specification.

Devices with interval cavities shall be subject to PIND testing.  Devices shall either have a pre-cap visual or a DPA performed.

Screening specifications shall fully identify the tests to be performed, the test conditions, failure criteria, and lot rejection criteria.  For lot acceptance or rejection, the Percentage of Defectives Allowable (PDA) in a screened lot shall be in accordance with that required in the closest military part specification.

Part screening results, failed parts, parts subject to destructive tests, and non-flight parts subjected to non-destructive tests shall be archived and retained for the life of the mission, unless otherwise directed by the PCB.

### Magnetic Devices

Transformers and inductors will be manufactured at UCB using magnetic components purchased from Magnetic, Inc. and Phillips, to commercial specifications, and MWS Heavy Armored Polythermaleze wire (HAPT), also purchased to commercial specifications.  Parts and wire will be carefully visually inspected before and after winding.  Units may be potted using approved materials at UCB.  Electrical tests and measurements will verify correct operation of the completed units as follows:

- DC Winding resistance

- Winding Inductance

- Turns ratio or Voltage Ratio

- Polarity

- Dielectric Withstanding Voltage

- Induced Voltage

- Self Resonant Frequency

- Visual

Qualification shall be based on similarity; the same procedures and personnel will be used that have fabricated magnetic at UCB for several successful NASA missions.

### Plastic Encapsulated Microcircuits (PEMs)

The use of PEMs shall be restricted to applications where no similar high reliability hermetically sealed device exists.

Each lot of PEMs shall be qualified by subjecting samples to Highly Accelerated Stress Testing (HAST) or Steady State Temperature Humidity Bias Life testing in order to qualify and assess overall package integrity of the lot.

All PEMs shall be subjected to screening to eliminate random early life failures, including temperature cycling, burn-in, electrical measurements and C-SAM (C-Mode Scanning Acoustic Microscopy.

The PCB shall approve the details of the qualification and screening procedures.

PEM may be susceptible to moisture intrusion, and shall be protected during shipping, storage, and assembly. The flight units shall be packaged in a Moisture Barrier Bag with desiccant and humidity indicator cards.

### Units and Subassemblies

Function of units or assemblies that are purchased, as "off the shelf" hardware items shall be analyzed for mission criticality.  When loss of off the shelf units does not compromise mission success, on a case-by-case basis, these units may be considered exempt from the parts control requirements of this section, subject to approval of the program office and the parts control board.  However, additional unit level testing such as thermal cycling or thermal vacuum testing, may be directed by the PCB or project in lieu of additional part level screening.

When failure of such units represents significant compromise to mission success, an analysis of the parts used within the units shall be performed.  The parts shall be evaluated for screening compliance to GSFC 311-INST-001, established reliability level, and include a radiation analysis.  Pending the results of this investigation, units may be required to undergo modification for use of higher reliability parts, or Rad hard parts.  All upgrade parts shall be subject to PCB approval.

Modifications such as additional shielding for radiation effectiveness or replacing radiation soft parts for Rad-Hard parts may be recommended.

### Field Programmable Devices

Field programmable devices such as PROMs that cannot be fully tested prior to programming shall be re-screened following programming, including burn-in and final electrical functional.  Note that Actel FPGAs do not require re-screening beyond that performed by the programmer, per the manufacturer’s recommendation:

http://www.actel.com/appnotes/ppbi_dc.pdf

### PIND Testing

The PCB for approval must review parts from lots exceeding 25% PIND failure.  This includes PIND testing for devices covered by section 5.2 also.

### Destructive Physical Analyses (DPA)

DPA procedures, sample size and criteria shall be performed per GSFC specification S-311-M-70 (TBR), Destructive Physical Analysis.  For small procurements, small lot sampling per 311-M-70 may be used.  The PCB on a case-by-case basis shall consider variation to the DPA sample size requirements, due to quantity used in flight, application criticality, part complexity, availability or cost.

## Derating

All flight parts will be derated to the levels of PPL, Appendix B.

## Radiation Tolerance

All EEE parts shall be selected to meet their application design requirements in the predicted radiation environment, including TID and SEE.

Parts shall have a TID tolerance of 33 Krads or more based on manufacturers data sheet, demonstrated technology hardness, or lot testing.  Shielding or special packaging may be used to achieve the desired tolerance.  The dose figure of 33 Krads assumes 5-mm aluminum shielding or equivalent for a 2 year nominal mission (RDM=1).

Parts shall be SEL-immune to a LET of >80 MeV-cm2/mg, or else shall be protected against damage by a protection circuit.  Parts that may affect critical functions that could damage the instrument shall be SEU-immune, or else shall use a Triple-Modular-Redundancy scheme to avoid any single SEU causing a failure.  Parts shall meet these criteria based on manufacturers data sheet, demonstrated technology hardness, or lot testing.

The rational/analysis/test data demonstrating the radiation tolerance of each part shall be documented.

## Alerts

The MAM will respond to GIDEP Alerts and NASA Advisories forwarded by GSFC, and determine if any flight hardware or parts inventory is affected.  In-coming EEE parts into the THEMIS flight stock will be screened against the GIDEP database.  Any parts for which there is an open Alert will either be removed from the flight stock and/or hardware or referred to the PCB.

## Parts Age Control

Integrated Circuits and Semiconductors that have a lot date code or screening record older than 7 years at the time of installation into PWBs shall be subjected to a room temperature re-screen and sample DPA as necessary per PCB recommendation.

## Parts Control Board

A Parts Control Board shall be set up to approve the screening and use of non-standard parts, re-testing due to age, failure history, GIDEP Alerts, or other reliability concerns, and other parts related issues called out above.  The PCB shall include THEMIS Parts Engineer and Mission Assurance Manager, in addition to an THEMIS subsystem lead engineer, QA, or parts engineer (depending on what system is involved) and the THEMIS Project Manager.  Documentation shall consist of the e-mail exchanges and the annotated PIL and related documents attached to such e-mail.  The PIL shall be the summary of the agreements reached by the PCB, including approval, screening requirements, etc.

# MATERIALS AND PROCESSES CONTROL REQUIREMENTS

## Selection Requirements

### Compliant Materials

Compliant materials will be used in the fabrication of flight hardware to the extent practicable.  In order to be compliant, a material must be used in a conventional application and meet these ELV criteria:

1. Hazardous materials requirements including flammability, toxicity, and compatibility as specified in EWR 127-1 (Sections 3.10 and 3.12).

2. Vacuum Outgassing requirements as defined in paragraph 6.1.6.

3. Stress corrosion cracking requirements as defined in 6.1.5.

### Noncompliant Materials

A material that does not meet the requirements of section 6.1.1 or meets the requirements of section 6.1.1 but is used in an unconventional application shall be considered to be a noncompliant material.  A Materials Usage Agreement (MUA) and/or a Stress Corrosion Evaluation Form are submitted to the GSFC THEMIS Project for concurrence for all noncompliant materials.

No pure Tin shall be used due to “Tin Whisker” problem.

### Conventional Applications

Conventional applications or usage of materials is the use of compliant materials in a manner for which there is extensive satisfactory aerospace heritage.

### Nonconventional Applications

The proposed use of a compliant material for an application for which there is limited satisfactory aerospace usage shall be considered a non-conventional application.  In that case, the material usage shall be verified for the desired application on the basis of test, similarity, analyses, inspection, existing data, or a combination of those methods.  The proposed use of a material in a non-conventional application requires that a MUA be prepared and reviewed by GSFC.

### Inorganic and Metallic Materials

A MUA and stress corrosion cracking evaluation form shall be submitted to Project for each material usage that does not comply with the MSFC-SPEC-522 stress corrosion cracking requirements.

### Non-metallic Materials

Materials will be noncombustible or self-extinguishing as much as possible. The outgassing properties of organic materials will be considered in their selection.  When tested to ASTM E-595, compliant materials will have less than 1 percent total mass loss and less and 0.1 percent collected volatile condensable mass.

Solithane conformal coat materials shall be avoided, with Uralane being preferred.

### Fasteners

Fasteners shall be selected and screened in accordance with 541-PG-8072.1.2.

### Lubricants

No lubricants are expected to be used.  Should lubricants be required, they will be included in the Materials list approved by Project.

### Consideration in Process Selection

Manufacturing processes will be selected so as to minimize changes to the material's properties.

### Shelf Life Controlled Items

Polymeric materials with an uncured limited shelf life will be identified with manufacturing data and storage conditions. Regular purchases of limited shelf life materials will be planned to assure that current date code materials are always available. Out of date materials will not be used on flight hardware.

Any other limited life material will be identified in the Limited Life Items list discussed in section 7.4.

### Magnetics Compatibility

Materials shall be selected non-magnetic wherever possible.  Use of any magnetic materials shall be reviewed with Project and the Magnetometer Co-Investigator to determine acceptability of use.

## Documentation

Documentation for materials and processes control will include:

- Engineering Drawings for materials application

- Materials and Processes List

The material list will be available to the Project.

## GIDEP Alerts

Materials selection and usage records sufficient to determine applicability of any Government Industry Data Exchange Program (GIDEP) alert shall be maintained.  These records shall typically be kept in the form of material Certificates of Compliance in the shop traveler or QA records.  Historical GIDEP Alert status shall be reviewed on receipt of materials, and new GIDEP alerts shall be checked against flight stock.  Any material against which there is an open Alert shall be removed from flight stock or refereed to the MRB.

# DESIGN ASSURANCE AND RELIABILITY

## Requirements

The THEMIS subsystems and associated test equipment will be designed to:

- Function properly during their expected lifetimes,

- Minimize or eliminate human-induced failures,

- Permit ease of assembly, test, fault-isolation, repair, and maintenance without compromising performance, reliability, or safety aspects.

## Implementation

The MSE shall ensure that system-level design and trade studies include reliability considerations. At the subsystem level, lead subsystem engineers will be responsible for reliability issues concerning their subsystem, and shall bring to the attention of the MSE any reliability concerns outside the scope of their allocated resources.

## Failure Modes and Effects Analysis

An spacecraft FMEA shall be performed by the MSE to identify failures whose impacts can be lessened.  A less formal analysis shall be performed by subsystem engineers, as part of the design process to identify likely failure modes and mitigation schemes.

## Limited Life Items

Limited life items shall be identified and avoided when possible.  The MSE shall compile a list of limited life items with input by the subsystem engineers. This list shall be available to GSFC.  Limited life items include all hardware that is subject to degradation due to age, operating time, or cycles, such that its expected useful life is less than twice the required life, when fabrication, test, storage, and mission operation are combined.  The MSE shall compile a record of total operating times for these items.

## Trending

A set of measurable parameters that relate to performance stability shall be identified for each instrument.  These parameters shall be measured and tracked as part of the standard functional testing starting at the instrument level.  Recording of these parameters shall be part of the standard Comprehensive Performance Test (CPT).  As part of the analysis of the results of each CPT, changes in the trending parameters shall be analyzed.  This process shall continue through spacecraft I&T and post-launch mission operations.

# QUALITY ASSURANCE REQUIREMENTS

## Support of Design Reviews

QA issues and the status of the QA program will be addressed in the reviews identified in section 2.3.

## Configuration Management

Configuration management is described in the THEMIS configuration management plans.

It is the responsibility of the design engineer to verify that the as-built configuration (as described in the traveler) is correct (compared to as-designed) prior to integration of an item into the next level of assembly.

## Identification and Traceability

Part numbers will be provided on each sub-assembly or PWB.  If sub-assemblies or assemblies are not unique, serial numbers will be used to identify them.

Mechanical parts will be serialized when they are not fully interchangeable. Significant sub-assemblies (such as a sensor assembly) will be serialized for traceability.

Records will be maintained to support a trace of any non-interchangeable part or material to the board or unit in which it was placed.  Parts from a given manufacturer with the same lot-date-code and screening history are considered to be interchangeable.  Similarly, any board or unit will be traceable backwards to the parts or materials from which it was built. Thus, if an ALERT were to identify a problem part, THEMIS could determine all places where the part exists in the instrument.

## Procurement Controls

The following procurement controls shall be imposed on all flight unit parts and materials purchases.

### Purchased Raw Materials

Purchase orders for raw materials will include a requirement for the results of physical and chemical tests, or a certificate of compliance.  Exceptions shall be approved by the GSFC via a waiver request, based on evidence of the acceptability of the material for the intended use.

Suppliers of materials will be requested to make acceptance test results available.

### Age Control and Limited-Life Products

Records will be kept on products having characteristics of degradation with use or age. Records will note date, when useful life was initiated, and date when life expires.

### Inspection and Test Records

UCB will require where necessary that suppliers maintain inspection and test records. Records that are to be provided with the deliverable item will also be specified.

### Purchase Order Review

All purchase orders for flight articles to verify the correctness of the purchase requirements and that all-applicable QA requirement have been included.

### Re-submission of Non-conforming Articles or Materials

If an article is deemed non-conforming by the contractor and returned to the supplier, the supplier will resubmit the article with evidence showing the article has been corrected, and with markings which clearly indicate that it is a "re-submitted part."  Non-conforming items shall be handled by the process described in section 8.8.

## Receiving Inspection

Upon receipt, all purchased products will undergo an inspection that includes:

- Verification that documentation meets the requirements of the Purchase Order;

- Verification that parts marking and packaging is consistent with the requirements of the Purchase Order;

- Verification of correct parts counts.

Parts will be handled in accordance with the UCB Space Physics Research Group ESD control plan (or equivalent contractor's plan), then bagged, marked, and placed into bonded flight stores.  Records shall be maintained for the life of the mission.

## Fabrication Control

### Manufacturing Certification Log

A Certification Log will be established for each manufactured component that will travel with the item through fabrication and inspection. Operations will be done per referenced documents, or documented directly in the logbook. Torque values, part serial numbers, etc. will be noted, and all entries will be signed and dated by the operator. Entries will include results of in process testing.

### Workmanship

The following NASA workmanship standards shall be used in the fabrication of the THEMIS flight hardware:

Soldering - NASA STD-8739.3, NASA STD-8739.2

Cable, Harness, and Wiring Interconnections - NASA-STD-8739.4

Conformal Coating and Staking - NASA-STD-8739.1

Printed Wiring Board Design - NHB 5300.4 (3K) or NASA STD S-312-003 Rev B.

#### Worker Certification

All flight segment soldering, wiring, and conformal coating/staking will be by technicians certified and trained as required.

### Process Control

Appropriate controls will be implemented for processes for which uniform high quality cannot be ensured by inspection alone.

## ESD Control

ESD control will be accomplished by the techniques and process controls described in the UCB Space Physics Group Electrostatic Discharge Control Plan, Revision B, dated May 1990.

## Non-conformance Control

The PI will perform non-conformance control for failures and discrepancies.  (A failure is a non-conformance discovered in testing, while a discrepancy is a non-conformance discovered at other times) The PI will track non-conformances with a Non-conformance Report that includes the following information:

- A description of the non-conformance,

- Analyses to determine the fundamental cause and any THEMISs to the rest of the flight instrument,

- Remedial action to be taken,

- Verification of the removal of the non-conformance, and

- Disposition of the non-conforming item.

Status of all failures and discrepancies shall be included in the Monthly Technical Report to Project.

### Discrepancies

#### Documentation.

Documentation of discrepancies will begin with receipt of procured materials or fabrication.

#### Initial Review Dispositions.

Discrepant products will be reviewed by engineering personnel to decide if they should be (a) returned for rework, (b) scrapped, (c) returned to supplier, or (d) submitted for MRB action. Initial reviews will be documented as described above.

#### Material Review Board.

The PI will designate an MRB to review all non-conformances, chaired by the THEMIS Project Manager.  The MRB will: determine dispositions, ensure remedial and preventive actions; verify implementation of all dispositions; and ensure accurate records are maintained.  MRB dispositions will specify one of the following:

- Repair: The MRB will approve repairs. Although standard repair procedures may be approved on a one-time basis, the MRB will track the number of standard repairs on a per unit basis to ensure that reliability or quality are not compromised by excessive repairs.

- Use-as-is.

- Waiver: To use or accept hardware at the spacecraft interfaces which does not meet contract requirements; this action will require GSFC Approval prior to implementation.

### Failures

#### Reporting

A failure report will be written for failures that affect the function of the flight segment or could compromise mission objectives.  Other issues shall be recorded in the traveler.  Reporting will begin with the first functional test of the fully assembled component and will continue through the flight segment.  All such shall be reported to GSFC within one business day, with preliminary documentation provided within five business days.  A sample Problem Report form is shown:

#### Failure Review Board.

The PI will designate an FRB to review each failure, chaired by the THEMIS Project Manager.  The FRB will: determine dispositions, ensure remedial and preventive actions; verify implementation of all dispositions; and ensure accurate records are maintained.  FRB dispositions will specify one of the following:

- Repair: The FRB will approve repairs. Although standard repair procedures may be approved on a one-time basis, the FRB will track the number of standard repairs on a per unit basis to ensure that reliability or quality are not compromised by excessive repairs.

- Use-as-is.

- Waiver: To use or accept hardware at the spacecraft interface which does not meet contract requirements; this action will require GSFC Approval prior to implementation.

Failures shall be documented on the Problem Report form, which shall also document any diagnostic and analysis data, plus the final resolution.  This form shall be signed-off by the FRB (including GSFC representative) for closure.

### Alert Information

The PI will support the Alert program by determining the relevance of each Alert submitted to UCB.  If action is required, the MRB will determine the approach to resolving the problem.  Sections 5.6 and 6.3 discuss Alerts further.

## Inspections and Tests

UCB will plan and implement an inspection and test program that will demonstrate that applicable requirements are met.  Inspection and in process testing will be completed prior to installation into the next level of assembly

Verification of soldering to NASA STD-8739.3 and/or NASA STD-8739.2 (as applicable) will be done by NASA-certified personnel other than the original operator.

The component-responsible engineer will review the hardware and documentation package prior to certification of readiness for the next assembly process.

The responsible engineer will perform an end-item inspection on each component.  It will be verified that the configuration is as specified in the released design documentation that workmanship standards have been met, and those test results are acceptable.

### Inspection and Test Records

Inspection and test records will be included in the manufacturing certification log for each deliverable component, to show that all manufacturing operations have been performed, the objectives met, and the end item fully verified.

### Printed Wiring Boards Inspections and Tests

Printed wiring boards shall conform to the requirements of Mil-P-55110F, and shall be qualified by inspection and test results.  PWB coupons shall be evaluated by GSFC or a GSFC approved laboratory for evaluation and approval prior to loading with flight parts.

## Metrology

Verification of the accuracy of test equipment to the necessary levels during testing and calibration of the flight instrument will be done by a combination of calibration by outside vendors (traceable to NIST) and cross-checking of one unit against another.  Equipment that has been calibrated shall have a label indicating the calibration date and how long that calibration is good for.  The engineer making a measurement is responsible for ensuring that the calibration is current when a calibrated measurement is required.  Any measurement that relates to the verification of performance, requirements, or calibration of the instrument should be made with calibrated equipment.

## Handling, Storage, Marking, Shipping, Preservation, Labeling, and Packaging

### Handling

Handling equipment will be proof tested at twice the intended load prior to use.

### Shipping

Shipping of the flight units or components will be done with the appropriate accompanying documentation and handling instructions.

## Government Property Control

UCB and its subcontractors shall be responsible for and will account for all property procured under the contract or provided by the government.  The University property control system and standard government property transfer forms will be used to accomplish this.

## End Item Acceptance

Prior to shipment of the THEMIS probe carrier assembly, the Acceptance Data Package will be assembled by the Project and reviewed by GSFC at the Pre-Ship Review.

## Ground Support Equipment

That part of the Mechanical and Electrical Ground Support Equipment that directly interfaces with the flight hardware shall be of high quality, and shall be subject to a subset of these Quality Assurance standards, such as Metrology, Non-conformance Control, Workmanship, Configuration Management, and Contamination Control as required.  The quality level of all GSE shall be sufficient to ensure that the flight hardware connected to it is not compromised, and the operations performed with the GSE are consistent, correct, and safe.

# CONTAMINATION CONTROL

## Project Requirements

The THEMIS spacecraft shall be built to meet the THEMIS contamination control requirements.  In addition, THEMIS instrument contamination concerns are addressed here.

## THEMIS Concerns

The THEMIS probes contain contamination sensitive detectors (Microchannel Plate detectors and Solid State Detectors).  The detectors are sensitive to dust, water, and most aromatic hydrocarbons.  Some analyzers contain high voltages (up to 3500V).  In addition, some surfaces such as radiators shall be contamination sensitive.

## Control Plan

Subsystems shall be fabricated from low-outgassing materials as discussed in section 6.1.6 of this PAIP to minimize contamination of itself or other instruments. All components will be inspected and/or cleaned prior to integration.

For the detectors, rather than impose a requirement on surface contamination, we rely on proven procedures.  Measuring contamination buildup is very difficult since many of these detectors are buried deep in the analyzers.  The procedures used have proven to be adequate on numerous previous missions.

The detectors are stored, handled, and installed into the flight instruments in appropriately clean environments (typically class 100 clean benches or storage containers) by experienced technicians.  Flight parts and GSE used in the vicinity of sensitive detectors shall be thoroughly cleaned prior to use.  In some cases, bake-out of subassemblies shall occur prior to integration of temperature and contamination sensitive parts to avoid cross-contamination or excessively long bake-outs at relatively low temperature.

Once installed into the flight hardware, the detectors are sealed behind covers with positive flow of high grade dry Nitrogen to prevent contamination.  In this configuration, good housekeeping cleanliness levels (or bagging) are adequate to maintain the cleanliness of the exterior of the instrument.  Prior to delivery to spacecraft integration, the exterior of the instrument shall be cleaned to meet the spacecraft-level cleanliness requirements.  A bake-out will be included as part of the subassembly thermal vacuum tests.

GSE used in the vicinity of flight detectors shall be class 100 clean room compatible, and shall be thoroughly cleaned prior to use.  Once the detectors are sealed into the flight unit, only good housekeeping requirements are placed on the GSE until such time as the instrument is cleaned for integration into the spacecraft.

Following cleaning and bake-out of the instruments for integration with the spacecraft, the instrument and GSE that must be used near the instrument must be maintained at adequate cleanliness levels per the Project Contamination Control Plan.  This will be achieved using clean rooms and/or bagging.

## THEMIS Requirements on Spacecraft I&T and Operations

Nitrogen purge shall be maintained on a near-continuous basis throughout I&T, at least up to encapsulation.  Occasional outages in the Nitrogen flow can be tolerated for a several hours.

For tests that require the removal of the aperture covers, exposing the detectors, the instrument should be bagged or otherwise maintained in a class 100,000 environment or better, and Nitrogen flow should be continuous.  Such exposures should be limited in duration to a few hours total.  Alternatively, longer duration at a better cleanliness level can be tolerated.

Some of the analyzers contain high voltages (up to 3500V).  These supplies can only be turned on in a good vacuum to avoid arcing.  This implies outgassing requirements, thermal vacuum test requirements, and powering off some of the supplies for thruster firings.

Spacecraft thermal vacuum testing will be required to verify contamination levels prior to opening of the ESA covers and HV application. At the end of the TV test, the chamber must be back-filled with clean dry Nitrogen.

# SOFTWARE ASSURANCE

## General

The Space Physics Group at the UCB Space Sciences Laboratory has had considerable experience in the development of real time processor-based systems for space flight use (including the first microprocessor system flown on a NASA satellite - ISEE-1) and computer-based ground support equipment. The group currently includes persons of considerable ability and experience in the software area.  The group has developed approximately 25 such systems over the past 15 years, all of which have been delivered on schedule and have been completely successful.

It is our intent to use the same type of organization and development procedures and coding practices on THEMIS that has proven to be successful on past programs.

Flight Software is the instrument computer code that runs in the micro processor(s), which are parts of the flight experiment package.  Only Flight Software is covered by this document.   GSE software shall be developed using reasonable practices, and shall only be controlled to the extent that critical GSE used for acceptance tests at the system level shall be under configuration control starting at the beginning of system-level testing.

## Software Development

THEMIS software development includes the following subsystems:

- IDPU flight processor (UCB)

- Bus Avionics Unit Flight Software (Hammers)

- Mission Operations Software (UCB)

Software will be by a small team of programmers.  Control is maintained by the programmer for a subsystem, who is responsible for maintaining the code and incorporating all changes at a single location throughout its lifecycle.

More details of the software development can be found in the THEMIS Software Development Plan.

## Documentation

Flight software will be documented at a minimum by the following:

| Software Development Plan | Describes the flight software development plan as called out in the THEMIS contract deliverables list. |
| --- | --- |
| Software Requirements: | Describes the functional requirements on the software to a level sufficient for a programmer to implement. |
| Software Users Guide: | Describes the software at the interface level for the end user (scientists, operations personnel and ground software programmers). |
| Software History Log: | This log will include all PFRs (with dispositions), results of acceptance testing, and detailed descriptions of any modifications made by uplinked code after launch. This is the programmer’s logbook. |

The software developer will be responsible for generating these documents and maintaining configuration control over them.  This control will consist of reviewing and implementing any document changes, maintaining a revision code on all document updates, and distributing the documents for review to interested parties.

Software with a long life cycle shall also have sufficient design documentation to allow a new programmer to maintain and update the software if required.

## Software Design Reviews

Software reviews will be performed at the same level and in concert with their subsystem review, either the IDPU or the BAU. These reviews include:

- Software Requirements review;

- Software Preliminary Design Review;

- Software Critical Design Review.

## Configuration Management

Configuration control on the software will be performed by the subsystem programmer, and any change requests or bug reports will be communicated to him. Version numbers will be assigned and maintained by the subsystem programmer. Version numbers will be reported in telemetry, identifying the functional capabilities of the loaded software. If software is loaded or patched, such that the new software is functionally different, the version number must reflect the change.

After the start of subsystem testing (IDPU or BAU), failure reports and change requests will be made to the subsystem lead via the Problem/Failure Report system. The subsystem lead will verify the problem and determine the cause.  If the problem can be fixed without impacting the functionality of the rest of the code and does not have a serious schedule impact, he may proceed with the solution, and distribute a new revision of the code for further tests. Problems with greater impact will be submitted to the FRB. The subsystem lead shall maintain a documentation of all software PFRs.

When all PFRs have been disposition and the final version of the code has completed acceptance testing, the code will be committed to the flight PROMs and installed into the flight hardware. From this point on, all change requests or problem resolution will be made via the FRB/MRB mechanism described in section 8 of this PAIP.  If a change is approved, the subsystem lead will implement the fix and issue a new release.  The new release will be submitted to a full acceptance before again committing to PROM.   No software will be used with flight hardware unless it in a known documented and verified state.

Any code to be uplinked after launch will be submitted to the same level of configuration control as was levied on the final version of the flight code, including acceptance testing on simulators prior to uplinking the code.  Any significant code uplink will be accompanied by a change in the code version number which is included in the instrument housekeeping, so that ground data processing software can determine what version of the software is running.

At all stages of the software development, a system of backups will be maintained to ensure that the failure of a system or media will not destroy more than 1 day's work. In addition, a backup copy will be maintained off-site, updated periodically.

# Appendix A - APPLICABLE DOCUMENTS

The following documents shall be applicable to this PAIP to the extent referenced herein.

| Document No. | Title |
| --- | --- |
| GEVS-SE | General Environmental Verification Specification for STS & ELV Payloads, Subsystems, and Components, rev A, dated June 1996 |
| EWR 127-1 | Range Safety Requirements Eastern and Western Range |
| GSFC 311-INST-001 | Instructions for EEE Parts Selection, Screening, and Qualification |
| GSFC PPL-21 Notice 1 | Preferred parts list |
| MIL-STD-975 (NASA) | NASA Standard Electrical, Electronic, and Electromechanical (EEE) Parts List |
| MSFC-SPEC-522B | Design Criteria for Controlling Stress Corrosion Cracking |
| ASTM E-595 | Standard Test Method for Total Mass Loss (TML) and Collected Volatile Condensable Materials (CVCM) from Outgassing in a Vacuum Environment (Re-approved 1999) |
| tbd | THEMIS Configuration Management Plan |
| tbd | THEMIS Contamination Control Plan |
| tbd | THEMIS Environmental Test Plan |
| 541-PG-8072.1.2 | Goddard Space Flight Center Fastener Integrity Requirements |
| NASA STD-8739.2 | Requirements for Surface Mount Soldering |
| NASA STD-8739.3 | Requirements for Soldered Electrical Connections |
| NASA STD-8739.4 | Requirements for Interconnecting Cables, Harnesses, and Wiring |
| NASA STD-8739.1 | Requirements for Conformal Coating and Staking of Printed Wiring Boards and Electronic Assemblies |
| NHB 5300.4 (3K) | Design Requirements for Rigid Printed Wiring Boards and Assemblies |
| NASA STD S-312-003 Rev B. | Design Requirements for Rigid Printed Wiring Boards and Assemblies |
| - | UCB Space Physics Group Electrostatic Discharge Control Plan, Revision B, dated May 1990. |
| MIL-P-55110 Rev F | General Specification for Printed Wiring Boards |
| tbd | THEMIS IDPU Software Development Plan |