## June 3rd, 2003

SCM Requirements

### Science Requirements

The SCM shall measure (with appropriate sensitivity) the magnetic components of waves responsible for

- (i) current disruption, and/or

- (ii) electron acceleration near X-line.

Current disruption can be driven by cross-field instability: F~0.1-1 Flh (Flh being the lower hybrid frequency), and/or be due to an instability driven by a parallel current: F~nFbe (Fbe being the electron bounce frequency across the current sheet). As close to Earth as 8RE, Flh~25Hz. In thin current sheets (~1000km),

Fbe~1-10Hz.

For the Hall-MHD models of reconnection, measurements of whistler mode waves near X-line provides a critical test to the theory. Whistler mode waves have F<Fce/2, Fce being the electron gyrofrequency. At 8Re Fce~1 kHz, at 20 Re, inside the tail current sheet  fce~100Hz, or less.

In summary the frequency range shall cover 8/4 kHz (Fce/2 at 5Re). Priority shall be on frequencies below 100 Hz, that deserve wave form measurements.

Performance requirements

(measurement range and accuracy, measurement resolution)

- The SCM frequency range shall be 1-8/4 kHz

- The sensitivity of the SCM @10Hz shall be better than 1pT Hz^1/2, and 0.1 pT Hz^1/2 at 1kHz

- Phase stability, temperature control…TB Discussed by Christophe (to morrow)

# Wave form measurements

| Survey |  |  | Particle Burst |  |  | Wave Burst |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/sec | BW | bits/s | S/sec | BW | bits/s | S/sec | BW | bits/s | S/sec | BW | bits/s |
| 64 | 0.1 - 32 Hz | 3072 | 256 | O.1-128 Hz | 12288 | 1024 | 0.1-512 Hz | 49152 | 4096 | 0.1-2048 Hz | 196608 |

#### Spectral measurements

The SCM FFT Spectra shall be 256/512? points, computed on board. Transmission of 16/32 points, with fixed df/f (~25/50%?),

Between  8/16 Hz – 4/8 kHz.

# SCM Requirements (cnd)

# Resource requirements

# (mass, power, data)

- The SCM shall not exceed the allocated mass budget (sensor 600g, pre-amps 200g)

- The SCM shall not exceed the allocated power budget (pre-amps 80mW)

# Fabrication, Integration and Test

# (Parts: MIDEX Quality Requirements, I&T, Hazards

# & Controls, Facilities, Calibration, Environmental Test, Spacecraft Integration)

- SCM calibration shall ensure absolute amplitude and phase requirements (<5% ) are met.

- Calibrations of SCM antennas and PA shall be made in a quiet site (Chambon la Forêt).

- On board calibration (one every day) will allow correction of the  transfer function, if needed.

- Tests of SCM: electrical, vibration (for 6 models), and thermal (for EM) shall be made in a TBD site.

- CETP personnels shall participate in the integrations.

- The physical mounting and electrical wiring of the SCM sensors shall be verified and recorded for each sensor on each probe.

- The EFI, SCM, FGM GSE shall support the joint tests and intercalibration of the combined fields system.

- SCM shall participate in spacecraft integration, as necessary.

# Contamination/radiation requirements

# (Electromagnetic, Electrostatic, radiation)

- SCM antennas shall be protected against electrostatic discharges.

- Magnetic noise radiated by the S/C and the other instruments shall not exceed SCM sensitivity (1pT /Hz^1/2@10Hz, and

0.05pT/ Hz^1/2 at 1kHz).

- The SCM-PA shall withstand 300 kRads.

- Ripples from solar cells rotation….