TITLE: Expedite Closure of Existing Orbit Debris Assessment Issues	RFA CODE:  13

REQUESTED BY:  Terry Ford

SPECIFIC REQUEST:  Expedite closure of existing Orbit Debris Assessment issues including:

- Analysis associated with lifetime of 2nd and 3rd Delta stages.

(2) Reliability of re-entry for single string spacecraft.

SUPPORTING RATIONALE: Early JSC buy off of the RTS plans will relieve risk on mission configuration, proceeding to next phase.

- RESPONSE:

- The THEMIS project shares the concerns of the reviewer on this issue. We have completed a preliminary orbital debris assessment, in coordination with KSC and JSC, and have submitted it to GSFC. An excerpt from the document is attached.

- With respect to the specific issues of 2nd and 3rd Delta stage lifetimes the following is a summary of our findings and our plan:

  - The GSFC-developed code “GTDS” which was used for re-entry calculations in Phase-A and in Phase-B shows that the 3rd stage re-enters within 10years with the requested injection orbit (637km perigee altitude). KSC suggests the possibility of a depletion maneuver to bring 2nd stage in with the requested injection orbit.

  - The JSC-developed code shows that the 3rd stage does not re-enter within 25years assuming the proposed perigee altitude. Only with at 250km does the JSC code show re-entry of 3rd stage and 2nd stage.

  - It is evident that the GSFC and JSC codes are not in agreement and it is unclear which one is correct.

- d.  A 250km perigee is not detrimental to the project, as it represents a 5% decrease in on-board fuel, which is well within the presently available 15% contingency and 15% additional fuel margin.

- The proposed plan is as follows:

- THEMIS will evaluate if the proposed 250km altitude represents a risk to the project, when the GSFC code is used to run re-entry calculations.

- THEMIS will evaluate if, when accounting for all known inefficiencies in the ascend maneuvers, and as the mission design becomes more refined, a 5% fuel reduction is indeed acceptable and does not decrease fuel margins below the program’s comfort level.

- If premature re-entry is found, or if fuel margins are depleted below the project’s comfort level due to the proposed 250km perigee then the project will either embark upon resolution of code-differences or request an exemption. A full answer to the above questions may necessitate updated launch vehicle performance data from KSC or Boeing, in order to determine if reduction in target inclination can reduce the losses in on-board fuel consumption. However, it is rather likely that the project will be able to live both with a lower perigee and a reduced injection orbit lifetime and no exemption will be necessary.

(2) With respect to the issue of reliability of re-entry for single string spacecraft, the project has followed the verbal recommendation of the reviewer and submitted the best current value of probe reliability (Ps=0.83 for single probe in 2years) for HQ evaluation. This reliability value is not expected to change significantly from final component selection. Early JSC approval on this value is therefore going to serve the project quite well, in terms of avoiding potential delays or component changes later in the program and we thank the reviewer for pointing out the importance of this issue at this stage.

THEMIS
Orbital Debris Assessment

SAI-RPT-0555

Revision – PDR Draft, Rev A

December 4, 2003

Contract No.:	NAS5-02099

- Task No.: 	00536.2.2.1.2.1.01.01

| Revision | Description | Date | Approval |
| --- | --- | --- | --- |
| PDR Draft | Initial Release at Probe/Probe Carrier PDR | 10/22/2003 |  |
| PDR Draft, Rev A | Incorporated comments on initial draft from GSFC and JSC, as well as information from KSC on LV | 12/4/2003 |  |

Signature Page

This is a controlled document. Any changes require the approval of the Configuration Control Board.

Prepared by:

Kevin S. Brenneman	Date

Systems Engineer

Reviewed by:

Tom Ajluni	Date

Manager, System Engineering

Reviewed by:

Ellen Taylor	Date

Chief Engineer

Reviewed by:

Manfred Bester	Date

Mission Operations Manager

Approved by:

Michael J. Cully 	Date

Director of Civil and Commercial Programs

Approved by:

Peter Harvey	Date

Project Manager

Approved by:

Vassilis Angelopoulos	Date

Principal Investigator

Table of Contents

Executive Summary	vi

1.0	THEMIS Program Background and Program Management	1

1.1	Project Objectives	1

1.2	Mission Description	1

1.3	Project Schedule	2

1.4	Project Management	2

1.5	References	2

2.0	THEMIS Mission Design and Operation Factors	2

2.1	THEMIS Hardware	2

2.1.1	Probe	2

2.1.2	Probe Carrier	5

2.1.3	Launch Vehicle	6

2.2	THEMIS Mission Parameters	8

2.2.1	Initial Orbits	8

2.2.2	Mission Orbits	9

2.2.3	Final Orbits	10

2.2.4	Attitudes	11

3.0	Assessment of Debris Generation During Normal Operations	11

3.1	Probe	11

3.2	Launch Vehicle	11

4.0	Assessment of Debris Generated by Explosions and Intentional Breakups	12

4.1	Accidental Explosions	12

4.1.1	Probe	12

4.1.2	Probe Carrier	12

4.1.3	Launch Vehicle	13

4.2	Intentional Breakups	15

5.0	Assessment of Debris Generated by On-Orbit Collisions	15

5.1	Assessment of Collisions with Large Objects During Mission Operations	15

5.1.1	Probe	15

5.1.2	Launch Vehicle	17

5.2	Assessment of Collisions with Small Debris During Mission Operations	18

6.0	Assessment of Postmission Disposal Of Space Structures	19

6.1	Disposal For Final Mission Orbits Passing Through LEO	19

6.1.1	Probe	19

6.1.2	Launch Vehicle	21

6.2	Reliability of Postmission Disposal Operations	21

7.0	Assessment of Survival of Debris from the Postmission Disposal Atmospheric Reentry Option	21

7.1	Probe	21

7.2	Launch Vehicle	23

8.0	Conclusion	24

8.1	Probe	24

8.2	Delta II 3rd Stage/Probe Carrier	25

Appendix A - Definitions	1

List of Figures

Figure 2-1  THEMIS Probe Structure	4

Figure 2-2  THEMIS Probe Stowed & Deployed	4

Figure 2-3  THEMIS Probe Carrier With And Without Probes	5

Figure 2-4  THEMIS Probe Carrier Structure	6

Figure 2-5 Illustration of Delta II 2nd stage	7

Figure 2-6 Illustration of Delta II Delta II 2nd stage	8

Figure 6-1  Probe P1 And P2 Lifetime Versus Lunar Phase	20

List of Tables

Table ES-1  Probe Orbital Debris Guideline Compliance Status	vi

Table ES-2 Launch Vehicle Orbital Debris Guideline Compliance Matrix	vi

Table 2-1  Initial Orbits	9

Table 2-2  P1, P2, & P5 Large Orbit Adjustments	9

Table 2-3  P1 & P2 Orbits, First Night Side Wedding Day	9

Table 2-4  First Day Side Wedding Day	10

Table 2-5  Second Night Side Wedding Day	10

Table 2-6  Second Day Side Wedding Day	10

Table 2-7  Final Orbits, Following Depletion Burns	11

Table 4-1 2nd Stage Failure Probabilities	13

Table 4-2 3rd Stage Failure Probabilities	13

Table 4-3 2nd Stage Relevant Failure Probabilities	14

Table 4-4 2nd Stage K-factor Calculations	14

Table 5-1  Large Object Probability Calculation	17

Table 5-2  2nd Stage Orbital Elements after 3rd Burn	18

Table 5-3  3rd Stage Orbital Elements after Burnout	18

Table 5-4  Small Object Probability Calculation (For Probe P2)	19

Table 6-1  Lifetime Calculation For Probes 3, 4, and 5	20

Table 7-1  Debris Casualty Area From Decaying Probes	22

Table 7-2  Casualty Area of the Delta II 3rd Stage with attached Probe Carrier	23

Table 8-1 Probe Compliance Status	24

Table 8-2 Orbital Debris Guideline Compliance Matrix	25

# Executive Summary

This report documents the compliance of the Time History of Events and Macroscale Interactions during Substorms (THEMIS) Project with the PDR deliverable guidelines of NASA Safety Standard 1740.14, ‘Guidelines and Assessment Procedures for Limiting Orbital Debris’. The project has just completed its Mission PDR as of November 14, 2003. A summary of the current Probe compliance status is provided in Table ES-1 below, and LV compliance is provided in Table ES-2 below.

Table ES-1  Probe Orbital Debris Guideline Compliance Status

| Guideline Number | Description | Met | Not Met | Not Applicable | Impact/ Issue |
| --- | --- | --- | --- | --- | --- |
| 3-1 | Operational Debris - LEO | X |  |  | None |
| 3-2 | Operational Debris - GEO | X |  |  | None |
| 4-1 | Accidental Explosion During Mission | Probably |  |  | None |
| 4-2 | Accidental Explosion After Mission | Probably |  |  | Battery depletion |
| 4-3 | Intentional Breakup- Long-term Risk |  |  | X |  |
| 4-4 | Intentional Breakup- Short-term Risk |  |  | X |  |
| 4-5 | Intentional Breakup- During Reentry |  |  | X |  |
| 5-1 | Collision with Large Objects | Probably |  |  | None |
| 5-2 | Collision with Small Objects | Possibly |  |  | Shielding |
| 6-1 | Disposal – LEO | Possibly |  |  | Lunar phasing |
| 6-2 | Disposal – Above LEO |  |  | X |  |
| 6-3 | Disposal - 12 Hour Orbits |  |  | X |  |
| 6-4 | Disposal Reliability |  | X |  | Single String design |
| 7-1 | Reentry Survivability | Probably |  |  | None |

Table ES-2 Launch Vehicle Orbital Debris Guideline Compliance Matrix

| Guideline | Compliance | Possible mitigation strategies |
| --- | --- | --- |
| 3-1a | Requires modification | - Launch date selection - Perigee reduction |
| 3-1b | Compliant |  |
| 4-1 | Non-Compliant | Requires new hardware |
| 4-2 | Compliant |  |
| 5-1 | Requires modification | - Launch date selection - Perigee reduction - Depletion burn |
| 6-1 | Requires modification | - Launch date selection - Perigee reduction - Depletion burn |
| 7-1 | Compliant (not 2nd stage) | Requires new hardware |