TITLE: Provide Flow-down of Safety/Inhibits For Separation and Propulsion	RFA CODE:  14

REQUESTED BY:  R. Schnurr

SPECIFIC REQUEST:  Provide flow-down of safety/inhibits for separation and propulsion system.

SUPPORTING RATIONALE: Propulsion system, power system, and C&DH did not show flow-down requirements for safety critical functions.

- RESPONSE

- Three catastrophic failure modes have been identified:

- 1) Inadvertent release of the instrument booms

- 2) Inadvertent actuation of the propulsion system (post propellant loading)

- 3) Inadvertent release of the probes from the probe carrier

- To handle them, the following mission level requirements have been added to the Mission Requirements Document (MRD) Rev B, with a Parent Requirement for these being P-18 (The THEMIS Project shall meet the requirements of EWR 127-1):

| SFTY-1 | GENERAL DESIGN POLICY 3.3 (a) | If a system failure may lead to a catastrophic hazard, the system shall have three inhibits (dual fault tolerant). |
| --- | --- | --- |
| SFTY-2 | GENERAL DESIGN POLICY 3.3 (b) | If a system failure may lead to a critical hazard, the system shall have two inhibits (single fault tolerant). |
| SFTY-3 | GENERAL DESIGN POLICY 3.3 (c) | If a system failure may lead to a marginal hazard, the system shall have a single inhibit (no fault tolerant). |
| SFTY-4 | GENERAL DESIGN POLICY 3.3 (d) | Probabilities of hazard occurrence shall be taken into consideration when determining the number of required inhibits. (See Chapter 1, Table 1-2, “Acceptability Guidelines for Prelaunch Area/Launch Complex Hazard Consequences and Probabilities Categories.”) |
| SFTY-5 | GENERAL DESIGN POLICY 3.3 (e) | Systems shall be able to be brought to a safe state with the loss of an inhibit. |
| SFTY-6 | GENERAL DESIGN POLICY 3.3 (f) | All inhibits shall be independent and verifiable. |
| SFTY-7 | GENERAL DESIGN POLICY 3.3 (g) | Design inhibits shall consist of electrical and mechanical hardware. |
| SFTY-8 | GENERAL DESIGN POLICY 3.3 (h) | Operator controls shall not be considered a design inhibit. NOTE: Operator controls are considered a control of an inhibit. |

In addition, the following subsystem flow-down requirements have been added or modified in the MRD Rev B:

| IN.BOOM-9 | All deployed booms shall include TBD inhibits to prevent inadvertent release of instrument booms. | Parents SFTY-1 thru SFTY-8 |
| --- | --- | --- |
| IN.DPU-42 | The IDPU LVPS shall use a separate 28+/-6V switched actuator supply from the Probe for all boom deploy mechanisms | Parents SFTY-1 thru SFTY-3 |
| IN. DPU-13 | The IDPU shall provide separate enable and activation commands for critical instrument functions such as boom deployments | Parents SFTY-1 thru SFTY-3 |
| IN.EPS-3 | The EPS shall provide a separate 28+/-6V switched actuator supply to the IDPU for all boom deploy mechanisms. | Parents SFTY-1 thru SFTY-3 |
| PB.RCS-15 | The RCS shall include TBD inhibits to prevent inadvertent release of hydrazine propellant. | Parents SFTY-1 thru SFTY-3 |
| PB.CDH-46 | The CDH shall include TBD inhibits to prevent inadvertent release of hydrazine propellant. | Parents SFTY-1 thru SFTY-3 |
| PB.MEC-13 | The Probe Bus separation system shall include TBD inhibits to prevent inadvertent release of the Probe from the Probe Carrier. | Parents SFTY-1 thru SFTY-3 |
| PB.CHD-48 | The CDH shall include TBD inhibits to prevent inadvertent release of the Probe from the Probe Carrier. | Parents SFTY-1 thru SFTY-3 |

This reflects that both the deployer and the deployed have joint responsibility to implement an acceptable system solution.