TITLE: Add Carrier De-Spin Requirements to the Launch Vehicle Requirements	RFA CODE:  21

REQUESTED BY:  J. Hair

SPECIFIC REQUEST:  Requirements need to be added to the MRD Launch Vehicle requirements to stipulate, at least, the requirement to de-spin carrier to 15 rpm prior to probe deployment and that software/hardware deployment signals shall be provided to carrier from Launch Vehicle after a certain event.

SUPPORTING RATIONALE: Requirements interaction between the Launch Vehicle and carrier needs to be document in MRD at these levels to show flow-down to other items and flow into specifications.

- RESPONSE

- The following requirements have been added in the Mission Requirements Document (MRD) Rev B to ensure that top level Probe Carrier requirements will be flown-down and addressed in the Launch Vehicle Mission Specification:

| ID | Requirement Statement – Rev B | Rationale |
| --- | --- | --- |
| M-62 | The Launch Vehicle shall despin (if necessary) the Probe Carrier to less than 15 rpm prior to probe deployment. | Worst-case separation analysis assumes the Probe Carrier is spinning at 15 RPM. |
| M-63 | The Launch Vehicle shall provide a separation signal to the Probe Carrier to indicate when the five probes should start the separation sequence. | Each probe provides a separation signal to its own separation system to deploy itself. The launch vehicle signal is a simple bi-level to start the timed sequence of the five probes. |