TITLE: Set Limitations on the Placement of the MAG and EFI Instruments		RFA CODE:  19

REQUESTED BY:  J. Hair

SPECIFIC REQUEST:  Requirements need to be added to set limitations on the placement of the MAG and EFI instruments relative to the Spacecraft and each other.

SUPPORTING RATIONALE: As it stands now, there is nothing that prevents the booms from being 1 cm long, forcing contamination sources on the probe to be sufficiently low to meet the I&T requirement or other contamination requirements. The requirements regarding contamination limits are open ended and circular. There needs to be either a distance associated with the applicable contamination requirement or a contamination limit on the probe. This cannot be buried in contamination specification. Once of these needs to be at the top level to break the logjam.

- RESPONSE

- The following requirements have been revised in the Mission Requirements Document (MRD) Rev B to identify a distance from the Probe at which the contamination requirements must be verified:

| ID | Requirement Statement – Rev A | Requirement Statement – Rev B |
| --- | --- | --- |
| M-30 | AC Magnetic Noise radiated by the Probe and the other instruments shall not exceed 10pT/Hz^1/2 @ 10 Hz and 1pT/Hz^1/2 at the SCM sensor. | AC Magnetic Noise radiated by the Probe and the other instruments shall not exceed 10pT/Hz^1/2 @ 10 Hz and 1pT/Hz^1/2 at 1 meter from the Probe. (Rationale: SCM sensor is on a 1-meter boom). |
| M-31 | DC Magnetic Noise radiate by the Probe and other instruments shall not exceed 1 nT at the FGM sensor. | DC Magnetic Noise radiate by the Probe and other instruments shall not exceed 1 nT at 2 meters from the Probe. (Rationale: FGM sensor is on a 2-meter boom). |

To ensure these requirements are met, all THEMIS elements shall comply with the THEMIS Magnetics Cleanliness Specification & Design Guidelines and the THEMIS Electrostatic Cleanliness Specification (as stated in the MRD, M-32 and M-34).  These documents are already underway, and will be in final form by PDR.  The documents will contain component-level contamination budgets developed for the baseline Probe configuration.  In addition, UCLA is currently on contract to provide a magnetics model of the Probe. To develop the model, Swales has provided UCLA with dimensioned design drawings, identifying relevant information such as exact sensor location and boom length.