TITLE: Deployables Torque Ratio	RFA CODE:  Rec

REQUESTED BY:  J. Hair

SPECIFIC REQUEST:

Deployed systems may not rely on momentum to ensure complete deployment. A torque ratio of 4:1 must be demonstrated thoroughly the entire range of motion of the deployed element.

SUPPORTING RATIONALE: This is to ensure proper deployment even if momentum is reduced due to unexpected friction during other phases of deployment.)

- RESPONSE

The THEMIS project is following GEVS guidelines by maintaining a minimum torque ratio of 3 for friction-dominated systems, exclusive of momentum, even though many do have significant momentum and could use a ratio of 2.25 according to the GEVS guidelines. We are not aware of any formal NASA requirement for maintaining a ratio of 4.