TITLE: Science End Product Requirements	RFA CODE:  Recommendation

### REQUESTED BY:  J.B. Joyce Dan Mark

SPECIFIC REQUEST:  Add requirements:

(1) List end use science data products

(2) Explicitly call out performance values. For example: Science data availability to community, space weather alert data availability, who it gets routed to (in case of alerts, etc.).

SUPPORTING RATIONALE: Need to list more than just individual sensor data items as requirements.  The end products trace directly to science goals (already listed as requirements) and are the items that drive software pipeline requirements (lower-level).

- RESPONSE

This is part of the data management plan which is currently under way; a preliminary version will be available at PDR.

Data products are in the attached, preliminary data quantities list. The list specifies quantity, time resolution, accumulation for P1-P5.

Data products become readily available immediately upon receipt and level zero processing, with a delay of 1 day since receipt.

Near-real time data accessibility is limited to commissioning phase, to ensure ACS stability and validate health and status of EFI and FGM and SCM mags during boom deployments.

There are no plans for near-real time release of data to NOAA-SEC center.

DATA QUANTITIES FROM TABLE INSTRUMENT DATA RATES REV F.3

| FGM |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | CH | Digits | Bits | Total mode |  | Explanations |  |  |  |  |  |  |  |
| Spin fit | 5 | 16 | 80 |  |  | Bx,By,Bz, sigma, Npoints @ 16bits |  |  |  |  |  |  |  |
| Fast survey | 3 | 16 | 48 |  |  | 3 components |  |  |  |  |  |  |  |
| Spinning Raw | 3 | 16 | 48 |  |  | 3 components |  |  |  |  |  |  |  |
| SCM |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function | CH | Digits | Bits | Total mode |  | Explanations |  |  |  |  |  |  |  |
| Spinning LF | 1 | 16 | 16 |  |  | dBx raw |  |  |  |  |  |  |  |
| Filter banks | 8 | 8 | 64 |  |  | One component (Axial or Radial) at 8 frequencies; intended for slow survey |  |  |  |  |  |  |  |
| Fast Survey | 3 | 16 | 48 |  |  | 3 components |  |  |  |  |  |  |  |
| Spinning Raw | 3 | 16 | 48 |  |  | 3 components; P-burst |  |  |  |  |  |  |  |
| PB Spectra | 32 | 8 | 256 |  |  | P-burst: 3-component spectra (Bpar, Bpertot or Bpar, BperX) at 16 frequencies (subset of WB spectra) |  |  |  |  |  |  |  |
| WB Spectra | 64 | 8 | 512 |  |  | P-burst: 3-component spectra (Bpar, Bpertot or Bpar, BperX) at 32 frequencies (full capability) |  |  |  |  |  |  |  |
| EFI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function | CH | Digits | Bits | Total |  |  |  |  |  |  |  |  |  |
| Spin Fit | 6 | 16 | 96 |  |  | Ez, A offset, B,C sin/cos amplitudes, sigma, Npoints @16bits |  |  |  |  |  |  |  |
| Filter banks | 8 | 8 | 64 |  |  | One component (Axial or Radial) at 8 frequencies; intended for slow survey |  |  |  |  |  |  |  |
| Fast Survey | 4 | 16 | 64 |  |  | 4 channels, 3 components and 1 subcommed data column; intended for fast survey |  |  |  |  |  |  |  |
| Raw-Spin | 4 | 16 | 64 |  |  | P-burst: 3 components and one subcommed: [(V1+V2)/2; V1, V5, V6] |  |  |  |  |  |  |  |
| PB Spectra | 32 | 8 | 256 |  |  | P-burst: 3-component spectra (Epar, Epertot or Epar, EperX) at 16 frequencies (subset of WB spectra) |  |  |  |  |  |  |  |
| WB Spectra | 64 | 8 | 512 |  |  | W-burst: 3-component spectra (Epar, Epertot or Epar, EperX) at 32 frequencies (Full capability) |  |  |  |  |  |  |  |
| SST |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function | CH | Digits | Bits | Total i & e |  | Explanations |  |  |  |  |  |  |  |
| FDF | 2048 | 8 | 16384 | 32768 |  | 8bits x 4 elevation x 16 phase angles x 32 energies |  |  |  |  |  |  |  |
| Fast Survey | 512 | 8 | 4096 | 8192 |  | 8bits x 4 elevation x 16 phase angles x 8 energies but fast (RDF subset of FDF and fast) |  |  |  |  |  |  |  |
| Slow Survey | 512 | 8 | 4096 | 8192 |  | 8bits x 4 elevation x 16 phase angles x 8 energies (RDF subset of FDF and slow) |  |  |  |  |  |  |  |
| N,P,V | 13 | 16 | 208 | 416 |  | 1 N, 3 V, 9 P @ 16 bits |  |  |  |  |  |  |  |
| ESA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function | CH | Digits | Bits | Total i & e |  | Explanations |  |  |  |  |  |  |  |
| FDF | 2816 | 8 | 22528 | 45056 |  | 8bits x 88 angles x 32 energies |  |  |  |  |  |  |  |
| Fast Survey | 1408 | 8 | 11264 | 22528 |  | 8bits x 88 angles x 16 energies (RDF subset of FDF and fast) |  |  |  |  |  |  |  |
| Slow Survey | 1408 | 8 | 11264 | 22528 |  | 8bits x 88 angles x 16 energies (RDF subset of FDF and slow) |  |  |  |  |  |  |  |
| N,P,V | 14 | 16 | 224 | 448 |  | 1Vpot, 1 N, 3 V, 9 P @ 16 bits |  |  |  |  |  |  |  |

DATA RATES FROM TABLE INSTRUMENT DATA RATES REV F.3

|  | Survey |  |  |  |  |  |  |  | Particle Burst (baseline science, all except HF modes) |  |  |  | Wave Burst (HF modes) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Slow Survey (Radbelt science) |  |  |  | Fast Survey (baseline science) |  |  |  |  |  |  |  | Wave Burst (TYPICAL) |  |  |  |
|  | S/spin | CH/agle-ergies | BW | bits/s | S/spin | CH/agle-ergies | BW [Sps] | bits/s | S/sec | CH/agle-ergies | BW [Sps] | bits/s | S/sec | CH/agle-ergies | BW | bits/s |
| FGM | 1 | 5 | DC-0.333Sps | 27 | 16 | 3 | 5.33 | 256 | 32 | 3 | DC-16 Hz | 1536 | 128 | 3 | DC-64 Hz | 6144 |
| SCM | 1 | 8 | FB: 1Hz-2kSps | 21 | 32 | 3 | 10.67 | 512 | 128 | 35 | 10Hz-128Sps | 6656 | 1024 | 67 | 32-512 Hz | 51200 |
| EFI | 1 | 14 | DC-2kSps | 53 | 32 | 4 | 10.67 | 683 | 128 | 36 | DC-128Sps | 8704 | 1024 | 68 | 32-512 Hz | 67584 |
| SST | 1 | 525 | spin mom | 160 | 1 | 2560 | spin RDF | 2816 | - | 1 | spin FDF | 10923 | - | 0 | - | 0 |
| ESA | 1 | 1422 | spin mom | 208 | 1 | 4224 | spin RDF | 7627 | - | 1 | spin FDF | 15019 | - | 0 | - | 0 |
| Totals (bits/second) |  |  |  | 469 |  |  |  | 11893 |  |  |  | 42837 |  |  |  | 124928 |

DATA VOLUMES FROM TABLE INSTRUMENT DATA RATES REV F.3

| Applies to Probe # |  |  | Compression factor = | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S/C Mode durations / orbits |  |  |  |  |  |  |  |  |  |
|  |  | Mbits/ | 24 | hrs |  |  |  | T_orbit = | 24 | hrs |  |  |  |  |  |  | Data Vol (Mbits) |
| P3/4/5 | Instrument | Slow Survey | Fast Survey | Particle Burst | Wave Burst | Total |  | S/C Mode |  | Rel. Allocation |  | Abs. Allocation |  | Hours | Seconds | Rate (bps) |  |
|  |  |  |  |  |  |  |  | T slow survey |  | 100% | T_orb | 100% | T_orb | 24 | 86400 | 469 | 41 |
|  | FGM | 2 | 10 | 7 | 3 | 21 |  | T fast survey |  | 45% | T_ss | 45% | T_orb | 10.8 | 38880 | 11893 | 462 |
|  | SCM | 2 | 20 | 28 | 22 | 72 |  | T P-burst |  | 11% | T_fs | 5.0% | T_orb | 1.19 | 4277 | 42837 | 183 |
|  | EFI | 5 | 27 | 37 | 29 | 97 |  | T W-burst1* |  | 10% | T_pb | 0.5% | T_orb | 0.12 | 428 | 124928 | 53 |
|  | SST | 14 | 109 | 47 | 0 | 170 |  | T W-burst2* |  | 3% | T_pb | 0.1% | T_orb | 0.03 | 114 | 468992 | 53 |
|  | ESA | 18 | 297 | 64 | 0 | 379 |  |  |  |  |  |  |  |  |  |  | 740 |
|  | Total | 41 | 462 | 183 | 53 | 740 |  |  |  |  |  |  |  |  |  |  |  |
|  | Compressed volume |  |  |  |  | 370 | Mbits |  |  |  |  |  |  |  |  |  |  |
|  | Rate/day |  |  |  |  | 370 | Mbits |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | S/C Mode durations / orbits |  |  |  |  |  |  |  |  |  |
|  |  | Mbits/ | 48 | hrs |  |  |  | T_orbit = | 48 | hrs |  |  |  |  |  |  | Data Vol (Mbits) |
| P2 | Instrument | Slow Survey | Fast Survey | Particle Burst | Wave Burst | Total |  | S/C Mode |  | Rel. Allocation |  | Abs. Allocation |  | Hours | Seconds | Rate (bps) |  |
|  |  |  |  |  |  |  |  | T slow survey |  | 100% | T_orb | 100% | T_orb | 48 | 172800 | 469 | 81 |
|  | FGM | 5 | 10 | 6 | 2 | 23 |  | T fast survey |  | 22% | T_ss | 22% | T_orb | 10.6 | 38016 | 11893 | 452 |
|  | SCM | 4 | 19 | 25 | 19 | 68 |  | T P-burst |  | 10% | T_fs | 2.2% | T_orb | 1.06 | 3802 | 42837 | 163 |
|  | EFI | 9 | 26 | 33 | 26 | 94 |  | T W-burst1* |  | 10% | T_pb | 0.2% | T_orb | 0.11 | 380 | 124928 | 47 |
|  | SST | 28 | 107 | 42 | 0 | 176 |  | T W-burst2* |  | 3% | T_pb | 0.1% | T_orb | 0.03 | 101 | 468992 | 47 |
|  | ESA | 36 | 290 | 57 | 0 | 383 |  |  |  |  |  |  |  |  |  |  | 744 |
|  | Total | 81 | 452 | 163 | 47 | 744 |  |  |  |  |  |  |  |  |  |  |  |
|  | Compressed volume |  |  |  |  | 372 | Mbits |  |  |  |  |  |  |  |  |  |  |
|  | Rate/day |  |  |  |  | 186 | Mbits |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | S/C Mode durations / orbits |  |  |  |  |  |  |  |  |  |
|  |  | Mbits/ | 96 | hrs |  |  |  | T_orbit = | 96 | hrs |  |  |  |  |  |  | Data Vol (Mbits) |
| P1 | Instrument | Slow Survey | Fast Survey | Particle Burst | Wave Burst | Total |  | S/C Mode |  | Rel. Allocation |  | Abs. Allocation |  | Hours | Seconds | Rate (bps) |  |
|  |  |  |  |  |  |  |  | T slow survey |  | 100% | T_orb | 100% | T_orb | 96 | 345600 | 469 | 162 |
|  | FGM | 9 | 10 | 4 | 1 | 24 |  | T fast survey |  | 11% | T_ss | 11% | T_orb | 10.6 | 38016 | 11893 | 452 |
|  | SCM | 7 | 19 | 15 | 12 | 54 |  | T P-burst |  | 6% | T_fs | 0.7% | T_orb | 0.63 | 2281 | 42837 | 98 |
|  | EFI | 18 | 26 | 20 | 15 | 80 |  | T W-burst1* |  | 10% | T_pb | 0.1% | T_orb | 0.06 | 228 | 124928 | 28 |
|  | SST | 55 | 107 | 25 | 0 | 187 |  | T W-burst2* |  | 3% | T_pb | 0.0% | T_orb | 0.02 | 61 | 468992 | 28 |
|  | ESA | 72 | 290 | 34 | 0 | 396 |  |  |  |  |  |  |  |  |  |  | 741 |
|  | Total | 162 | 452 | 98 | 28 | 741 |  |  |  |  |  |  |  |  |  |  |  |
|  | Compressed volume |  |  |  |  | 370 | Mbits |  |  |  |  |  |  |  |  |  |  |
|  | Rate/day |  |  |  |  | 93 | Mbits |  |  |  |  |  |  |  |  |  |  |

DATA FLOW FROM DATA MANAGEMENT PLAN DOCUMENT

The THEMIS end use data products will include:

- 24 hour spacecraft data files in CDF format (1 per spacecraft, includes all science and engineering data)

- Ground Based Observatory (GBO) data files (Image files in png format and raw mag data files

- Education and Public Outreach (EPO) ground magnetometer data files (raw)

- 24 hour calibration file in CDF format (1 per spacecraft)

- Science Summary Plots (Calibration Check) in gif or jpeg format as well as CDF

- Science Summary Plots (Standard) in gif or jpeg format as well as CDF

Spacecraft Data Files

The primary end use data product will take the form of a 24 hour data file for each spacecraft, which will be converted into the NSSDC's Common Data Format (CDF). The process to produce these files is initiated after the Ground Station (GS) has delivered all of the data received for a particular contact. At [GS File Reception + 4 hours] a CDF file will be produced, but will not be available until the lead operations scientist (TOHBAN) has reviewed the science summary plots and approved the calibration files used to produce the CDF Data file. Following this approval [GS File Reception) + 24 hours], the CDF Data files should be available via a web interface or through a search tool which will be part of the THEMIS Data Analysis  Software Package available from UCB. In addition to the 24 hour spacecraft CDF Data files, there will be 24 hour CDF spacecraft Calibration files available at the same frequency.

Ground Based Observatories (GBO)

The Ground Based Observatories (GBO) will be producing all sky images in png format as well as ground magnetometer data. The image data will be downloaded to the University of Calgary (UC) and UCB will initiate a data mirror of the site. Depending on the link between UC and the GBO, it might not be possible to download the full data set. With this as an exception, it is estimated that UCB and the general public will have access to the images by [image creation + 5 days].

The ground magnetometer data will be downloaded to the University of Alberta and UCB will initiate a data mirror of the site. Since the magnetometer data stream between the GBO and UA is quite small, it is expected that the data will be available by [real-time measurement + 24 hours].

Education and Public Outreach (EPO) Ground Magnetometers

The EPO ground magnetometer data will be retrieved from the individual sites and will be available at UCB by [real-time measurement + 24 hours].

Summary Data

There will be two families of summary data plots produced. Each will contain plots of instrument, GBO, and EPO mag data. The first will contain more detailed science and state of health data, which will allow the TOHBAN to quickly assess the operation of each segment as well as validating the instrument calibration files. These plots will be available by [Ground Reception + 6 Hours] and can be viewed and downloaded via a web interface. The second family of summary plots will display data for each segment in more general terms and be available by ground reception + 24 hours.