QUESTION 10A: How does the IDPU change hardware implementation of heritage instruments?

Most of the THEMIS instruments have a heritage in which serial digital communication with either a central or instrument-specific data processor unit was used. Thus the majority of the THEMIS instruments can be used with the proposed IDPU with minimal or no changes.

EFI: An electronics board moves from each boom unit and into the IDPU (the BEB); thus unlike previous missions, none of the boom units will contain any electronics other than the sphere sensors themselves. This change adds ~4-5 lines/boom unit of additional interface harnessing between the IDPU and EFI sensors, however this change is easily offset by gains in power and mass efficiencies obtained by centralizing the +/-100V power supplies for the BEB system in the IDPU.

SCM: No changes.

FGM: No changes except in serial digital communications protocol.

ESA: No changes.

SST: There are significant changes from the SST design on WIND, however the THEMIS SST implementation is nearly identical to the SST instrument currently under flight development for the STEREO project and the knowledge to implement THEMIS is well in hand.

QUESTION 10B: How will you handle interface issues with sensor suppliers?

For most sensor suppliers, an early effort was made in the design process to keep the interface between sensors and the IPDU as minimal and clean as possible. In the case of the ESA, SST, and FGM, simple two or three-line digital interfaces demark a clean break between instrument and IDPU systems. This enables foreign collaborators in particular to engage in much of their own design and testing work at their home institution, and simplifies tasks during focused I&T activities at UCB and Swales.

Exceptions to this concept of interface simplicity include the EFI and DFB. Since the EFI is being built at Berkeley along with the IDPU, close interaction between EFI scientists and IDPU engineers will enable these more complex interfaces to be optimized and instituted via daily interaction.

The Digital Fields Board (DFB) is a complete, 6U-size board within the IDPU that will be designed of UCB by Co-I R. Ergun in Colorado, with interfaces to both the EFI and SCM. Parts procurement, fabrication, and testing will be performed at UCB using the standard instrument inventories and facilities.  Dr. Ergun constructed a very similar signal processing system for FAST while a scientist at UCB in cooperation with the current THEMIS IDPU engineer R. Abiad and THEMIS Co-I G. Delory. To facilitate the smooth implementation of all DFB interfaces, R. Ergun, IDPU engineer R. Abiad, and the SCM team will establish a joint ICD. Additionally, the choice of a standard 6U-VME style IDPU form factor simplifies the mechanical interface requirements for the DFB physical design.

QUESTION 10C: What is the planning for individual instrument delivery and integration?  How is this affected by the participation of international partners?

(See I&T schedule detail, next page).

The primary impact of the participation of international partners on the IDPU/Instrument I&T plan is the desire to minimize the amount of time foreign teams must be physically present. This translates into the requirement that I&T activities for foreign teams are scheduled in widely spaced but tightly focused blocks of time. Additionally, SSL personnel will be trained in the operation of instruments produced at outside institutions so that they may be operated in the absence of international Co-I members.

QUESTION 10D: What is the overall estimated reliability value for IDPU?

The design philosophy behind the IDPU development process includes:

- The use of grade 2 or higher parts for critical items such as the CPU board, certain blocks of memory, and power supplies, exceeding MIDEX guidelines

- Vigorous in-house testing program

- Built-in fault protection and recovery

The successful implementation of this design philosophy and manufacturing process control has been demonstrated during numerous past missions, including HESSI, FAST, and others. Based on the successful heritage of IDPU development, we plan to evaluate the overall reliability of the IDPU during design trade studies in phase B as part of the ongoing overall probe and mission reliability assessment within mission systems engineering.

QUESTION 10E: Why do you feel a qualification program is not necessary for the IDPU?

As discussed in the CSR Section F6, we have planned and budgeted for an IDPU qualification program prior to flight unit builds.  The Engineering Test Unit (ETU) or Engineering Model (EM) is flight-like and is subjected to a full suite of functional and performance characterizations under nominal and off-nominal operating and launch environments.  Our flight units will be acceptance tested relative to the ETU baseline performance.

Question 10C: Detailed I&T schedule