QUESTION 1: Please describe the mission profiles for each probe and for scenarios in which one probe replaces another in the constellation.  Include detailed delta-v profiles for each scenario including pertinent orbital parameters.  What is the operational sequence of events for moving an already-operating probe to replace a failed one?

- HIGH LEVEL NOMINAL MISSION PROFILE:

- Progress of all five Probes through all phases of the mission is shown in detail in the following Mission Profile Table (Expansion of CSR Figure F-1A that were too detailed to include due to space limitations)

- This mission profile takes advantage of the initial injection orbit to allow the 3rd-Stage/PCA to re-enter passively at end-of-mission, while maintaining a sufficiently stable (>1.8 yr. Lifetime) orbit for L&EO and probe initial checkout operations (30 days).

- The THEMIS Mission Modes are well defined and understood across all elements of the team at this early phase, allowing for the optimizations and operational simplicity achieved during the Phase A study.

- DETAILED NOMINAL MISSION DELTA-V PROFILES – BASIC PARAMETERS AND CONSTRAINTS:

There are several basic parameters for the delta-V profiles:

- mass properties of the THEMIS probes

- propulsion system required characteristics

- initial orbital elements at injection

- drift in orbital elements due to terrestrial and lunisolar perturbations

- constraints on end-of-mission orbits required to comply with the NASA orbital debris mitigation (all THEMIS space elements re-enter within 25 years or less) requirements.

These parameters were evaluated over a sensitivity range in order to establish their nominal and extrema values to provide the basis for the Mission Requirements that flow down to the flight, launch, and operations elements. These basic parameters are tabulated and enumerated below:

- Probe Masses (predicted base masses, w/out contingency):

Instrument Base Mass:  20.9 kg.

Bus Dry Base Mass:  41.0 kg.

Total Dry Base Mass:  61.9 kg.

Total Base Mass + Full Fuel Tank:  96.4 kg.

Maximum (mature) probe limit mass (wet):  134.0 kg.

- Propulsion:

Axial thrust burn efficiency:  98.0%.

Isp:  215 s.

Max. hydrazine @ 4:1 blowdown:  34.5 kg.

- Orbits -- Definitions and Required Adjustments:

- Table 1: Nominal Elements, Drifts, and LV Dispersion –

- This table presents the nominal perigee, period and inclination  (placement and target) for each of the probes.  It also shows the drift rates in inclination due to lunar perturbations for P1 and P2, as well as the perigee drift of all probes, based on GTDS simulations.  LV dispersion in perigee, apogee, and inclination is also included here.

- Table 2: Stable Perigee/Apogee for P1 and P2 Ascent Scenarios –

- This table presents the perigee and apogees chosen for the intermediate point in the P1 and P2 ascent scenarios.  These values were chose to mitigate the risk of a possibly unstable orbit caused by an aborted perigee burn.

Table 3: Required Period Adjustments, Probe-by-Probe –

- This table shows the delta-V required to adjust Probe orbital periods during the first two science seasons.  The orbital period of probes P1, P2 and P5 is adjusted to optimize constellation alignment during the science season and maximize the number of conjunction hours achieved by the constellation.

Table 4:  Re-Entry Requirements:

This table shows the end-of-mission perigees for all Probes to achieve the NASA orbital debris mitigation requirements.  Lunar perturbations are used to force re-entry of P1 and P2 within 10 yrs. by adjusting orbital period at end of mission. Atmospheric drag is used to force re-entry of P3-P5 within 10 yrs. by reducing perigee at end of mission.

- DETAILED NOMINAL DELTA-V PROFILES, PROBE-BY-PROBE:

PROBE 1:

- Inclination change from 23 to 7 degrees between maneuvers 4 and 5, and 8 and 9 is achieved passively, through lunar perturbations, with concomitant savings in fuel (see Table 1).

PROBE 2:

- Inclination change from 15 to 7 degrees between maneuvers 4 and 5, and 8 and 9 is achieved passively, through lunar perturbations, with concomitant savings in fuel (see Table 1).

NOMINAL DELTA-V PROFILE (con’t):

PROBES 3, 4, AND 5:

- Fuel requirements for orbital placement of P1 and identical Probe design lead to large fuel margins on P3-P5,with largest on P3 and P4.

- Large fuel margins on P3 and P4 allow for either probe to replace any of the other probes at any point in the mission.

- DELTA-V PROFILE -- PROBE REPLACEMENT SCENARIO DETAILED MANEUVER SEQUENCE:

- This represents the worst case (i.e. replacing P1 by P3 or P4 to bound maximum energy/propellant for replacement maneuvers)

- Assumes that all Probes are fully deployed at their nominal mission orbits before P3 or P4 are “called into duty” to replace P1 (worst case operational assumption).

- Even after full replacement operations are completed, positive fuel margin still exists on the replacement probe (P3 or P4).

- Replacement probe operations include end of mission maneuver to satisfy orbital debris requirements even in these contingency modes.

- Operations for Probe Replacement Scenario are identical to operations for other maneuvers (see answer to Question 2 for thrusting maneuver operations sequence details).