QUESTION 7: Please specify the worst-case data storage requirement for each probe. Clarify the average data volume of 750 Mbits – Is it per day or per orbit?  If per day: (a) Is the data storage sufficient to store 4 days of data plus a missed contact for Probe 1; and (b) How are overlaps in coverage of the probes handled at the ground station?

Each Probe accumulates 750 Mbits of science and engineering data per orbit. The science instruments are turned on near apogee only, where the conjunctions occur. Lossless compression by a factor of 2 is applied onboard, so that 375 Mbits of data have to be stored on each orbit. The onboard memory has a capacity of 256 Mbytes, thus allowing data storage for more than 5 orbits.

When data are transmitted to the ground, an overhead of 12% for CCSDS frame formatting and 14% for Reed-Solomon code symbols is applied. The resulting total telemetry data volume amounts to approximately 480 Mbits per orbit per Probe, or 1.44 − 2.4 Gbits per day for all 5 Probes combined.

The required downlink time per orbit is 20 minutes at a rate of 400 kbps for Probe ranges below 20,000 km. The available downlink time at the Berkeley Ground Station and at Universal Space Network’s Perth, Australia ground station each have a 60 − 100% margin, as is illustrated in the table below, which was reproduced from the Concept Study Report. This large margin allows for playback of data from missed passes and also flexibility to resolve pass scheduling conflicts between the Probes. However, since the three inner Probes are dispersed in mean anomaly, the overlap of passes at either ground station is rather small to begin with. Typically, neither Probe 1 nor Probe 2 conflicts with Probes 3 − 5.

| Probe | Mission Required Link Access [min/year] | BGS Available Link Access [min/year] | USN-AU Available Link Access [min/year] |
| --- | --- | --- | --- |
| 1 | 1825 | 3740 | 3663 |
| 2 | 3650 | 6248 | 6952 |
| 3 | 7300 | 12153 | 11990 |
| 4 | 7300 | 12606 | 11925 |
| 5 | 7300 | 12723 | 14793 |

In addition to the 20-minute passes used for science and engineering data recovery, each Probe will be contacted at least once per day to obtain Doppler tracking data for orbit determination purposes and to monitor the state-of-health of the Probe bus and all instruments.

As additional contingency, THEMIS telemetry data may also be recovered at Universal Space Network’s South Point, Hawaii ground station.