QUESTION 3A:

Describe in detail the system pointing error analysis and the precession maneuver analysis including the type of analysis performed, the analysis assumptions, and the error sources, budgets, etc. that substantiate the pointing and stability capabilities presented.

Pointing Requirements and Budget

The THEMIS Mission requires pointing control and knowledge for both the spin axis and the instantaneous orientation of each instrument. Figure ACS1 illustrates the relationships between instruments and the orientation of the spacecraft with respect to the plane of the ecliptic. Details of the requirements are as follows:

Figure ACS1:  Pointing line-of-sights associated with each of the THEMIS instruments.

Control:  The absolute error between the ground commanded observation plane normal, zObsPlane, and each of the individual z-axis reference frames for each instrument, shall lie no further than 11.25 degrees, 3-sigma, from one another.  Since this is a single axis requirement, a single angular quantity can be used to show compliance. None of these errors shall exceed 11.25 degrees except under the following conditions:  periods lying closely to the entry and exit of eclipses, and, periods during and closely following station keeping maneuvers.

This requirement shall encompass the following:

-	Errors between the Observation Plane and the instantaneous Probe momentum vector

-	Principal axis offset of the aggregate Probe

-	Any Boom flexure resulting from mass property asymmetries

-	Wire Boom alignment perpendicular to momentum vector

-	Body reference to instrument alignment errors no measured on ground.

-	Controller servo errors and Estimation errors

Attitude Knowledge:  Unlike control accuracy and stability, the knowledge requirement is a full three-axis requirement, and is stated as follows:  the instantaneous attitude knowledge error shall be less than 1 degree, 3 sigma, in each axis.  This error is a measurement between truth and ground-based attitude estimation, relating the Probe body frame and the ECI frame.

Significant portions of instrument misalignment are fixed and will be calibrated by the science team during on-orbit operations.

Attitude Stability:  Attitude stability is defined as the variation of the spacecraft spin axis over specified periods of time. The quantities held to this requirement are the z-axis vector directions of each of the instruments.  These errors shall not deviate by more than 0.1 degrees, 3-sigma, over any 12 hour period excluding those involving sun or moon eclipses and station keeping maneuvers.  Furthermore, these errors shall not deviate by more than 1.0 degrees, 3 sigma, over any 5 day period excluding those involving sun or moon eclipses and station keeping maneuvers.

# Spacecraft Pointing Budget and Subsystem Allocation:

Table ACS1 summarizes the pointing budget used to validate the spacecraft performance against mission requirements.  Each of the instruments are grouped together by likeness of mounting method and carried as such in the table delineating between the various dynamics events.  For example, the wire booms tend to closely self-align with the principal axis of the spacecraft while the EFI’s on the axial booms will experience deviation due to boom flexure caused by this same misalignment.

Table ACS 1 also lists several notes explaining the sources of the error terms.

Table ACS1.  Pointing Budget and Subsystem Allocation

ACS Subsystem Analysis Description

Analysis for the ACS Subsystem was completed using two separate tools:

- Multi-Body, time domain simulation for attitude estimation, assuming

- Ground based estimation routines for attitude determination processing.

Figure ACS2:  Attitude estimation scheme based upon using two measurement points separated by an extended period of time.

The use of these tools, along with the parameters used to drive  them follows.

Estimation:

On-board processing reads, time-tags, and telemeters sensor data for ground-based algorithms. Two attitude estimation schemes were analyzed: MSSS only and the MSSS with the Flux Gate Magnetometer (FGM).  Both approaches used a recursive, least squares algorithms with an initial “guess” based upon a TRIAD solution computed at the beginning of the run.  Analysis was completed demonstrating the performance of both schemes over a wide range of attitude orientations with respect to the plane of the sun the ecliptic.

Figure ACS 3: Estimation Performance for the MSSS-only and MSSS/FGM configurations

The MSSS provides precision sun vector line-of-sight vector (2 axes) comprised of an azimuth angle of the sun with respect to the spacecraft X-Y plane, and a sun crossing reference time corresponding to sun angle in this plane. Because a third axis is required for attitude determination, two sun sensor readings are required separated by time in order to allow the apparent motion of the sun, approximately 1 degree per day, to manifest the third angle of rotation.  Figure ACS 2 depicts the geometry involved with this estimation scheme. The two azimuth readings taken at t1 and t2 move with respect to one another as a function of the time separation, and the angle of the ecliptic with respect to the line-of-sight reading from the MSSS. Figure ACS 3 depicts performance of the MSSS over 30 day observations.  Anticipated on-orbit attitudes consistent with the mission profile are contained in the red circle whose diameter is 11.25 degrees.  The worst case error is 0.39 degrees, which does improve over periods beyond 30 days. The angle labeled Azimuth denotes the elevation with respect to the spacecraft X/Y plane and the Bore Sight Angle is the orientation of the ecliptic plane about the measured line of sight.  Note: the 11.25 degrees is based upon the worst-case attitude of the spin axis with respect to the plane of the ecliptic, manifested in Azimuth, Bore-Sight or a combination of the two.

In order to provide a more expeditious means of three-axis estimation, the FGM data was made available from the instrument DPU in order to augment the ground estimation scheme. A batch TRIAD algorithm is used. Figure ACS 4 also includes the performance for this two-sensor scheme over a two hour period lying closely to the point of perigee. Both sets of analysis include sensor misalignment, noise, measurement inaccuracies, and data latencies.  The error range for the MSSS scheme is narrower than that of the TRIAD approach since is it will be relied during the science portion only, with the latter providing a more robust operating envelope for mission operations.  The 11.25-degree operational zone is also shown for the TRIAD scheme.

Note the data contained in Figure ACS 3 for the MSSS/FGM combination is not symmetric about the 0/0 angle points.  This is due to the alignment of the earth magnetic field in inertial space for this run.  This particular scenario represents the worst case for estimator convergence.

Two single axis gyros aligned along the spin transverse spin axes provides short-term attitude precession verification useful prior to large orbit maneuvers. The QRS11 gyro is sampled at 10 hz, integrated and telemetered down at 1 hz for ground verification. This accommodates scenarios requiring large tips for apogee maneuvers. The tip-burn-tip sequence can be executed safely within a couple of hours in order to minimize the thermal effects of moving the spin axis relative to the sun line.  Performance of this gyro integration scheme during precession maneuvers is included in the analysis section that follows.

Thruster Control:

 Thruster control is implemented using 4-1 N thrusters, 2 oriented axially for primary delta-v in the +Z direction spin axis orientation, 2 oriented tangentially for spin control and side delta-velocity thrusting during science operations. A minimum pulse width of 1 second is assumed for sizing and pulse placement. Table ACS 2 lists the nominal forces and torques (unit magnitude) available assuming the baseline spacecraft layout. The tangential thrusters are intentionally placed on either side of the spacecraft center-of-mass in the z-axis in order to decrease the transverse torque disturbance observed during simultaneous firings for side thrusting.

Table ACS2: Nominal Thruster Layout

|  | A1 | T1 | A2 | T2 |
| --- | --- | --- | --- | --- |
| X (m) | 0.406 | 0.436 | -0.392 | -0.311 |
| Y (m) | -0.406 | -0.319 | 0.392 | 0.441 |
| Z (m) | 0.046 | 0.135 | 0.046 | 0.526 |
| F, X (N) | 0.000 | -0.707 | 0.000 | -0.707 |
| F, Y (N) | 0.000 | -0.707 | 0.000 | -0.707 |
| F, Z (N) | 1.000 | 0.000 | 1.000 | 0.000 |
| T, X (N-m) | -0.406 | -0.102 | 0.392 | 0.174 |
| T, Y (N-m) | -0.406 | 0.102 | 0.392 | -0.174 |
| T, Z (N-m) | 0.000 | -0.534 | 0.000 | 0.531 |

Figure ACS 4: Attitude error build up during 75 degree precession maneuver.

Thruster commanding is accomplished via unique ground computed sequences capable of three modes of operation:

- Sun synchronous pulsing triggered by the MSSS sun-crossing indicator. Parameters include thruster selection, pulse width, number of pulses, phase angle of pulse start with respect to the sun crossing, and the start time of the maneuver. This mode is used for spin axis orientation maneuvers before and after station keeping burns. Maneuvers using the tangential thrusters will also make use of this mode in order to execute “side” thrusting in the deployed configuration.

- Continuous fire commanding will be used for the major orbit raising and inclination maneuvers. This mode is commanded with parameters specifying thruster selection, fire time, and absolute time of burn.

- Thruster pulse train comprised of thruster selection, number of pulses, pulse width, delay time between pulses and start time for the sequence.  This mode is available in order to provide a more robust pulsing scheme capable of dead-beating nutation or wire boom librations during spin adjusts.

Like the sensor processing software, this logic runs at a 10 hz update rate.

Simple Algorithm for Precession Maneuver Calculations:

A simple algorithm for computing precession commands is given below:

The momentum equation for attitude dynamics is defined as

If the spin vector() and the angular momentum vector() is assumed to be parallel, then the momentum equation becomes

With the control torque () for the short interval time () with the thrust torque command, the momentum equation can be computed by the following equation.

Here the command torque () can be achieved by the thruster force () and the thruster moment arm ().

.

Where the thruster force () is parallel to the momentum vector ().

The radius vector at the thruster firing time instance() can be computed by the following equation.

where the cap denotes the unit vector of the capped quantity.

Ground-based Command Generation and Monitoring: 

In addition to the attitude determination processing, orbit determination is accomplished on the ground via coherent ranging with the spacecraft RF subsystem. As a result, orbit augmentation is planned and a set of thruster commands and mode selection is generated. Additional commanding is required to actually arm and execute each sequence.  As all of the burns will be executed while in view of the ground operations center, sufficient telemetry tools will be provided in order to monitor performance real time.

In addition to the command safe guards in place, a simple fault protection algorithm is present within the on board software in order to autonomously shutdown any active thruster sequence. This logic monitors the real time sun aspect angle and the spin period and compares them to a ground commanded reference.  If they fall outside a commanded threshold, the maneuver will be terminated. These thresholds will be commanded for each thruster maneuver and design such that spacecraft safety is preserved given any early termination condition. By relying upon the dynamics of spin axis stability, and operational and onboard safe guards, the implementation of an autonomous acquisition module is avoided.

Each orbit augmentation burn will be comprised of the following command sequences:

- Thruster pulse train sequence for the pre-burn tip maneuver to be synchronized with the MSSS sun crossing. Fault protection thresholds for the spin rate will also be generated.  Depending upon the size of the tip, a set of parameters for the aspect angle will also be generated.

- Continuous fire thruster sequence is generated for the axial thrusters.  If side thrusting is to be executed (science operations only) then a pulse-modulated sequence using thrusters T1 and T2 will be uploaded.  Fault protection parameters will also be updated.

- Thruster pulse train and fault protection parameters will be uploaded for the tip back to safe attitude.

- Spin adjust sequence via continuous or pulse modulation will be executed given the actual performance of the above sequence.

Each step of this burn sequence will be closely monitored and used in the planning of the next step. Also, through subsequent orbit determination solutions, the thruster can be calibrated in order to account for off-nominal thruster performance.

ACS Dynamics and Control Analysis:

A high fidelity simulation was built in order to include the following dynamics effects:  6 degree of free motion relevant to a spin stabilized spacecraft, wire booms behaving as rigid rods with two degrees of freedom at the base, 6 rigid booms orientated on the spacecraft consistent with the axial EFI’s and the magnetometer booms, MSSS and TAM models, and a simple thruster model producing force and torque reactions.  All aspects of this model were validated against analysis related to each appendage, often based upon “first principals.”

Probe Coordinates and Unit Numbering:

Figure 2 shows the coordinate system to be utilized for the each Probe Carrier.  The primary features are as follows:

- Origin location.  The coordinate system is placed at the geometric center of the separation plane of the probe.  The lowest reaching portion of the attachment ring, whose center is aligned with the Z-Axis, defines this plane.  Like the Probe Carrier, this single, unambiguous point shall be identified with a sufficient number of fixed features in order to make a proper physical reference point for integration purposes.

- Z-Axis.  This axis is “up”, normal to the Probe separation plane.

- X-Axis.  This axis is aligned normal to the face housing Wire Boom #1.  This face has an adjacent corner, moving counter clock-wise, housing the ESA.  In other words the ESA shall fall in the +X/+Y quadrant of the Probe coordinate system.  Note: This was chosen so as to provide an obvious indicator o the X/Y orientation when sitting on the floor during integration operations.

- Y-Axis.  This axis completes a right-handed orthogonal triad.

Mass Properties Used for Analysis:

Two sets of mass properties were used in order to access performance of the control system:

These were computed using the expected dry mass properties, with two fuel “bodies” distributed outboard and perpendicular to the spin axis.  The cross products of inertias for the stowed case are assumed to be 0.025 kg-m-m based upon conservative performance of available spin balance tables (i.e. Miller Table).

QUESTION 3B: Discuss the simulation results shown in Table F-8.  Specifically, what were the start and end conditions for each case and do these conditions correlate to the expected operational configurations?

Mission Phase Validation: In order to prove feasibility of the design as applied to the full mission profile for THEMIS, a simulation run matrix tracing through the various phases was executed. Table ACS 4 outlines the runs that were used to bound all performance characterization. These results show the feasibility of the ACS subsystem, and its capability to execute the initial ascent, deployment and science operations.  Values shown in RED depicted updates not reflected in the Phase A Concept Study Report

Peak errors annotated in the right hand columns in order to characterize: peak momentum vector pointing perturbation as a total angle from the initial attitude, spin rate error introduced relative to the initial spin rate, delta velocity error introduced in the body-z axis, delta-velocity error introduced in the body x-y transverse axes, and maximum wire boom deflections from the nominal radial orientation as a total angle combining in-plane and out-of-plane deflections.

The runs sets are discussed by each of the five major groupings.

- Orbit Raising Operations in Stowed Configuration. These runs show the ability to do large attitude precession and orbit raising maneuvers. A 75 degree precession is demonstrated in order to show a worst case condition for the control system.  In all cases, burn attitude was verified to better than 5 degrees using the integrated QRS11 gyro signal. Figure ACS 4 shows the attitude error build up during this attitude maneuver. The burn times reflected twice the longest burn required for P1. 

Cases 1a and 1b were used to show the insensitivity of control to spin rate.  The worst-case angular error of the momentum vector was found to be well within the 5 degree errors assumed in the propellant budgets.

This series also included delta-velocity burns using the axial thrusters.  The burn lengths were chosen to be twice the maximum burn length for P1, the bounding case.

Finally, a spin down from 30 RPM to 15 RPM was demonstrated mimicking the operation prior to magnetometer boom deployments.   Results were benign.

- Magnetometer-Boom Deployment and spin-up.  Case 4 and 5a concentrated on spin axis stability verification considering each boom deployed individually. Favorable inertia ratios are provided in Table ACS-4.  Time domain simulations were not exercised as the results were attained via simple calculations.

Case 5b, on the other hand, used the simulation tool in verify spin up from 12.3 RPM to 30 RPM, the operation required prior to wire boom deployment.  This case also rendered benign errors.

- Nominal Wire Boom Deployment. The spin-up procedure required during wire boom spin-up was assessed in these runs. Booms are deployed in pairs in two stages in order to capture a simple sequence. Spin up performance to the desired 30-RPM rate, and worst-case wire boom deflection is provided.  The most exciting case was found in Case 6a, Wire Boom 1 and 2 halfway out.  Large wire boom deflections were observed to get to 21 degrees, but damp out over several minutes.  The other cases were more and more benign as the aggregate wire boom configuration progressed.  Continuous thruster commanding was used in each of these cases, which makes the analysis quite conservative.  Alternatively, pulse modulated firing synchronized to dead-beat the primary libration mode could be used to reduce the angular deflections if deemed necessary.

Figure ACS 5: Performance during 30 degree precession maneuver, with wire damping.

Off-Nominal Wire Boom Deployment. These runs considered worst-case dynamics conditions resulting from possible failure scenarios.  Here, each boom was deployed individually in order to inject worst-case mass property asymmetry. Spin-up maneuvers were performed consistent with the previous set of runs.  Case 6b, Wire Boom 1 completely deployed by itself, proved to be the only challenging test case.  A modulated thruster pulse-train was used to dead-beat the libration, resulting in acceptable results.  Regardless of the unlikelihood of the scenarios explored in this series of runs, the results show how truly robust the THEMIS control scheme is.

- Axial Boom Deployment.  These cases represent another close-form stability analysis, with the resulting inertia ratios provided in Table ACS-4.

- Science Mode Operations and Passive Boom Libration Damping.  Precession and delta-velocity maneuvers were executed on the fully deployed spacecraft, based upon twice the longest maneuver required by P1 during the science operations (Like the initial orbit raising sequence, the P1 probe is the bounding case). Burns were executed via T1/T2 side thrusting and A1/A2 axial thrusting.  Also, the precession maneuvers simulated bound the attitude adjustments required every three months to maintain relative probe to sun tilt relationships during science operations. Figure ACS5 shows a 30 degree tip.  This run shows the damping impact of the wire booms.

Passive Nutation Damping: 

Since it is assumed nutation damping is accomplished through passive means, a simple analysis was done to ascertain damping time constants for the following cases:  fuel damping on stowed spacecraft with 75% and 25 % fill fraction, fuel damping on deployed spacecraft case with 25 % fill, and, wire boom damping on deployed spacecraft. The effects of fuel and wire damping were not combined so as to retain a conservative estimate. In the case of fuel damping with a 25% fill fraction, an initial nutation angle of 10 degrees damps below 0.1 degrees in less than 5 minutes stowed and less than 12 minutes fully deployed. For wire booms a conservative damping ratio of 0.05% was used to assess the time required to damp out an initial 30-degree wire boom deflection in symmetric and asymmetric orientations.

Figure ACS 6 depicts these two runs:  the top plot shows the symmetric case where linear velocity oscillates in response to the libration, eventually damping out, and the bottom plot shows how the asymmetric case is revealed in the spin rate undulation.

Spin rate of 10 RPM oscillating due to 30 degree boom deflection

Linear velocity versus time, with +/- y wire booms deflected 30 degrees back (symmetric).  X velocity eventually damps out with 0.5 % damping at the boom attach points

Figure ACS 6: Wire Boom damping for symmetric and asymmetric boom deflections.

QUESTION 3C: Discuss the impact of a failed latch valve on the control accuracy.

In the event of a failed latch valve, one propulsion tanks’ fuel would not be accessible for the duration of the mission.  As a result, asymmetric mass properties would be present due to the consumption of fuel out of the remaining available tank.  As this condition would be known at the beginning of the mission (latch valves are opened after launch and never closed) such a failure would direct the mission team to select this particular anomalous probe to fill either the P3 or P4 slot.  These probes only use a maximum of 3.13 kg of fuel (nominal), resulting in a small mass property perturbation.  The resulting central body mass properties are given below:

Using these results, a series of simulations were re-run in order to demonstrate the accuracy of the control system.  These runs are summarized in the Table ACS 5, and compared to those same runs executed using symmetric mass properties.

Figure ACS 7: Test Case 1A with failed Latch Valve, 75 degree precession.

As can be quickly observed, control accuracy is slightly impacted and well within requirements.  Case 1a reveals a large momentum vector error at 11 degrees.  This is due to the large mass property uncertainty between the ground and reality.  However, the estimation accuracy is still around 1 degree, allowing the ground to identify this error during and after the burn.  This is shown in the Figure ACS 7.  In reality, the ground would have a better idea of the mass imbalance and update the mass property estimation in order to reduce these errors during the initial burn.

The major reason for the observed performance insensitivity to this mass property asymmetry in the effect the deployed booms have on the aggregate principal axis.  The wire booms “de-magnify” any central body offset by applying a large torque that acts to straighten out the spin axis towards the booms axis of symmetry.  A detailed explanation of this effect follows.

# Relationships among the rigid appendage deflection, the angular velocity vector, and center body principal misalignment angle:

Consider the following Euler equation for the rigid body rotational dynamics:

In case of no external torque (), a steady-state equilibrium condition () makes .

If the initial angular rate and mass configuration (moment of inertia, ) are not in an equilibrium state, the dynamics with damping will converge to the equilibrium state.

Since the analytical equilibrium state for the configuration with four rigid appendages is hard to find due to the moment of inertia’s dependency of the appendage deflections, we compute the steady state from the results of CASE 15’s hi-fi simulation. The computed equilibrium state is verified by the following procedures.

1) Check the initial equilibrium condition

,

The above non-zero  implies that the initial condition is not an equilibrium state.

2) Check : the following simulation result with 2000 sec illustrates that  converges to zero.

,

Here the nutation angle is . The steady state out-of-plane appendage deflection angles based on the simulation are about 0.43 degree, which confirms that the steady state appendage deflections consistent with the nutation angle.

The figure to the right illustrates the final principal offset angle with respect to center body principal axes offset angle about x-axis only. The figure is based on the simulation with 20rpm spin, 0.5% damping, and 30o out-of-plain initial appendage deflection.