QUESTION 2: Present the maneuver operations plan for both the large delta-v maneuvers and the side thrusting maneuvers including hardware configurations, commanding, and dynamic stability concerns?

- The maneuver operations plan describes the attitude and orbit adjustment procedures required to place all Probes into their initial science orbits and to perform infrequent orbit adjustments so that conjunction time is optimized throughout the mission.

- All attitude and orbit maneuvers consist of a sequence of firings of a subset of the four thrusters on each Probe, in either pulse or continuous mode. The details of the thruster firing sequence depend on the particular maneuver and are generated by GMAN.

- All maneuvers are performed by executing a time sequence of commands stored onboard while the Probe is in ground contact. Maneuvers executed near perigee will be monitored via a TDRSS two-way link (see answer to Question # 12).

- All maneuvers follow a common procedure:

            - Define initial (pre-maneuver) and target (post-maneuver) Probe state vector and Probe attitude

            - Verify Probe parameter table that is used as input to GMAN

            - Run GMAN to determine optimum thruster sequence and Probe trajectory, including attitude reorientation maneuvers

            - Review proposed trajectory and Probe attitude evolution with SatTrack 3-D visualization tool and determine available station and/or TDRSS view periods

            - Confirm availability of required ground station and/or TDRSS resources

            - Develop detailed event timeline, including Probe states and ground assets

            - Generate and verify Probe memory load (time sequence of stored on-board commands for subsystem configuration, thruster firings and subsystem de-configuration)

            - Perform command execution validation test with the probe testbed simulator to ensure proper thruster and duration results

            - Perform end-to-end data flow tests with all ground stations and/or TDRSS elements involved in a particular maneuver

            - Load thruster firing sequence into Probe memory

            - Verify load in Probe memory

            - Verify two-way communications with Probe

            - Verify pre-maneuver attitude

            - Configure and verify state and performance of subsystems (gyros, key temperatures, tank pressure)

            - Monitor execution of the firing sequence in real-time (key temperature sensors, gyros, attitude vector)

            - If an anomaly occurs, abort the firing sequence either by the automated on-board fault detection system, or by real-time ground command

            - After successful execution of the firing sequence verify de-configuration and state of subsystems (gyros, key temperatures, tank pressure)

            - Determine post-maneuver attitude and state vector and compare with target

            - Determine if subsequent maneuver to adjust attitude and/or state vector is required

- Under configurations of deployment and failure modes, the Probes have positive passive spin-stability margin resulting in no concerns for dynamic stability (See answers to Questions # 1, 3, and 9 for additional detailed information on Probe replacement contingency operations, fuel imbalance worst case effects, reliability and nominal and failure mode effects of appendage deployments that demonstrate this).

- Side thrusting and axial thrusting have convergent wire boom damping and do not excite unfavorable oscillations as has been demonstrated on the POLAR mission for fully deployed satellite precession to burn attitude and back with highly damped wire boom modes.

- The table at the end of this writeup contains the complete maneuver sequence for Probe 1 orbit insertion, which involves the largest number of individual maneuvers, to illustrate the fidelity of our operational simulation and understanding.

- The following flowchart figure summarizes the major ground and flight operations functions planned to implement the strategy described.  This sequence is employed for either a typical axial or side thrusting single maneuver event:

Flight Operations Functions

Maneuver Operations (Typical) Sequence

Ground Operations Functions

“Look Ahead” Orbit Propagation Indicates Upcoming Maintenance Maneuver is Desired

Current/Desired Orbit/Attitude used in GMAN

Specific Maneuver Events, Attitudes, and Durations Formulated

Discrete Thruster Profile and Pulse Sequence Formed

Discrete Stored Command Sequence Generated via MPS

Upload TLM Table, Firing Sequence, & Downlink Rate Selection

Offline Validation of Entire Stored Command Sequence Performed on Probe Hi-Fidelity Test-bed

Power-up Gyro’s, Catalyst Bed Heaters (Pre-Heat)

Downlink On-Board CMD Buffer

Compare Flight to Ground Reference Image to Verify Proper Sequence Load

Verify CMD & TLM link via GN, TDRSS, USN, or DSN

Verify Current Attitude via Sun Sensor Data

Verify Gyro Performance, Catalyst Bed Heater Functionality, Propellant Tank Pressure, Valve/Fuel Line Temperatures & States, and Pre-Maneuver Attitude

Power-off Catalyst Bed Heaters

Execute Burn Sequence

Periodic Long-Term Calibration of Pulse Timing and Thruster Performance

Monitor Key Temperatures, Attitude, and State Vector

On-Board Failure Detection/Correction (FDC) Logic (gyro rates, sun-sensor attitude limits, etc) Aborts Sequence if Anomaly is Detected

Power Down Gyros

Verify Tank Pressure, General Probe Health & Safety, 2-Way Ranging

Turn Off Transmitter

Determine New Orbit and New Attitude