QUESTION 5: The Master Equipment List shows several components (e.g., gyros, thermostats, valves, transducers, RCS heaters) at zero power required.  How are these accounted for in the power analysis?

The Master Equipment List has average power dissipations for the nominal science mode as the probes spend the vast majority of their lifetimes operating in that mode.  The table below denotes peak and average power dissipation for each component questioned.

| MASTER EQUIPMENT LIST ZERO POWER ITEMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Vendor/Model | Peak Pwr | Ave Pwr | Duty Cycle | Comments |
| Gyro's | Systron Donner QRS11 | 1.6 W | 0 W | 0% | Both only on during thruster firings |
| Thermostats | Elmwood | 0 W | 0 W | N/A | Passive device |
| Latch Valve | Vacco V1E1074701 | 21.8 W | 0 W | 0% | 500 ms pulse each, one time actuation as part of initial checkout |
| Thruster Valve Assy | GD MR-103G | 16.4 W | 0 W | 0% | 100% duty cycle for any thruster pair, only on during thruster firings |
| Thruster Cat Bed Heaters | GD MR-103G | 6.5 W | 0 W | 0% | Same for axial or tangential pair, only on for 30 min prior to a burn |
| Pressure Transducer | Paine 2137626002 | 1.12 W | 0 W | 0% | Only on for a 5 minute sample per orbit |
| RCS Heaters | Tayco | 6 W | 2 W | 33% | Book kept under thermal |

Each of the two gyro’s consume 0.8 W for the 1.6 W total shown and are switched on only during thruster firings.  Thermostats are passive devices that consume no power.  Each of our two latch valves consume 21.8 W during activation but are operated only once as part of each probe’s initial checkout.  Thruster valves are rated at 8.2 W each but only the axial pair are expected to operate at 100% duty cycle.  Each thruster cat bed heater consumes 3.25 W for approximately thirty minutes and are nominally used in pairs (radial or axial).  Both the thruster valves and cat bed heaters are activated only during thruster firings periods and are normally off.  The pressure transducer is used for long term trending and is activated only just prior to the nominal ground contact period once per orbit.  RCS heater budget is book kept under the thermal subsystem that has a twowatt average power allocation during nominal science operations.

As stated above these components have a very close to zero average power dissipation in the nominal science mode power analysis except for the RCS heaters that were accounted for under the thermal subsystem.  Our power analysis also accounted for thruster operations but that level of detail was omitted from the CSR due to page limitations.

Our thruster operations power analysis is based on mature power numbers and includes the following; probe bus, all instruments on, one pair of cat bed heaters on for 30 minutes, one pair of thrusters on 100% duty cycle, transmitter on 100%, EOL solar array performance, and worst case sun angles (sun on the top or bottom deck only).  At the beginning of phase A, the mission systems requirement set a working time for the longest planned burn time at 60 minutes;  This yields a 21% battery DOD.  As phase A progressed, the longest burn (P1) was calculated to be 72 minutes, which can be broken into two <60 minute events to satisfy the requirement.  However, our actual calculated performance, limiting battery DOD to 50%, yields a maximum allowable burn time of approximately 2.5 hours, far beyond that needed to satisfy our maneuver profile so we feel that we have plenty of margin to increase operational flexibility and can accomplish this longest burn event in the most efficient way possible (single burn) to expedite operations.  Note additional power margin can be achieved by powering down the instruments as we don’t acquire valid science data during thruster operations.