QUESTION 15. Many of the cited heritage instruments were significantly more expensive than the proposed instruments. What are the technical differences and why are you confident in the cost proposed for the THEMIS instruments?  Please explain the THEMIS cost versus mass calculation.

Technical Differences.  Naturally, there are a number of simple changes to make these sensors fit the THEMIS probe mechanically and electrically. For example, the FGM circuit needs to interface to the THEMIS computer instead of the ROMAP computer, and thus, there are trivial differences in connector type, pinout, etc. At the same time, improvements in some parts will allow substantially smaller, lighter and easier to use sensors. For example, the booms are changing from the pyrotechnic devices to resettable Shaped Memory Alloy (SMA) devices which UCB developed for flight use in HESSI, Lunar Prospector and other programs.

Instrument Mass .v. Cost Modeling. Over the past 20 years or so, UCB/SSL has produced a number of flight instruments for NASA and USAF. When available, cost data from these projects was collected and reduced to determine approximate cost by weight and by complexity. These projects were reduced to the following categories:

- EBOX : Electronics Box (src EFI & HESSI)

- MECH : Mechanism with Few Electronics Parts (src Cluster)

- SENSOR : Mixed Mechanical and Electronic Parts (src FAST)

Using these simple categories, the THEMIS sensor cost was compared to the grass roots budget.  The largest discrepancies were the SST and EFI booms, and in both cases, the grass roots estimate suggests there is sufficient funding for the effort. Overall, the grass roots budget matches the model within 6%.

Heritage Instruments Costs .v. THEMIS.  Generally, these heritage instruments were developed in large NASA programs, in which the contracts had a greater scope of effort than that in the named THEMIS WBS element. Typically, the efforts in previous programs included:

- Science, Management, Systems Engineering, Quality Assurance

- Mission Operations and Data Analysis

- Significant Instrument Design and Development

- Sensor Fabrication and Test

- Instrument Main Electronics

- Instrument Flight Software

In the THEMIS organization, however, we have coordinated the main electronics and flight software efforts for all the sensors, centralized the management, QA, scientists, etc., as well as the mission operations, etc. The selection of heritage components reduced the development costs. So, a given sensor WBS includes only the following items:

- Engineering for Interface-driven Modifications

- Sensor Fabrication and Test in Quantity

Finally, keeping the development schedule reasonable is an essential ingredient in keeping the costs down. Heritage instrumentation such as Polar EFI had a 9 year BCD schedule and Cluster had an 11 year BCD schedule, whereas we know from Cluster II that 1.5 FTE can assemble and test 1 boom per month. Clearly, the THEMIS use of heritage devices compresses the schedule and provides substantial savings.