QUESTION 12: Describe the use of TDRSS during early mission operations and quantify the dependency that THEMIS has on the TDRSS and DSN-Madrid networks. Have you confirmed that these resources are available?

TDRSS H / I / J support is required for the launch and ascent-to-orbit phase of the mission. Probe state-of-health can be monitored as soon as the fairing is opened. Also, the release of the Probes from the PCA can be monitored.

During the L&EO phase of the mission, TDRSS support is required to ensure two-way communications during maneuvers that are executed near perigee.

The total number of TDRSS supports is summarized in the table below. The quoted support durations include 20 minutes of overhead for each support to allow for TDRSS system set-up.

| TDRSS Support Summary |  |  |
| --- | --- | --- |
| PCA | Probe Monitoring During Ascent Phase | 6.0 h |
| Probe 1 | 4 Apogee Raise Maneuvers Performed Near Perigee @ 1.25 h Each | 5.0 h |
| Probe 2 | 3 Apogee Raise Maneuvers Performed Near Perigee @ 1.0 h Each | 3.0 h |
| Probe 3 | 1 Apogee Trim Performed Near Perigee @ 1.0 h Each | 1.0 h |
| Probe 4 | 1 Apogee Trim Performed Near Perigee @ 1.0 h Each | 1.0 h |
| Probe 5 | 4 Period Change Maneuvers Performed Near Perigee @ 0.7 – 1.7 h Each | 5.0 h |
| Total Support | Entire Mission Duration | 21.0 h |

The TDRSS support requirements are rather basic. The THEMIS probes carry a TDRSS compatible transponder that has been flown previously on FUSE and EO-1, and allows monitoring and control of the Probes via the S-Band Single Access (SSA) forward and return service. The telemetry and command data rates are both 1 kbps. The following tables summarize the link budget between the THEMIS Probes and TDRSS. THEMIS does not have any TDRSS tracking requirements.

Since the Probes are in stable orbits, there is ample margin to schedule the near-perigee maneuvers depending upon availability of the TDRSS service.

TDRSS H / I / J services are anticipated to be available during the 2006-2008 time frames. The THEMIS team is in contact with the CSOC representative to formulate a draft Project Level Service Agreement.