QUESTION 9: Describe the full boom deployment sequence, including conditions required for successful deployment. What is the impact of a failure to successfully deploy one or more booms?  How will the boom deployment scenario be tested prior to launch?

DETAILED THEMIS APPENDAGE DEPLOYMENT SEQUENCE

1.

Spin to 15 RPM to release MAG Booms (15 to 13 RPM despin during boom deployment)

2. Spin to 30 RPM to prepare for Wire Boom Deployment, then release door caging

– Deploy AB pair to 5m radius, then CD pair to 5m (30 to 11 RPM momentum despin)

– Deploy AB pair to 10m, then CD pair to 10m (11 to 4 RPM despin)

– Spin to 26 RPM to unfurl all Sphere Reels

– Deploy AB pair to 12m, then CD pair to 12m (26 to 18 RPM despin)

– Spin to 24 RPM, deploy AB pair to 14m, then CD pair to 14m (24 to 17 RPM despin)

– Spin to 22 RPM, deploy AB pair to 16m, then CD pair to 16m (22 to 17 RPM despin)

– Spin to 20 RPM, deploy AB pair to 18m, then CD pair to 18m (20 to 16 RPM despin)

– Spin to 20 RPM, deploy AB pair to 20m, then CD pair to 20m (20 to 16 RPM despin)

3. Release Axial booms (pair release baselined, can be individual with on adverse effects)

4. Trim final (Operational) spin rate to 20 RPM, if needed

NOTES:

• All sequences may be interrupted by ‘Safe Hold’ without risk to a probe

• IDPU controlled deploy logic stops longer boom if difference is > 0.1m

• Angular momentum is conserved in deployments, so sequence does NOT affect fuel consumed

• Deployment can be verified by spin changes, spin flutter & nutation frequencies

MASS PROPERTIES USED IN BOUNDING ANALYSES

CORE PROBE (all properties were derived from detailed 3D solid CAD models)

5kgm2 < Ixx < 15kgm2  ;  5kgm2 < Iyy < 15kgm2  ;  9kgm2 < Izz < 23kgm2

70 kg < Mass < 137kg

MAG BOOMS

FGM = 0.60 kg max. ; SCM = 1.30 kg max. ; Boom & cabling = 0.084 kg/m

RADIAL WIRE BOOMS

Sphere = 0.100 kg; Preamp = 0.080 kg; cable = 0.008 kg/m; 1.5m fine wire = 0.001 kg/m

STIFF AXIAL BOOMS

Main boom & cable = 0.149 kg/m; Preamp = 0.050 kg; 1m whip sensor = 0.008 kg/m

NOTES: Mass, mass moments, CG, center of percussion, and flexural stiffness (EI) values are computed from first principal models. Appendage flexibility modifiers (modeled after Meirovitch & Calico, 1972) are included in mass property calculations.

BACKGROUND

The boom deployment sequence is derived from a long list of missions that include: S3-2, S3-3, VIKING, FREJA, CRRES, ISEE-B, POLAR, FAST, & CLUSTER II. Each of the deployment sequences are defined in a linked, interactive spreadsheet useable in real time during commissioning. Past spreadsheets have been generated in the popular software of their day, including Visicalc, Multiplan, Quattro, and now Excel. They are interactively used to verify (and update) events in real time during the probe commissioning phase by providing both the despin predicts and the dynamic spin flutter and nutation modes.

DEPLOYMENT CONTROLS

MAG BOOMS – Free Release, no interaction needed. May be verified by both ACS transients and science data

RADIAL WIRE BOOMS - The IDPU controls motor relays with “clicks” feedback from each boom unit. Ground commands are of the form of “deploy AB pair N clicks”. A cam and micro-switch on the motor driven cable drive meter wheel provides IDPU feedback in the form of two on/off clicks per revolution. The IDPU powers the motors and then counts clicks, as well as monitoring the A-B differential. The faster boom is stopped if the differential is greater than two, to assure symmetric boom deployment. Both boom motors are turned off when the N clicks are achieved. This click data is also available in housekeeping telemetry. A tension based “End of Wire” micro-switch stops the boom motor in the event of a cable jam or end of wire. This simple algorithm provides reliable deployments, even with over the horizon loss of contact. Each of the deployments may be verified by both ACS transients and science data.

STIFF AXIAL BOOMS – Free Release, no interaction needed. May be verified by both ACS transients and science data

PROBE PASSIVE SPIN STABILITY & FAILURE MODES

The stability (defined by the Izz / Irr ratio) increases for all deployment events, except the final axial boom release, because the roll moment is increasing faster than the two principal transverse moments. The symmetric wire boom deployment algorithm prevents excessive CG shifts.

The decision to deploy axial booms is reserved for the Operations Team after a full dynamic evaluation of each probe, in the event of a partial or full boom deployment failure. In the mission design, boom properties were chosen to accommodate total failure of one wire boom, while achieving the GSFC historical guideline for the minimum stable probe moment ratio of 1.04. This moment ratio is very conservative, at 1.35 for the normal mission profile. A failed appendage deployment does not preclude safe deployment of the radial appendages and will not compromise the dynamic stability.

DEPLOYMENT TESTING

The following tests have proven to be effective on many prior flight appendages nearly identical to those chosen for THEMIS.

MAG BOOMS are tested by a combination of “ceiling drop tests” and “spin table tests”, where the major part of this activity is correlating the measured test results to the math models to confirm on orbit predicts and margins. Adequate thermal and/or thermal vacuum tests have been defined on a “case by case” basis as the design and modeling matures. Given their early mission deployments, extreme cold tests should not be needed, meaning that (non-condensing) thermal tests would be adequate.

WIRE BOOM mechanisms will be tested on the bench, as well as in Thermal Vacuum using a compact, programmable tension “Take-up Mechanism” that rewinds the cable onto a larger spool. Deployment tests are controlled by GSE that mimics IDPU logic.

The stiff (~1 Hz) AXIAL BOOMS are planned to be tested in up/down orientations to minimize the gravity effects (as was the case for FAST booms). More elaborate offloading techniques have been used for much longer booms in programs such as POLAR, but are nor anticipated for THEMIS. Thermal vacuum deployments are accomplished on a horizontal test track that has several mid-span rollers to reduce gravity induced bending to an acceptable level.

Post PROBE (Mission I&T Level) integration tests have historically been limited to the form of “First Motion” tests (MAG), or qualified “Break-Out” boxes that verify needed power and logic signals (SPB & AXB) plus physical clearance verification. In the context of our proven “first motion” test scheme, past NASA Project Offices have concurred with those approaches and typically deem the value of “full up tests” to be marginal, given the need to disassemble for boom re-stowing, as well as based on the confidence from the high fidelity testing prior to integration. This is based on the damage risk potential in these ancillary operations.