QUESTION 8A: Why do you have confidence that the reliability approach will meet the mission life requirement?

- The five Probe constellation mission architecture and the ability of either Probe 3 or Probe 4 to replace any other Probe, should one experience a complete block failure, allows for mission resilience and provides a variety of avenues to accomplish the science objectives.  This constellation architecture is analogous to an on-orbit satellite in storage and thus eliminates single point failures within the mission architecture.

- The overall Probe architecture allows for graceful performance degradation if individual elements fail (for example dual axial and tangential  thrusters, gyro’s and magnetometer (backups to the sun sensor), N+1 redundancy in memory blocks, solar array string or battery cell failure only degrade capacity).  In addition, an individual instrument failure does not preclude operation of the rest of that probe’s instrument complement and represents a minor loss of mission science data collection capability.

- We used reliability analyses, coupled with our Failure Modes & Effects Analysis (FMEA), as tools to characterize and evaluate our system design to gain insight into key operational functions.  Our confidence in mission success results from of our time-tested design practices, the selection of heritage components, in-process statistical production feedback (as result of multiple parallel builds), qualified manufacturing processes, and a rigorous and comprehensive component, subsystem, and system level test program.

QUESTION 8B: Present your assumptions and initial conditions for the analysis.

- Individual Probe string model (From CSR Figure F-57) and the associated element reliabilities are:

- Analysis considered main mission mode (assuming completed successful launch mode)

- Use of extensive TRW (concatenation of industry historical data) and prior Swales missions (EO-1) component databases convolved into board/element level configurations for each major function to derive estimated Mean Time Between Failures (MTBF’s)

- MTBF’s used to derive associated individual element reliabilities for both two year (design life) and one year (minimum life) timeframes

- Constellation reliability uses the “k of n” probability approach via a binomial series expansion to calculate the mission reliability

QUESTION 8C: What is the single-probe reliability, and how does it correlate to the overall mission reliability?

- Prior String Model was used to produce the Single Probe reliabilities in Table F-37 for both the 2 year (design) and 1 year (minimum) mission timeframes

- Resulting Probe and Constellation Reliabilities (CSR Table F-37)