Question 6: GSFC Spec 311-INST-1A [CSR, Page 100] applies only to instruments, and calls for Grade 3 parts only for non-essential instruments with lifetimes less than one year.  Please explain your decision to use Grade 3 parts (with some up-screens to Grade 2) for key parts in critical subsystems for the entire spacecraft.

Answer:

The top-level document that the THEMIS EEE parts program is responsive to the GSFC-410-MIDEX-003 Rev A dated June 25, 2002 (heron called GSFC-410). Consistent with section 5.2 of GSFC-410 which states that “In general all parts should be selected and processed in accordance with GSFC 311-INST-001 for Grade 3 quality level” the THEMIS EEE parts program has established as a minimum Grade 3 quality level for both instruments and probe EEE parts. Therefore the THEMIS EEE parts program, which a Draft document has been generated for Phase A, has implement GSFC-311 Grade 3 quality level as the minimum standard for all EEE parts.

However the satellite probes critical components such as the BAU Power Board, BAU Communication Board, Miniature Spinning Sun Sensor (MSSS) and Transponder have established a Grade 2 or better parts program. In fact, for inexpensive/short lead parts such as diodes, resistors, capacitors, small transistors etc. we have specified Grade 1 parts (JANTXV etc.).  For the BAU Computation board, which is provided by UCB, the basis of which is the STEREO program (Grade 2 or better program) we have instituted a Grade 2 or better parts program.  In summary the Probe Bus has instituted a Grade 2 or better parts program.

As in past IDPU efforts, Engineers and QA identify critical circuitry and purchase higher grade parts accordingly (e.g. CPU & LVPS). In general, the best available parts above the requirement are purchased.