QUESTION 4A: Six previous missions are cited as FSW heritage.  Identify the software baseline against which the reuse percentages in Table F-12 were calculated.

Answer: The CSR page 56 Table F-12, that contained the reuse percentages by module, has been upgraded to more clearly identify the software baseline used as shown below.

| Table F-12 Rev A Flight Software Heritage and CPU Utilization |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsystem | Function |  | % Heritage Reuse |  | Mission Heritage |  | % CPU Utilization |  |
| Operating System & S/W Bus |  | Operating environment for defined tasks; Inter-task communication. Library functions |  | 20 |  | FAST, WIRE |  | 10 |
| Stored Command Processor |  | Execute time based commands for instrument; activate downlink & playback of stored science & H/K data |  | 40 |  | FAST, WIRE |  | 2 |
| Software Manager |  | Manage memory loads & dumps to accomplish flight s/w patch to NORMAL MODE CODE |  | 20 |  | FAST, WIRE |  | 4 |
| Command Ingest |  | Receive, validate, and distribute CCSDS command packets |  | 40 |  | FAST, WIRE |  | 2 |
| Time Manager |  | Maintain & adjust time to subsecond resolution (32-bit sec., 16 bit subsec.) |  | 40 |  | FAST, WIRE |  | 2 |
| Telemetry & Status Monitor |  | On-board health & safety monitoring |  | 40 |  | FAST, WIRE |  | 2 |
| Housekeeping Telemetry Collection |  | Collects & formats H/K data into a CCSDS packet for storage and downlink |  | 40 |  | FAST, WIRE |  | 2 |
| Data Storage |  | Format engineering & H/K data packets, with status, into CCSDS transfer frames for maintenance & playback |  | 50 |  | FAST, WIRE, EO-1 |  | 3 |
| Telemetry Output (Downlink) |  | Control formation of real-time transfer frames containing system packet telemetry and the downlink of both real-time & playback frames interleaved |  | 50 |  | FAST, WIRE |  | 6 |
| Instrument Manager |  | Supports interfaces with the instrument for commanding and receipt of housekeeping data |  | New |  |  |  | 3 |
| Memory Scrub + Checksum |  | Scrubs memory and diagnostic checksums (background task) |  | 60 |  | FAST, WIRE |  | 1 |
| ACS Tasks |  | Ingest ACS ground CMD’s, initiate I/O buffers & data arrays, collect & process sensor/actuator data, ACS math functions, process thruster CMD sequences |  | 30 |  | FAST, EO-1 |  | 8 |
| Failure Detection & Correction |  | Provide ACS failure, detection and correction using health & safety data |  | New |  | (EO-1) |  | 2 |
| Subtotal Average CPU % Utilization |  |  |  |  |  |  |  | 47 |

As described in our technical approach for flight software development, the THEMIS flight software is based on heritage software design from the Small Explorer (SMEX) missions from Goddard Space Flight Center. Starting with the SAMPEX mission, the flight software for successive missions (FAST, SWAS, TRACE and WIRE) was utilized as the basis for continued mission support. The command and data handling functions were enhanced and improved for each successive mission as the same basic C&DH functions were required for each mission. The same is true for THEMIS. For example, command ingest and telemetry processing functions for CCSDS protocol, on-board data storage and playback, and software-bus data handling and task scheduling are essentially the same software components, modified, re-integrated and tested to meet specific mission requirements. For attitude control support, the math functions used for the EO-1 flight software were originally developed for the TRMM and RXTE missions. Due to the similarity in both the C&DH and simple ACS functions of the FAST spacecraft, we intend on utilizing the FAST flight software as well as the most recent SMEX mission (WIRE) to baseline the THEMIS flight software system. The WIRE flight software contains the latest improvements of C&DH functionality as well as having the advantage of continued “hours” of execution. The FAST resource requirements are similar to those planned for the THEMIS system. Utilizing the math components and the design architecture for failure detection and correction from the EO-1 mission also supports our approach of using existing/tested flight software components to develop the THEMIS flight software.

Risk reduction of utilizing heritage flight software does not stop there. We gain an added advantage from using existing software components in the integration and test approach. Because the entire SMEX mission flight software used the ITOS ground system, also planned for THEMIS, we currently have test procedures that can also be used as our baseline for re-integration and testing the THEMIS flight software. The same approach applies here; we can use existing STOL procedures and modify them to test the modified flight software. This will allow more time for regression testing of the heritage software and therefore allow more time to test new requirements for the THEMIS mission.

Further flight software risk mitigation will be realized from a recent development opportunity at NASA GSFC for the Living with a Star Space Environmental Testbed (LWS-SET) program. This mission will use the 80196 processor, that is also used for the THEMIS mission. Over the past two weeks, the Hammers Company has conducted several technical interface meetings with the technical support staff for the CMX real-time operating system and the IAR compiler and debugger development tools (COTS products) for the 80196 processor. We are also conducting demonstrations to the LWS-SET avionics team of sample code provided by the COTS vendor. We are days away from starting development of C&DH flight software for the LWS-SET mission that will provide instrument support for command and telemetry processing. This complimentary development effort will certainly have “greased the skids” before we start development of THEMIS flight software.

QUESTION 4B. Provide the basis for the IDPU software estimate of 3000 lines of code.

IDPU FSW Lines of Code Estimate. As stated in the CSR, the lines of code needed for THEMIS IDPU was estimated to be between 3000 and 4000 lines. For costing purposes, we gave the larger number (4000) to IV&V. This estimate was based upon using an 80196 processor in assembly language. Our best estimate to date is itemized below as 3947.

Processor Differences. Differences between the 80196 and 8085 upon which much of the heritage comes, were taken into account using a factor of .540 . This factor is determined using a comparison of opcode lengths of the two processors for specific types of operations, multiplied by standard proportions of the usage of those types of opcodes. For example, the 80196 has some opcodes which have 3 operands (2 source and 1 destination), while the 8085 has only 1. The expected population of this 3 to 1 advantage is only 15% however. Calls and jumps will remain one line of code, no matter which processor is used. The basic result of this is that an 80196 program should be 1/1.85 or 54% of the equivalent 8085 routine in terms of lines of code (not bytes)

| Ratio of 8085 to 80196 | Population | Advantage | Ratio |
| --- | --- | --- | --- |
| Jumps/Calls | 30% | 1 | 0.3 |
| Add/Mul 2 | 30% | 2 | 0.6 |
| Add/Mul 3 | 15% | 3 | 0.45 |
| Other | 25% | 2 | 0.5 |
|  |  |  | 1.85 |

Heritage Modules. A basic understanding of the functions required of the CPU card led to the selection of a number of modules of similar functionality from prior projects. For example, THEMIS housekeeping module will function much like the HESSI module, reading a number of analog values and building CCSDS packets for telemetry. Taking the lines of code from HESSI (without the comment lines) and applying the 80196/8085 factor, we arrive at an estimate for the THEMIS housekeeping module.

For modules in THEMIS for which we have no heritage, we took a generic module from HESSI of the same kind of function. For example, the FGM interface controller will require a module of about the same complexity as the HESSI Particle Detector; the SST controller will require high voltage ramps and such, which is similar to the HESSI PMT.

One module is actually from the STEREO project 80196 and so the estimate listed is the actual lines of code in the module and has no conversion applied.

There are some known over estimates in the table. For instance, the EFI Sweep module analysis is contained in the sweep module. THEMIS will use only the sweep and leave the analysis off. We did not think it necessary to investigate how much of the module could be cut out at this point.