FAST-SPEC-012

Revision A

Small Explorer-Fast Auroral Snapshot

“SMEX-FAST”

Magnetic Contamination and EMI/EMC

Control and Implementation Plan

17 September 1993

REVISION HISTORY

DATE		DESCRIPTION		INITIALS

ORIGINAL RELEASE	10/14/92

REVISION A		09/17/93		INC. CCR# FAST-160

# APPROVAL SIGNATURES

David Everett: ________________________________________	Date: _______________

(FAST Electrical Systems Engineer: Code 743.1)

James Watzin: ________________________________________	Date: _______________

(Small Explorers systems Engineer: Code 740.2)

Chuck Carlson: _______________________________________	Date: _______________

(FAST Mission Principal Investigator, University of California, Berkeley)

## CONCURRENCE SIGNATURES

Timothy Gruner: ______________________________________ 	Date: _______________

(FAST MUE Lead Engineer Code: 745.2)

Haydee Maldonado: ____________________________________	Date: _______________

(Transponder Lead Engineer: Code 727.3)

Ken Hersey: __________________________________________	Date: _______________

(Antenna System Lead Engineer: Code 737.1)

Tom Spitzer: __________________________________________	Date: _______________

(Power Lead Engineer: Code 734.1)

Mary Walker: _________________________________________	Date: _______________

(ACS Lead Engineer: Code 745.1)

John Lyons: ___________________________________________	Date: _______________

(Solar Array Engineer: Code 734.4)

Teresa LaFourcade: _____________________________________	Date: _______________

(Harness Lead Engineer: Code 743.3)

Giulio Rosanova: _______________________________________	Date: _______________

(FAST Mechanical Lead Engineer: Code 741.3)

# TABLE OF CONTENTS

1.0 	INTRODUCTION							1

2.0	APPLICABLE DOCUMENTS						1

2.1	Referenced documents							1

2.2	Background documents							2

3.0	ORGANIZATIONAL RESPONSIBILITIES					3

3.1	Electrical systems engineer responsibilities					3

3.2	Design engineer responsibilities						3

4.0	TECHNICAL DISCUSSION						3

4.1	EMI Philosophy								3

4.2	Operational Considerations						4

4.3	Electromagnetic compatibility						4

4.4 	Potential emitter and receptor characteristics					4

4.5	Electrostatic cleanliness							4

4.6	Magnetic cleanliness							4

5.0	EMI REQUIREMENTS							4

5.1	Basis for EMI requirements						4

5.2.1	Conducted emissions							5

5.2.1.1	DC power leads (low frequency)						5

5.2.1.2	DC power leads (high frequency)						5

5.2.1.3	Antenna terminal								5

5.2.2	Conducted susceptibility							5

5.2.2.1	DC power leads (low frequency)						5

5.2.2.2	DC power leads (high frequency)						6

5.2.2.3	Intermodulation								6

5.2.2.4	Spurious rejection							6

5.2.2.5	Cross modulation								6

5.2.2.6	DC power spikes								7

5.2.3	Radiated emissions							7

5.2.3.1	Electric fields								7

5.2.3.2	Magnetic fields								7

5.2.4	Radiated susceptibility-electric fields					7

5.2.5	DC magnetics requirements						8

5.3	Pegasus and carrier aircraft RF environment					8

6.0	DESIGN GUIDELINES							8

6.1	Grounding and power/signal distribution					8

6.1.1	Primary power wiring							9

6.1.2	Electrical bonding							9

6.1.3	Bundle shields and connectors						9

6.1.4	Signal distribution							9

6.1.4.1	Signals between spacecraft enclosures					9

6.1.4.2	Ground support equipment (GSE) interface signals				9

6.1.4.2.1	Twisted-shield pair wiring						             10

6.1.4.2.2	Shield terminations						             10

6.1.4.2.3 Differential signal transmission					             10

6.1.3.4	Pegasus launch vehicle interface signals				             10

6.1.4	PC board and motherboard requirements				             10

6.2	Wire harness requirements						             11

6.2.1	Power and signal classes						             11

6.2.1.1	Class 1 wires							             11

6.2.1.2	Class 2 wires							             11

6.2.1.3	Class 3 wires							             12

6.2.1.4	Class 4 wires							             12

6.2.1.5	Class 5 wires							             12

6.2.2	Wire harness mechanical requirements				             12

6.3	DC-DC converter requirements					             12

6.4	Switched inductive load spike suppression				             12

6.5 	Open collectors							             12

6.6	Signal rise and fall times						             13

6.7	Resistivity of spacecraft surface materials				             13

6.8	Magnetic materials						             13

6.9	Solar array wiring						             13

6.10	Filtered-pin connectors						             13

7.0	QUALITY ASSURANCE						             13

7.1	Test philosophy							             13

7.2	Test plans							             14

7.3	Test procedure							             14

7.4	Test fixture							             15

8.0	NOTES								             15

8.1	Acronyms							             15

8.2	EMI checklists							             16

8.2.1	Checklist A: circuits to be shielded and filtered			             16

8.2.2	Checklist B: elimination of spurious responses			             16

8.2.3	Checklist C: selection of components				             17

8.2.4	Checklist D: electro-explosive devices				             17

## LIST OF FIGURES

Figure 4.4-1	Frequency map							F-1

Figure 5.2.1.1-1	CEO1 limits							F-3

Figure 5.2.1.2-1	CEO3 limits (narrowband						F-4

Figure 5.2.1.2-2	CEO3 limits (broadband)						F-5

Figure 5.2.2.1-1	CSO1 limits							F-6

Figure 5.2.2.4-1	CSO4 limits							F-7

Figure 5.2.2.6-1 	CSO6 limits							F-8

Figure 5.2.3.1-1	REO2 limits (lower frequency narrowband)				F-9

Figure 5.2.3.1-2	REO2 limits (higher frequency narrowband)			             F-10

Figure 5.2.3.1-3	REO2 limits (lower frequency broadband)			             F-11

Figure 5.2.3.1-4	REO2 limits (higher frequency broadband)			             F-12

Figure 5.2.3.2-1	REO4 limits						             F-13

## LIST OF TABLES

Table 4.4-1	FAST frequencies						T-1

Table 5.2-1	EMI requirements/verifications per level of assembly			T-4

Table 5.2.5-1	Magnetization requirements					T-5

Table 5.3-1	Pegasus RF environment						T-6

Table 5.3-2	Carrier aircraft RF environment					T-7

- INTRODUCTION

This document establishes the Electromagnetic Compatibility (EMC) , Electromagnetic Interference (EMI), electrostatic cleanliness, and magnetic cleanliness details for the Small Explorer-Fast Auroral Snapshot (SMEX-FAST) mission being developed by the Goddard Space Flight Center (GSFU) and the University of California, Berkeley (UCB). These details consist of the project management plan, performance requirements, design guidelines and quality assurance requirements needed to assure that the SMEX_FAST spacecraft will be electromagnetically, electrostatically and magnetically compatible with the environment, the Pegasus launch vehicle, and itself throughout all phases of the mission.

- APPLICABLE DOCUMENTS

2.1          Referenced documents

The documents listed here are applicable to the extent as specified herein. In cases of a conflict in requirements, this document shall have precedence over the referenced documents.

GEVS-SE		General Environmental Verification Specification for STS and ELV Payloads,

January 1990		Subsystems and Components

MIL-STD-461C		Electromagnetic Emission and Susceptibility Requirements for the Control of

4 August 1986		Electromagnetic Interference

MIL-STD-462		Electromagnetic Interference Characteristics, Measurement of

Notice 6 (USAF)

15 October 1987

MIL-B-5087B		Bonding, Electrical and Lightning Protection for Aerospace Systems

Interim Amendment 3

(USAF)

24 December 1984

FAST-SPEC-005		Requirements Document for SMEX Fast Auroral Snapshot (FAST) Mission

28 April 1992

FAST-SPEC-013	(Prelim)	Mission Unique Electronics (MUE) Requirements Document

19 December 1991

OSC A7055		Pegasus-TOMS Interface Control Document

20 February 1992

  - Background documents

In addition to the above referenced documents, the documents listed below were used in the development of this document. This list is provided solely for the benefit of the reader who desires a deeper understanding of the driving factors behind this document.

Background Specifications and Standards:

MIL-STD-463A	Definitions and Systems of Units, Electromagnetic Interference and Electro-magnetic Compatibility Technology

MIL-STD-1541A (USAF)	Electromagnetic Compatibility Requirements for Space Systems

MIL-E-6051D	Electromagnetic Compatibility Requirements, System

Notice 1

26 February 1988

FAST-SPEC-005	Requirements Document for SMEX Fast Auroral Snapshot Explorer (FAST)

Mission

FAST-SPEC-013 (Prelim)	Mission Unique Electronics (MUE) Requirements Document

19 December 1991

Other sources:

“Global Geospace Science (GGS) Mission EMC/EMI Design and Implementation Requirements,” CDRL315, Contract No. NAS5-30503, 29 April 1991.

“Global Geospace Science (GGS) Mission ESC Design and Implementation Requirements,” CDRL 319, Contract No. NAS5-30503, 22 May 1991.

“Optimum Electrical Systems Design With Regard to EMI and EMC as Applied to ISTP,” Robert G. Martin (Swales) memorandum, 7 August 1990.

“Spacecraft EMI Design Recommendations,” Robert G. Martin (Swales) memorandum, 7 August 1990.

“Noise Reduction Techniques in Electronics Systems,” Henry W. Ott, John Wiley & Sons, 1976, ISBN 0-471-02992-0.

“Grounding and Shielding Techniques in Instrumentation,” Ralph Morrison, John Wiles & Sons, 1977, ISBN 0-471-02992-0.

“Electromagnetic Interference and Compatibility, Volume 1: Electrical Noise and EMI Specifications,” Donald R.J. White, Don white Consultants, 1971.

“Electromagnetic Compatibility Handbook,” J.L. Norman Violette and Michael Violette, Van Nostrand Reinhold, 1987.

- ORGANIZATIONAL RESPONSIBILITIES

3.1          Electrical Systems engineer responsibilities

The electrical systems engineer shall be responsible for developing the EMI Control and Implementation Plan, providing technical advice to the design engineers, reviewing the designs for compliance to EMI requirements, developing the EMI test plans and procedures, monitoring the EMI tests and writing the EMI test reports.

  - Design engineer responsibilities

The various design engineers shall develop designs that are in compliance with the requirements stated herein, provide pertinent design data when required for EMI design reviews, assist in developing EMI test plans and reports, and correct any problems that are uncovered during EMI testing of their designs.

- TECHNICAL DISCUSSION

Details describing the mission requirements and spacecraft configuration may be found in FAST-SPEC-005.

  - EMI Philosophy

The goal of this EMI control and implementation plan is to assure that the SMEX-FAST spacecraft is compatible with its environment, the Pegasus launch vehicle, and itself consistent with constraints on spacecraft weight, power and cost.  First and foremost, since this mission measures low-level magnetic and electric fields and small charged particles, any option considered must provide for adequate science and spacecraft performance. Due to programmatic constraints, this plan will accept the transponder and Pegasus interfaces and EMI performance as given and will make recommendations on the rest of the spacecraft to correct any shortcomings.

  - Operational considerations

From the EMI point of view, there are two major operational factors that must be considered.  The first is that the spacecraft must be able to transmit real-time telemetry while taking accurate science data simultaneously.  The second is that the ACS will not drive the spin coil and precession coil while science data is being taken.

  - Electromagnetic compatibility

The spacecraft contains many different types of scientific instruments and electronic circuits.  Inadvertent radiation to or from these circuits (either inter-system or intra-system) can corrupt science and/or housekeeping data, cause improper operation of the spacecraft and/or launch vehicle.  The plan will make recommendations to develop an electromagnetically  compatible spacecraft that avoids these problems.

  - Potential emitter and receptor characteristics

A frequency mapping of potential emitters and receptors is shown in Figure 4.4-1. The exact frequencies are shown in Table 4.4-1.

  - Electrostatic cleanliness

Any spacecraft orbiting through space plasma is susceptible to differential charging if improperly designed.  Any significant differential charge on the spacecraft can distort the science data and even cause damaging arcing if large enough.  This plan will make recommendations to develop an ‘electrostatically-clean’ spacecraft that avoids the differential charging problem.

4.6	Magnetic cleanliness

Any significant permanent or current-induced magnetic dipole moments generated by the spacecraft bus can distort the science data.  This plan will make recommendations to develop a “magnetically-clean” spacecraft that avoids this problem.

- EMI REQUIREMENTS

5.1	Basis for EMI requirements

GEV-S-SE and MIL-STD-461C (for Class A2a equipment) are used as the sources of EMI requirements with modifications provided by UCB and OSC.

5.2	Specific EMI requirements

The various EMI requirements and verification procedures shall be applied to the spacecraft and its boxes as shown in Table 5.2-1.  Details of the requirements and verifications are contained in the following subparagraphs.  For EMI purposes, the spacecraft has only two levels of assembly: individual boxes and the assembled spacecraft itself.  A box is defined to be an enclosure with self-contained electronics and wiring where the enclosure is mounted to the spacecraft structure.

    - Conducted emissions

      - DC power leads (low frequency)

For all spacecraft power and power-return leads (including switched power and Pegasus interface power), the narrowband emissions shall be limited to the levels shown in Figure 5.2.1.1-1.  Testing shall be in accordance with MIL-STD-461 and MIL-STD-462, test method CE01, with limits shown in Figure 5.2.1.1-1.

      - DC power leads (high frequency)

For all spacecraft power and power-return leads (including switched power and Pegasus interface power), the narrowband emissions shall be limited to the levels shown in Figure 5.2.1.2-1, and the broadband emissions shall be limited to the levels shown in Figure 5.2.1.2-2.  Testing shall be in accordance with MIL-STD-461 and MIL-STD-462, test method CE03, with limits shown in Figure 5.2.1.2-1 and Figure 5.2.1.2-2.

      - Antenna terminal

Conducted emissions on the transponder RF port shall not exceed 34 dBμV for narrowband emissions and 40 dBμV/MHz for broadband emissions with the receiver on and the transmitter off.  Harmonics and all other spurious emissions from the RF port while the transmitter is on shall not exceed –55 dBW.  Testing shall be in accordance with MIL-STD-462, test method CE06.  CE06 testing shall be performed before the transponder is integrated with its antenna system.

    - Conducted susceptibility

      - DC power leads (low frequency)

The test sample shall not exhibit any malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when electromagnetic radiation whose magnitude is no greater than the limits shown in Figure 5.2.2.1-1 is injected into the spacecraft power leads (including switched power and Pegasus interface power).  The applicable frequency interval shall be 30 Ha to 50 Hz.  Testing shall be in accordance withMIL-STD-462, test method CS01.

      - DC power leads (high frequency)

The test sample shall not exhibit any malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when electromagnetic radiation whose magnitude is no greater than 1 Volt from a 50 Ω source is injected into the spacecraft power leads (including switched power and Pegasus interface power).  The applicable frequency interval shall be 50 KHz to 400 MHz.  Testing shall be in accordance with MIL-STD-462, test method CS02.

      - Intermodulation

- Signal generator #1 is set to 66 dB above the level required to obtain the standard reference

output (as specified in MIL-STD-462) but shall not exceed 10 dBmμ.

b.        Signal generator #2 is set to 66 dB above the level required to obtain the standard reference

output (as specified in MIL-STD-462), but the generator output level shall not exceed 10

dBμ.

Testing shall be in accordance with MIL-STD-462, test method CS03 and shall be applied to the transponder.

      - Spurious rejection

The transponder receiver shall not exhibit any undesired response when subjected to spurious signal whose limits are shown in Figure 5.2.2.4-1.  Testing shall be in accordance with MIL-STD-462, test method CS04, and shall be applied at the box level.

      - Cross modulation

The transponder receiver shall not exhibit, due to cross modulation, any malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when subjected to the following from signal generator #2: a signal 66dB above the level required to obtain the standard reference output (as specified in MIL-STD-462), but not to exceed a power output level of 10dBμ. Testing shall be in accordance with MIL-STD-462, test method CS05, and shall be applied to the transponder at the box level.

      - DC power spikes

The test sample shall not exhibit any malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when spikes having the waveform shown in Figure 5.2.2.6-1 into the spacecraft power leads (including switched power and Pegasus interface power).  Testing shall be in accordance with MIL-STD-462, test method CS06.

    - Radiated emissions

      - Electric fields

Unintentional electric fields radiated by the spacecraft and its individual boxes shall not exceed the limits shown in Figures 5.2.3.1-1 and 5.2.3.1-2 for narrowband emissions and Figures 5.2.3.1-3 and 5.2.3.1-4 for broadband emissions. Testing shall be in accordance with MIL-STD-462, test method RE02 with the revised test frequency range and limits shown in Figures 5.2.3.1-1, 5.2.3.1-2, 5.2.3.1-3 and 5.2.3.1-4.

      - Magnetic fields

The magnetic field radiated emissions test will measure power in the specified bandwidth.  Unintentional magnetic fields radiated by the spacecraft and its individual boxes shall not exceed the limits shown in Figure 5.2.3.2-1 over the frequency interval 20 Hz  to 1 MHz. The test shall be performed at 1 meter. Should additional sensitivity be required, the test may be performed at 30cm, with 15dB added to the specified levels. The search-coil magnetometer sensitivity shown in Figure 5.2.3.2-2 demands a tight specification for radiated magnetic fields.  This test replaces RE04 and will probably require a search-coil magnetometer.

    - Radiated susceptibility-electric fields

The test sample shall not exhibit permanent malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when subjected to the radiated electric fields specified herein.  Units that are mounted external to the spacecraft structure (i.e. outside of the volume defined by the solar array substrate, the instrument deck, and the radiation shield) shall use the following levels:

Frequency range		Launch mode		Normal mode

14KHz to GHz		10V/m			10 V/m

2 GHz to 18 GHz		10 V/m			10 V/m

5 GHz to 6 GHz		100 V/m*		N/A

*2600 Hz pulse repetition frequency with a 10% duty cycle.

Units that are located entirely within the spacecraft structure shall use levels that are a tenth of (20db below) the above limits.  Intentional electric field sensors that operate within the frequency range of the test shall be removed or disabled without otherwise disabling the spacecraft during test. Testing shall be in accordance with MIL-STD-462, test method RS03.

    - DC magnetics requirements

The maximum magnetic fields generated by individual boxes shall be as shown in Table 5.2.5-1.  The maximum DC magnetic field generated by the spacecraft shall be no greater than 1 nT at the fluxgate magnetometer. These requirements will be tested by making measurements of the magnetic fields generated by the spacecraft while operating in the magnetic test facility located at GSFC.

  - Pegasus and carrier aircraft RF environment

During the captive carry and launch phases of the mission, the spacecraft shall not exhibit permanent malfunction, degradation of performance, or deviation from specified indications (beyond the normal tolerances) when subjected to the RF environment shown in the Table 5.3-1 for the Pegasus RF environment and Table 5.3-2 for the carrier aircraft RF environment (requirements taken from OSC A70055).  The requirements of the paragraph are verified by performance of the tests specified in 5.2.4.

- DESIGN GUIDELINES

6.1	Grounding and power/signal distribution

The spacecraft grounding and power distribution system shall be as described in paragraph 3.8.1 of FAST-SPEC-005.

    - Primary power wiring

Primary power is defined to be the spacecraft power bus generated by the power subsystem and consists functionally of two wires: +28 Vdc and +28 Vdc Return. It includes connections to the power subsystems (including relayed power) and the primary side of all DC-DC converters used in the science instrumentation and other spacecraft bus subsystems.  Primary power shall be carried on twisted pair wiring to reduce normal mode magnetic dipole moments. Power is typically carried on multiple pairs of wires for redundancy; all of the power wires should be twisted together.  As a guide, roughly 20 turns per foot will be effective for wire twisting.  At the DC-DC converters, both the +28 Vdc return wires together and to spacecraft chassis at a common point within the MUE that is as close to the battery as possible.  The common return point and DC-isolation at the converters will prevent converter noise currents from contaminating the spacecraft chassis.

    - Electrical bonding

Electrical bonding of boxes and spacecraft chassis members to each other is critical to maintaining low impedance to return current flow.  Bonding between the conductive parts of spacecraft chassis, box enclosures, connectors mounted on the enclosures/spacecraft chassis, and cable shields shall be in accordance with MIL-B-5087, Classes R, C, S, and H.

6.1.3	Bundle shields and connectors

If bundle shields are used, they shall be terminated to the shells of all connectors through as low an impedance path as possible.  Mated connectors shall have a dc resistance of no greater than 2.5 milliohms from the shell of one connector to the shell of the other.  The bundle shields will reduce electrostatic coupling between wires internal and external to the shields.

6.1.4 	Signal distribution

6.1.4.1	Signals between spacecraft enclosures

Requirements on signal distribution between the spacecraft enclosures are described in FAST-SPEC-013.

      - Ground support equipment (GSE) interface signals

6.1.4..2.1  Twisted-shieled pair wiring

GSE interface signals and power shall be carried on twisted-shielded pair wire.  The wire twisting will reduce normal mode magnetic dipole moments.  As a guide, roughly 20 turns per foot will be effective for wire twisting.

        - Shield terminations

Shields for GSE interface signal pairs whose frequency is less than 1 MHz shall be connected to chassis at the transmitter only and shall be continued through any intervening connectors.  Shields for GSE interface signal pairs whose frequency is greater than 1 MHz shall be connected to chassis at both the transmitter and the receiver and shall bi continued through any intervening connectors. Daisy-chaining of shield terminations at connectors shall be minimized to the maximum extent consistent with weight and cost constraints.  If daisy-chaining of grounds at intermediate connectors must be used, no more than 5 shields may be daisy-chained at a connector.  Daisy-chaining shall be consistent if used at more than one location in the signal path.  The shields will reduce electrostatic coupling between wires internal and external to the shields.  All shields present at connectors mounted on an enclosure shall be terminated to the exterior of the enclosure.

        - Differential signal transmission

Standard interface signals shall be differentially transmitted and received to reduce common mode electromagnetic coupling between the signal and other wires.  Each differential transmitter shall be source-terminated to the characteristic impedance of its transmission line and shall be DC-isolated from its enclosure by at least 1 Megohm.  Each differential receiver shall provide DC-isolation of at least 1 Megohm between each wire of the differential pair to each other and the enclosure chassis.  The isolation requirements will prevent signal currents from contaminating the ground plane.  The timing skew between the signal edges of any pair shall be no greater than 6 ns.  The timing skew requirement will reduce unintentional radiation from the signal pair.

      - Pegasus launch vehicle interface signals

The SMEX-FAST interface to the Pegasus launch vehicle is the standard interface offered by Orbital Sciences Corporation (OSC).  This interface shall be in accordance with the Interface Control Document section of FAST-SPEC-013.

    - PC board and motherboard requirements

Power distribution on multiplayer PC boards and motherboards shall use at least one layer as a ground plane and at  least one layer for supply voltages.  It is recommended that PC boards and motherboards using multiple supply voltages should distribute these voltages on separate layers if practical, or as a minimum, use separate layers to distribute power for digital and analog circuits.  The recommendations on power distribution will help to minimize the series impedance in the power circuit, leading to reduction in common impedance coupling of interference.  All PC boards shall use bypass capacitors located as near to the power point of entry to the board as practical.  Each integrated circuit shall have bypass capacitors located near the power pins of its package.  It is recommended that long, closely-spaced signal tracks be avoided if practical to minimize coupling effects. Single-ended transmission of signals from PC board to PC board is allowed.  If necessary, sensitive signals may be shielded from on-board interference by sandwiching the signal tracks between 2 ground planes.  Sensitive signals may also be shielded at the PC board connector by surrounding the signal pin by grounded pins, if practical.  It is strongly recommended that potential EMI emitters and receptors be identified and separated and/or shielded from each other in the motherboard.

  - Wire harness requirements

    - Power and signal classes

To the maximum extent possible, wires in the following classes shall not be run in bundles containing wires of another class.

      - Class 1 wires

The following wires are defined to be Class 1 wires:

- DC circuit wiring carrying steady-state or transient currents above 1A,

- Relay and solenoid wiring, and

- All primary power wiring.

      - Class 2 wires

The following wires are defined to be Class 2 wires:

- wires carrying currents between 50 Ma and 1A, and

- wires carrying digital signals.

      - Class 3 wires

The following wires are defined to be Class 3 wires:

- wires carrying signals to sensitive instruments,

- wires carrying signals to sensitive circuits, and

- instrumentation wiring.

      - Class 4 wires

Cables carrying transmitted and received RF between the transponder and antennae are defined to be Class 4 wires.

      - Class 5 wires

Meltwire and electro-explosive device (EED) wires are defined to be Class 5 wires.

    - Wire harness mechanical requirements

All wire harness runs shall be routed as close to the spacecraft structure as possible to reduce magnetic dipole moments.  All wire harness runs shall be clamped to the structure at frequent intervals to minimize generation of fields caused by relative motion between the harness and the structure.  Parallel runs of  bundles of differing class shall be avoided to the maximum extent possible.

  - DC-DC converter requirements

DC-DC converters shall meet the requirements defined in paragraph 3.8.3 of FAST-SPEC-005.

  - Switched inductive load spike suppression

Switched inductive loads, such as relay coils and the like, shall be bypassed with a parallel, reversed-biased diodes to suppress high-voltage inductive spikes caused when current flow in interrupted.

  - Open collectors

In those cases where single-ended signals are used between enclosures, driving

  - Solar array wiring

Solar array wiring shall be in accordance with paragraph 3.9.7 of FAST-SPEC-005.

  - Filtered-pin connectors

Filtered pin connectors shall be considered as one option to prevent conducted EMI from entering into and escaping from an enclosure.

- QUALITY ASSURANCE

  - Test philosophy

The system, subsystems, and components shall meet the applicable portions of MIL-STD-461 and MIL-STD-462 as interpreted by this document.  The spacecraft shall operate without undesirable EMI interactions with the Pegasus launch vehicle.  Test plans shall be developed and tests performed by electrical systems engineer subject to the review of OSC to assure spacecraft/Pegasus compatibility.  Tests shall be performed on subsystems and components to show compliance to the applicable requirements and to provide information that would enhance the compatible integration of the subsystem/component into the overall spacecraft.  Modifications and changes to component design after EMI testing shall be reviewed by the electrical systems engineer to determine the need for additional testing.

  - Test plans

An EMI test plan shall be prepared for the spacecraft and each compenent/subsystem in accordance with Table 5.2-1. As a minimum, the plans shall include the following details:

- the means of implementation and application of the needed test procedures,

- nomenclature, description and mode of operation of the test sample,

- description and RF ambient profile of the test site (open space, anechoic, and/or shielded enclosure,

- test frequencies of which extraneous outputs, susceptibilities, and intermodulation products may be expected,

- considerations and regulations regarding the operation of the test sample and measureing equipment in unshielded areas (eg. FCC or FAA regulations),

- personnel required, including GSFC, UCB and OSC representatives and performance personnel,

- readout and detector functions to be used in measuring equipment, where applicable,

- dummy loads, filters, dummy antennae, signal samplers, and similar items to be used and their description (eg. VSWR, isolation, and loss) in the range of interest,

- method of loading and triggering the test sample during tests,

- method and criteria for determining malfunction or degradation of performance during susceptibility tests,

- expected overall accuracy of measurements and external influencing conditions, and

- detailed list of test equipment.

7.3	Test procedure

The spacecraft EMI test procedure shall be prepared by the electrical systems engineer.  An EMI test procedure shall be prepared for each subsystem and component test by the responsible lead engineer and approved by the electrical systems engineer.  The test procedure shall contain the detailed method of testing including but not limited to the following:

- detailed step-by-step procedures and test set-ups with maximum use of photographs, drawings and diagrams,

- nomenclature, serial numbers and general characteristics of test equipment (eg. Transfer impedance of current probes and effective lengths of antennae), and

- methods and dates of last calibration of interference measuring equipment and calculations to show expected accuracy of each where required.

7.4	Test fixture

The test fixture used to support the spacecraft shall be constructed such that significant RF potential differences shall not exist within the structure.  The fixture shall be bonded to the shielded enclosure by a maximum DC resistance of 2.5 milliohms.  The test fixture shall make provisions for the bonding of the spacecraft to the fixture in accordance with the bonding requirements of this document.

- NOTES

  - Acronyms

ACS	attitude control subsystem

dB     	decibel

DC	direct current

EED	electro-explosive device

EMC	electro-magnetic compatibility

EMI	electro-magnetic interference

FAA	Federal Aviation Administration

FAST	fast auroral snapshot

FCC	Federal Communications Commission

GSFC	Goddard Space Flight Center

MUE	mission unique electronics

OSC	orbital sciences corporation

PC	printed circuit

R-C	resistor-capacitor

RF	radio frequency

SMEX	small explorer

UCB	University of California, Berkeley

VSWR	voltage standing wave ratio

  - EMI checklists

These checklists have been provided as a helpful reminder of good EMI control practices.  A ‘yes’ or ‘not applicable’ answer to any question requires no further thought.  A ‘no’ answer requires careful consideration of the potential EMI impact and should be resolved by discussion with the electrical systems engineer.

8.2.1	Checklist A: circuits to be shielded and filtered

- Have chopper, converter, relay, motor, clock or timing circuits with fast rise time or high PRF, and similar circuits been filtered using filters selected for their attenuation characteristics at the noise frequencies?

- Has the DC-DC converter transformer been electrostatically shielded?

- Has bandpass filtering been used on transmitter outputs or receiver inputs?

- Is the shielding material suitable for the frequency range of interest?

- Have decoupling capacitors been used on internal power connections?

- Have shielded subassemblies been used in the equipment?

- Have RF chokes and inductors been used to confine the RF energy to the desired circuits?

- Have low-level or susceptible circuits been physically separated from EMI producing circuits within an enclosure?

- Have toroids been used to minimize the leakage field of inductors and have inductors been cross-oriented to minimize coupling?

- What effect do vent holes have on the box’s radiated emissions.?

    - Checklist B: elimination of spurious responses

- Have short lead lengths been used for all circuits, especially capacitors in RF circuits?

- Has damping been used in circuits capable of oscillation?

- Have critical dimensions been avoided, considering the enclosure as an RF cavity?

- Have all feedback loops been designed to prevent oscillation under worst-case conditions?

- Have high-power and low-power stages of units been isolated?

- Is the proposed enclosure bonding adequate at the known critical radio frequencies?

- Have components that are not self-resonant in the intended frequency range been used except when required by the circuit functionality?

    - Checklist C: selection of components

- Are diodes or other suppression components being used across relay coils?

- Are R-C circuits being used across relay contacts?

- Are capacitors being used directly across DC motor brushes?

- Are toroids or other low-leakage inductors being used?

- Are separate connectors being used for sensitive and EMI-producing circuits?

- Is twisted-pair and/or shielded wiring being used?

- Is balanced-circuit design being used?

    - Checklist D: electro-explosive devices

- Has the EED been bonded to the structure per MIL-B-5087?

- Have firing circuit conductors been routed as twisted-shielded pairs?

- Have bundle shields been circumferentially-terminated?

- Have EED cables been isolated in accordance with the wire harness separation classifications?

- Internal to components, has EED wiring been isolated from all other wires?

- Have separate connectors been provided for EED wiring?

Table 4.4-1: FAST frequencies

| Unit | Signal type | Frequency |
| --- | --- | --- |
| IDPU | Processor oscillator | 20.0000 MHz |
|  | Processor clock | 10.000 MHz |
|  | Processor primary cycle A | 2.5000 MHz |
|  | Processor primary cycle B | 2.0000 MHz |
|  | Processor primary cycle C | 1.6667 MHz |
|  | UART oscillator | 153.8462 KHz |
|  | UART lock | 9.6154 KHz |
|  | S/C clock | 4.1943 MHz |
|  | S/C sync pulse | 1.0000 Hz |
|  | Memory interface clock | 4.1943 MHz |
|  | Memory cycle rate | 419.4304 KHz |
|  | ADC sample rate | 32.7680 KHz |
|  | ADC clock | 2.6214 MHz |
|  | ESA shift clock | 1048.5760 KHz |
|  | Spin phase clock | 3276.8000 KHz nominal |
|  | DSP system clock | 32.0000 MHz |
|  | DSP serial shift clock | 16.0000 MHz |
|  | DSP parallel input clock | 8.0000 MHz |
|  | Wave-particle correlator system clock | 16.0000 MHz |
|  | H/S burst memory system clock | 4.0000 MHz |
|  | H/S burst memory sample clock A | 2.0000 MHz |
|  | H/S burst memory sample clock B | 1.0000 MHz |
|  | H/S burst memory sample clock C | 500.0000 KHz |
|  | H/S burst memory sample clock D | 250.0000 KHz |
|  | IDPU and instrument DC-DC converters | 62.0000 KHz nominal |
| Transponder | Transmit | 2214.9999 MHz |
|  | Receive | 2039.6458 MHz |
|  | 2F1 | 18,458,333 Hz |
|  | 13F1 | 119,979,166 Hz |
|  | 26F1 | 239,958,333 Hz |
|  | 30F | 276,875,000 Hz |
|  | 208F1 | 1,919,666,667 Hz |
|  | F2 | 12.25 MHz |
|  | 2F2 | 24.5 MHz |
|  | Fosc | 4.096 MHz |
|  | Fosc/2 | 2.048 MHz |
|  | Transponder DC-DC converter | 62.5 KHz |
| MUE | Processor card oscillator | 8 MHz |
|  | S/C clock, Up/downlink | 18 MHz |
|  | S/C clock | 4,194,304 Hz |
|  | Coil driver converter, MUE DC-DC converter | 125KHz |
|  | RS-422 clock | 151 KHz |
|  | Other | 1 Hz |
|  | Other | 64 Hz |
|  | Other | 4.096 KHz |
|  | Other | 1 MHz |
|  | Other | 2 MHz |
|  | Other | 4 MHz |

Table 4.4-1: FAST Frequencies (continued)

| Unit | Signal type | Frequency |
| --- | --- | --- |
| ACS sensor/actuators | Spin coil PRF | 12 Hz |
|  | Sun sensor DC-DC converter | 6 KHz |
|  | ACS magnetometer drive frequency 2nd harmonic | 15 KHz 30KHz |
| Science instruments | Wire & axial boom sensors | DC to 2 MHz |
|  | Search coil magnetometer | 10 Hz to 1 MHz |
|  | Fluxgate magnetometer received passband Drive frequency 2nd harmonic | DC to 100 Hz 9 KHz 18 KHz |

Table 5.2-1: EMI Requirements/verifications per level of assembly

| Requirement | MIL-STD-462 test method | Reference paragraph number | Level of assembly Spacecraft Box |  |
| --- | --- | --- | --- | --- |
| DC power leads (low frequency) | CE01 | 5.2.1.1 | N | A |
| DC power leads (high frequency) | CE03 | 5.2.1.2 | N | A |
| Antenna terminal | CE06 | 5.2.1.3 | N | B |
| DC power leads (low frequency) | CS01 | 5.2.2.1 | N | A |
| DC power leads (high frequency) | CS02 | 5.2.2.2 | N | A |
| Intermodulation | CS03 | 5.2.2.3 | N | B |
| Spurious rejection | CS04 | 5.2.2.4 | N | B |
| Cross modulation | CS05 | 5.2.2.5 | N | B |
| DC power spikes | CS06 | 5.2.2.6 | N | A |
| Electric fields | RE02 | 5.2.3.1 | S | C |
| Magnetic fields | RE04 | 5.2.3.2 | S | C |
| Electric fields | RS03 | 5.2.4 | S | C |
| DC magnetic properties | N/A | 5.2.5 | S | C |

LEGEND

A = test performed on all units interfacing to spacecraft power, including switched power and Pegasus    interface power (solar arrays, batteries, MUE, IDPU, shunt boxes)

B = test performed on transponder

C = test performed on all boxes

N = no test performed at this level

S = test performed on spacecraft

CE = conducted emissions				RE = radiated emissions

CS = conducted susceptibility			RS = radiated susceptibility

## Table 5.2.5-1: Magnetization Requirements

| Type | Background Magnetization (Tesla) | Maximum Magnetic Field (nT) at 50 cm at 100 cm |  |
| --- | --- | --- | --- |
| Initial permanent magnetization | 0 | 32 | 4 |
| After exposure to 0.0015 Tesla magnetizing field | 0 | 64 | 8 |
| After exposure to 0.005 Tesla demagnetizing field | 0 | 16 | 2 |
| Induced magnetization | 3 x 10-5 | 16 | 2 |
| Stray fields DC 0.01 – 0.1 Hz 0.1 – 1.0 Hz | 0 0 0 | 16 8 8 | 2 1 1 |

Table 5.3-1 Pegasus RF Environment

| Source | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Function | Command destruct | Tracking transponder | Tracking transponder | Alternate telemetry | Booster telemetry |
| Role | Receive | Transmit | Receive | Transmit | Transmit |
| Band | UHF | C-band | C-band | S-band | S-band |
| Frequency (MHz) | 416.5 | 5765 | 5690 | 2269.5 | 2288.5 |
| Bandwidth | 180 KHz | N/A | 14 MHz @ 3dB | TBD | 315 KHz @ 3 dB |
| Power output | N/A | 400 W Peak | N/A | 5 W | 5 W |
| Sensitivity | -107 dBm | N/A | -70 dBm | N/A | N/A |
| Modulation | FM | Pulse Code | Pulse Code | FM | PCM/FM |

## Table 5.3-2 Carrier Aircraft RF Environment

| Source | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Function | Comm | Comm | Telemetry | Video telemetry | Video telemetry |
| Role | Receive-transmit | Receive-transmit | Transmit | Transmit | Transmit |
| Band | UHF | UHF | L-Band | C-Band | S-Band |
| Frequency (MHz) | 268.1 | 286.8 | 1435-1525 | 4583 | 1727 |
| Bandwidth | Standard A/C radio | Standard A/C radio | 1 MHz @ 60 dB | 6 MHz @ 60 dB | 6 MHz @ 60 dB |
| Power output | Standard A/C radio | Standard A/C radio | 5 W | 10 W | 10 W |
| Sensitivity | Standard A/C radio | Standard A/C radio | N/A | N/A | N/A |
| Modulation | Standard A/C radio | Standard A/C radio | FM | FM | FM |

## Figure 5.2.2.4-1:	CS04 Limits

f0	receiver tuned frequency or band center for amplifiers.

f1	lowest tunable frequency of receiver band in use or the lowest frequency of amplifier passband

f2	highest tunable frequency of receiver band in use or the highest frequency of amplifier passband

W	bandwidth between the 60 dB points of the receiver selectivity curve as defined in the test  sample’s technical requirements on the control plan.

Limits:

  - The limit at A is 60 dB above the input level required to produce the standard reference output.  The standard reference output is based on a -112dBm input.

  - The limit at B shall be set as follows:

  - Receivers: 0 dBm applied directly to the receiver input terminals.

  - Amplifiers:  the limit shall be as specified in the test sample’s technical requirement or control plan.  If no limit is defined in the above documents, the 0 dBm value shall be used.

Figure 5.2.2.6-1: CS06 Limits