Goddard Space Flight Center

SPACE TECHNOLOGY 5

SPACECRAFT

EMI / EMC TEST PROCEDURE

ST5-495-423

September 2004

As Run Date: ________	    S/N: __________ WOA #: _______________

THIS PROCEDURE CONTAINS HAZARDOUS OPERATIONS

ST5 Configuration Controlled Document

Any changes require ST5 Project Approval.

Check the ST5 Project website at http://st5.gsfc.nasa.gov to

verify that this is the correct version prior to use.

Prepared by:	_________________________		Date:	__________

Marty Brown, I&T TC

Reviewed by:	_________________________		Date:	__________

Tom Gostomski, I&T Lead

Reviewed by:	_________________________		Date:	__________

Karen Stewart, Power Lead

Reviewed by:	_________________________		Date:	__________

George Jackson, CDH Lead

Reviewed by:	_________________________		Date:	__________

Peter Rossoni, Mechanical Lead

Reviewed by:	_________________________		Date:	__________

Sam Placanica, GN&C Lead

Reviewed by:	_________________________		Date:	__________

Angie Russo, GN&C Hardware Lead

Reviewed by:	_________________________		Date:	__________

Jason Badgley, Propulsion Engineer

Reviewed by:	_________________________		Date:	__________

Mike Rhee, Propulsion Lead

Reviewed by:	_________________________		Date:	__________

Donya Douglas, Thermal / VEC Lead

Reviewed by:	_________________________		Date:	__________

Terry Sullivan, Electrical Systems Lead

Reviewed by:	_________________________		Date:	__________

Keith Chamberlin, S/C Systems Lead

Reviewed by:	_________________________		Date:	__________

Evan Webb, Systems Lead

Reviewed by:	_________________________		Date:	__________

Ted Dyer, EMI / EMC Group

Approved by:	_________________________		Date:	__________

Rick Stickle, Safety Engineer

Approved by:	_________________________		Date:	__________

Roger Counts, Quality Assurance

REVISION/CHANGE RECORD

| Revision | DATE | AUTHORIZATION | REVISION/CHANGE DESCRIPTION | PAGES AFFECTED |
| --- | --- | --- | --- | --- |
| - |  |  | Initial Release | ALL |

1.	INTRODUCTION	8

2.	TEST DESCRIPTION	8

2.1.	Spacecraft Conducted Emissions (CE01, CE03 & CE CMN)	8

2.2.	Spacecraft Self Compatibility	8

2.3.	Spacecraft Self Compatibility with X-Band	8

2.4.	Spacecraft Narrowband Radiated Emissions (RE02)	9

2.5.	Spacecraft Broadband Radiated Emissions (RE02)	9

2.6.	Spacecraft Radiated AC Magnetic Field Emissions (RE04)	9

2.7.	Spacecraft Radiated Susceptibility (RS03)	9

2.8.	Tests To Be Performed	9

2.9.	Test Methods	9

2.10.	Test Limits/Levels	10

2.11.	PURPOSE	10

2.12.	SCOPE	10

2.13.	CHANGES TO THIS PROCEDURE	10

2.14.	PROBLEM REPORTING	10

2.15.	POST TEST BRIEFING REQUIREMENTS	10

2.16.	TRANSPORT AND TIME REQUIREMENTS	10

3.	REFERENCE DOCUMENTS	11

3.1.	REFERENCES	11

3.2.	LIST OF STOL PROCEDURES	11

3.3.	LIST OF EMERGENCY PROCEDURES	11

4.	FACILITY ENVIRONMENTAL REQUIREMENTS	12

4.1.	FACILITY	12

4.2.	ENVIRONMENTAL REQUIREMENTS	12

4.2.1.	Temperature & Humidity	12

4.2.2.	Contamination Control Requirements	12

4.2.3.	Electrostatic Discharge Control Requirements	12

5.	PERSONNEL REQUIREMENTS	13

5.1.	PERSONNEL PROTECTIVE EQUIPMENT	13

5.2.	REQUIRED PERSONNEL	13

5.2.1.	Spacecraft Test Conductor (TC)	13

5.2.2.	spacecraft  Electrical & mechanical Integration Technicians	13

5.2.3.	Quality Representative	13

5.2.4.	SUBSYSTEM ENGINEER	13

5.3.	SIGNOFF PERSONNEL	14

5.4.	Additional Support Personnel	14

5.5.	PERSONNEL ASSIGNMENTS AND RESPONSIBILITY REVIEW	14

6.	HAZARDS	14

7.	SAFETY	15

7.1.	SAFETY SUPPORT	15

7.2.	SAFETY PRECAUTIONS	15

7.2.1.	TEMPORARY TEST CONFIGURATIONS	15

7.2.2.	WARNINGS DURING TEST	15

8.	ELECTROSTATIC DISCHARGE (ESD)	16

8.1.	Electrostatic Discharge (ESD) Prevention	16

8.2.	Meter Measurement Considerations	16

8.3.	BOB and Harness Installation Considerations	16

9.	GSE, TOOLS AND EQUIPMENT	17

9.1.	EQUIPMENT CALIBRATION	17

9.2.	GSE	17

9.2.1.	EGSE	17

9.2.2.	EMI/EMC EGSE	18

9.2.3.	MGSE	18

9.3.	TOOLS AND EQUIPMENT LIST	19

9.4.	COMMUNICATIONS DISCIPLINE	19

9.4.1.	Voice	19

9.5.	BOB INSTALLATION	19

10.	EGSE CONFIGURATION	20

10.1.	NORMAL SPACECRAFT EGSE TEST CONFIGURATION	20

11.	PRE-ENVIRONMENTAL TEST ACTIVITIES / VERIFICATIONS	21

11.1.	HARDWARE S/N RECORDING	21

11.2.	HARDWARE GSE GROUND VERIFICATION	21

12.	TEST SEQUENCE	22

12.1.	CONFIGURE GSE AND SPACECRAFT	22

12.1.1.	Configure Spacecraft EGSE NORMAL Setup per Figure 12.1.1	22

12.1.2.	Safe To Mate	22

12.1.3.	Verify the Configuration of Spacecraft EGSE Setup per Figure 12.1.1	22

12.1.4.	Spacecraft Pre-EMI Functional Test	23

12.2.	CONDUCTED EMMISSIONS & COMMON MODE NOISE TESTS	24

12.2.1.	Configure Spacecraft EGSE for CE01 Test (7.2V Bus) per Figure 11.2.1	24

12.2.2.	Verify that BUS Cable adaptor (GD 2059236-1 P2) is connected to Flight Harness P22 (GD 2054900). Record in the Mate/De-Mate Log if Connector Saver is not used.	24

12.2.3.	Verify that BUS Cable adaptor (GD2059236-1) pigtail Lugs are connected to the Oscilloscopes as follows.	24

12.2.4.	Connect an Unpopulated 44 pin BOB to BUS Cable adaptor (GD 2059236-1) P1 looking into the Flight Harness. (Ref. 8.5)	25

12.2.5.	Measure the Resistance P22 looking into the Flight Harness side of the BOB between the following pins: (Ref. 8.5)	25

12.2.6.	Remove the BOB from BUS Cable adaptor GD 2059236-1 P1. (Ref. 8.5)	25

12.2.7.	Connect BUS Cable adaptor GD 2059236 P1 to C&DH J22	25

12.2.8.	Verify EMI/EMC Test Equipment is isolated from AC ground.	25

12.2.9.	Ambient Room Scan	25

12.2.10.	Ambient Room “EGSE ON” Scan	25

12.2.11.	Power ON the Spacecraft	26

12.2.12.	Configure the Spacecraft for Worst Case Emissions Mode	26

12.2.13.	Verify that the HPA is OFF.	27

12.2.14.	Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (+) SIDE	27

12.2.15.	Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE	27

12.2.16.	Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus	28

12.2.17.	Turn ON the HPA	28

12.2.18.	Repeat with HPA ON +5V & +7.2V Bus (+) SIDE	28

12.2.19.	Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE	28

12.2.20.	Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE	28

12.2.21.	Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus	28

12.2.22.	Turn OFF the HPA	29

12.2.23.	CE01 Test Results	29

12.2.24.	CMN Test Results	29

12.2.25.	Recharge Battery if voltage is <= 7.5V.	29

12.2.26.	Record the Battery Voltage and Current.	29

12.2.27.	Perform CE03 +5V & +7.2 V Bus (+) SIDE	29

12.2.28.	Perform CE03 +5V & +7.2 V Bus (-) SIDE	29

12.2.29.	Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus	29

12.2.30.	Turn ON the HPA	30

12.2.31.	Repeat with HPA ON +5V & +7.2V Bus (+) SIDE	30

12.2.32.	Perform CE03+5V & +7.2 V Bus (-) SIDE	30

12.2.33.	Perform CE03 +5V & +7.2 V Bus (-) SIDE	30

12.2.34.	Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus	30

12.2.35.	Turn OFF the HPA	30

12.2.36.	CE01 Test Results	30

12.2.37.	CMN Test Results	31

12.2.38.	Recharge Battery if voltage is <= 7.5V.	31

12.2.39.	Record the Battery Voltage and Current.	31

12.2.40.	Power OFF the Spacecraft	31

12.3.	Self Compatibility	32

12.3.1.	Reconfigure the Spacecraft test setup Self Compatibility test per Tables & Figure 12.3.1	32

12.3.2.	Verify the Configuration of Spacecraft EGSE Setup per Figure 9.4.1	34

12.3.3.	Power ON the Spacecraft by execution of the STOL procedure st5intgpwron at the ASIST Primary workstation.	34

12.3.4.	Configure the Spacecraft for Most Susceptible Mode	34

12.3.5.	Verify the Spacecraft Thruster is in Delta-V mode.	34

12.3.6.	Perform Self Compatibility test	34

12.3.7.	Power OFF the Spacecraft	34

12.4.	RADIATED EMISSIONS	35

12.4.1.	Reconfigure for Radiated Emissions (RE02) according to Tables & Figure 12.5.1	35

12.4.2.	Verify the Configuration of Spacecraft EGSE Setup per Figure 9.4.1	37

12.4.3.	Power ON the Spacecraft by execution of the STOL procedure	37

12.4.4.	Configure the Spacecraft for Worst Case Emissions Mode	37

12.4.5.	Perform RE02 (Radiated Narrowband Electric Field)	37

12.4.6.	Power OFF the Spacecraft	37

12.4.7.	Recharge Battery if voltage is <= 6.5V.	37

12.4.8.	Record (RE02) Noise Locations	37

12.4.9.	Reconfigure for Radiated Emissions RE04 Test according to setup per Table & Figure 12.5.9	38

12.4.10.	Power ON the Spacecraft by execution of the STOL procedure	40

12.4.11.	Configure the Spacecraft for Maximum Magnetic Emissions Mode	40

12.4.12.	Perform RE04 (Radiated AC Magnetic Field Emissions –Maximum Magnetic Emissions Mode)	40

12.4.13.	Configure the Spacecraft for Maximum Magnetic Emissions Mode.	40

12.4.14.	Perform RE04 (Radiated AC Magnetic Field Emissions –Science Data Collection Mode)	40

12.4.15.	Power OFF the Spacecraft	40

12.4.16.	Recharge Battery if voltage is <= 6.5V.	40

12.4.17.	Record (RE04) Noise Locations	40

12.4.18.	Radiated Emissions (RE04) Setup Photos.	42

12.5.	RADIATED SUSCEPTIBILITY	43

12.5.1.	Reconfigure for Radiated Susceptibility RS03 according to Tables & Figure 12.6.1	43

12.5.2.	Verify the Configuration of Spacecraft EGSE Setup per Figure 12.6.1	45

12.5.3.	Power ON the Spacecraft by execution of the STOL procedure	45

12.5.4.	Configure the Spacecraft for Most Susceptible Mode.	46

12.5.5.	Perform RS03 (Radiated Susceptibility Electric Field)	46

12.5.6.	RS03 Test Results	46

12.5.7.	Power OFF the Spacecraft	46

12.5.8.	Recharge Battery if voltage is <= 6.5V.	46

12.5.9.	Configure Spacecraft EGSE NORMAL Setup per Figure 9.0.1	46

12.5.10.	Spacecraft Aliveness Test	46

12.6.	RF Self Compatibility	47

12.6.1.	Reconfigure for RF Self Compatibility according to Tables & Figure 12.7.1	47

12.6.2.	Power ON the Spacecraft	48

12.6.3.	Turn ON the HPA	49

12.6.4.	Turn OFF the HPA	49

12.6.5.	Power OFF the Spacecraft	49

12.7.	POST EMI FUNCTIONAL TEST	50

12.7.1.	Spacecraft Post EMI Functional Test	50

13.	TEST COMPLETION	51

13.1.	Problem Record Disposition	51

13.2.	Test Completion Report/ WOA Closeout	51

14.	TEST SEQUENCE / SUMMARY	52

15.	TEST SCRIPT SUMMARY	53

15.1.	TBD_WORST_EM.PRC	53

15.2.	TBD_MOST_SUS.PRC	53

15.3.	TBD_RAD_SCI_ COL.PRC	54

15.4.	TBD_CON_SCI_ COL.PRC	55

15.5.	TBD_MAX_MAG.PRC	56

15.6.	TBD_HPA_LOOP.PRC	56

# INTRODUCTION

This document is the test procedure to be used for the ST5 Spacecraft EMI/EMC test.

# TEST DESCRIPTION

This EMI/EMC testing will consist of Conducted Emission measurements, Spacecraft Self Compatibility (similar to Conducted Susceptibility without noise injection), Radiated Emissions, and Radiated Susceptibility testing.

Conducted Emissions Measurements and Self Compatibility testing will be performed in Bldg. 29 ST5 I&T area, prior to S/C final button up, due to accessibility of the PSE Connectors J12 and J22 prior to Spacecraft closure.

Radiated Emissions, and Radiated Susceptibility testing will be performed at the Clean EMI/EMC SCA facility in Bldg. 7, 1st floor. GEVS levels will be used.

Spacecraft Functional tests will be performed before and after EMI/EMC testing.

See Section 7.0 for the EMI/EMC Test Flow.

The EGSE will be fused at 6 amps for both the BattSim and DPC services, and at .375 amp for each S/A service.  Both power and return lines will be fused.

See Figure 4.5.1 for EGSE to Spacecraft configuration.

## Spacecraft Conducted Emissions (CE01, CE03 & CE CMN)

With all S/C Subsystems powered ON and in nominal on-orbit mode of operation, which will generate the most EMI, the S/C power bus is characterized using oscilloscope and spectrum analyzer. The bus characterization is plotted against ST5 unique CE01, CE03 & CE CMN limits. This data may then be used to identify the source of any anomalies encountered during S/C Self-Compatibility testing and for comparison to component level conducted susceptibility tests. It is required that the GSFC EMI/EMC group provides the oscilloscope and spectrum analyzer for this test.

## Spacecraft Self Compatibility

With all S/C Subsystems, Instruments and Experiments (heretofore referred to as Subsystems) powered ON and in a nominal on-orbit mode of operation, which is most susceptible to EMI, each Subsystem is individually exercised through modes of operation, which generate the most EMI. Upsets in telemetry, unexpected change of state or corruption of Science data is indicative of EMI susceptibility. Voltage measurements of the Power Bus shall be made in the time domain using an oscilloscope having a minimum band pass of 100 MHz. The amplitude of the voltage ripple shall be no greater than 0.5Vpp. It is required that the GSFC EMI/EMC group provides the oscilloscope for this test. Record load ON/OFF times and results on Data Sheet (Appendix D).

## Spacecraft Self Compatibility with X-Band

With the S/C placed in its Worst Case On-Orbit emissions mode/Most Susceptible On-Orbit mode, the X-band transmitter will be turned on. “NO-OP” commands will be sent to the S/C. Telemetry will be monitored for any anomalous change in state or behavior. X-band system threshold testing will also be performed at this time. This test will verify S/C self-compatibility with the X-band RF system.

## Spacecraft Narrowband Radiated Emissions (RE02)

With the S/C in its worst-case on-orbit emissions mode, a probe will be used to identify locations around the S/C where radiated emissions are at the highest levels. An antenna will be positioned one meter from these locations and will be used to measure Narrowband Radiated Emissions. This location will be annotated on Figure TBD. The collected data will be plotted against the limits specified for the S/C. This data will be used to ensure compatibility with the Pegasus ELV. An integrated worst case GEVS and ELV levels will be used for this test. See Figure TBD. If the S/C fails the test, the test will be rerun with the S/C in its Science Data Collection mode.

## Spacecraft Broadband Radiated Emissions (RE02)

With the S/C in its worst-case on-orbit emissions mode, and the antenna locations identified above Broadband Radiated Emissions sweep will be performed. The collected will be plotted against the limits specified for S/C. See Figure TBD. This data will be used to ensure compatibility with the Pegasus ELV.

## Spacecraft Radiated AC Magnetic Field Emissions (RE04)

With the S/C in it’s worst-case on-orbit emissions mode, and using the antenna locations identified above AC Magnetic Field Radiated Emissions sweep will be performed. The collected will be plotted against the limits specified for S/C. See Figure TBD. This data will be used to ensure compatibility with the Pegasus ELV.

## Spacecraft Radiated Susceptibility (RS03)

Using several types of antennas, the S/C will be exposed to RF energy representative of RF sources seen at the launch site, from the launch vehicle and on-orbit from ground base radar. The S/C will be placed in its most susceptible on-orbit mode. During this testing, telemetry will be monitored for any anomalous change in state or behavior. An integrated OSC Pegasus ELV and GEVS Spec levels will be used for this test. Record Sweep times and results on Data Sheet (Appendix E).

## Tests To Be Performed

The tests to be performed are listed in Table 2.8.1. See Appendix C for the ST5 Spacecraft Test Flow.

| Test Number | Test | GEVS Spec | MIL-STD |
| --- | --- | --- | --- |
| CE01/CE03 | Conducted Emissions Measurements | 2.5.2.1.a & c | MIL-STD-462 |
| CE CMN | Common Mode Noise | 2.5.2.1.b | MIL-STD-461C & 462 |
| Self Compatibility | Spacecraft Self Compatibility Test | ST5 Project |  |
| RE02 | Radiated Narrowband Electric Field | 2.5.2.2.c | MIL-STD-461C & 462 |
| RE04 | Radiated AC Magnetic Field Emissions | 2.5.2.2.b | MIL-STD-461C & 462 |
| RS03 | Radiated Susceptibility Electric Field | 2.5.3.2.a | MIL-STD-461C & 462 |
| Self Compatibility W/RF | Spacecraft Self Compatibility Test W/RF | ST5 Project |  |

Table 2.8.1

## Test Methods

The Spacecraft is tested in accordance with methods established in MIL-STD-461C & 462, Notice 1, and MIL-STD-463.  The test set-ups shall be similar to those specified in MIL-STD-462.

## Test Limits/Levels

Test limits/levels are specified in the TBV (Mission Requirements Document  (ST5-495-051)), as amended from GEVS-SE and MIL-STD-461C, 462, and 463.  RE and RS testing are performed to GEVS-SE levels.  Conducted Emissions Measurements and Spacecraft Self Compatibility testing are performed to ST-5 unique levels.  The test limits/levels are specified in this document.

## PURPOSE

The purpose of this procedure is to safely run and qualify the ST5 Spacecraft through EMI/EMC testing.

## SCOPE

This procedure applies to the EMI / EMC testing of the ST5 Spacecraft.

## CHANGES TO THIS PROCEDURE

Discrepancies in this procedure discovered just prior to or during performance of the procedures shall be redlined, initialed and dated on the official copy of the procedures by the Test Conductor (TC). The Quality Assurance (QA) representative shall also initial all changes.

## PROBLEM REPORTING

Anomalous behavior of the test article is defined as failure to meet the expected criteria delineated by this procedure.  All anomalous behavior of the test article, test equipment, software, or other items associated with the test procedures shall be documented in accordance with the ST-5 Integration and Test Plan, ST5-495-060.  Work Order controls will be used to control any process continuation.

In short, a PR/PFR will be generated against the anomaly, referenced back to the WOA.  Additionally, the PR/PFR will be entered into the Problem Record (PR) section of the Spacecraft Log Book.

## POST TEST BRIEFING REQUIREMENTS

Successful completion of this procedure must be reported to the Cognizant Quality representative, Integration and Test Manager, and the Program Office so that the required steps may be performed to insure data integrity and that the proper documentation is completed. Successful completion must be noted on the Spacecraft Log Book.

## TRANSPORT AND TIME REQUIREMENTS

The transport of the test article must follow the guidelines presented in the ST-5 Contamination Control Plan (ST5-495-105), ST-5 Magnetic Control Plan (ST5-495-058), ST-5 ESD Control Plan (ST5-495-199), and the System Safety Program Plan (ST5-495-040). A shipping container will be supplied by the Mechanical Engineering Group to ensure compliance to ST5 program requirements.

# REFERENCE DOCUMENTS

## REFERENCES

The appropriate reference documents for this test are listed below.

| Document No. | Document Title |
| --- | --- |
| ST5-495-361 | Spacecraft EMI/EMC Test Plan |
| ST5-495-040 | System Safety Program Plan |
| ST5-495-051 | Mission Requirements Document for ST5 Project |
| ST5-495-105 | Contamination Control Plan |
| ST5-495-199 | Electrostatic Discharge Control (ESD) Plan |
| ST5-495-305 | Umbilical Console Integration Verification Procedure S/C #1 |
| ST5-495-414 | Umbilical Console Integration Verification Procedure S/C #2 |
| ST5-495-415 | Umbilical Console Integration Verification Procedure S/C #3 |
| ST5-495-129 | Spacecraft Functional Test |
| GD 2059236-1 | ST5 Harness J22 BUS Adapter |
| GD 2059247 | ST5 Harness J12 SA Adapter |

## LIST OF STOL PROCEDURES

The STOL procedures to be used for this test are listed below.

| NAME | DESCRIPTION |
| --- | --- |
| st5intgpwron.prc | Power ON Spacecraft |
| St5_emi_cfg.prc | Spacecraft Configuration Procedure |
| st5intgpwroff.prc | Power OFF Spacecraft |

## LIST OF EMERGENCY PROCEDURES

The emergency procedures for this test are listed below.

| NAME | Document Title |
| --- | --- |
| st5eoff.prc | ST5 Emergency power off Procedure |

# FACILITY ENVIRONMENTAL REQUIREMENTS

## FACILITY

Spacecraft EMC / EMI Conducted Emissions and Self Compatibility testing will occur at the GSFC, Greenbelt, Maryland ST5 I&T Area in Building 29, room 150.

Spacecraft EMC / EMI Radiated Emissions & Radiated Susceptibility testing will occur at the GSFC, Greenbelt, Maryland EMI / EMC lab in Building 7, Large EMC room (SEA).

## ENVIRONMENTAL REQUIREMENTS

The environmental requirements for this test are as delineated in this section.

### Temperature & Humidity

The Temperature for the test facility must be between:

65 to 80 degrees Fahrenheit or 22 + 4 degrees Celsius.

The Humidity for the test facility must be between:

30% to 70%.

### Contamination Control Requirements

General cleanliness measures shall be implemented to maintain a 100K level.  Refer to the ST-5 Contamination and Control Plan (ST5-495-105).

### Electrostatic Discharge Control Requirements

Normal ESD preventative measures will be implemented in accordance with the ST-5 ESD Control Plan (ST5-495-199).

# PERSONNEL REQUIREMENTS

This section delineates the personnel required to perform this test and the protective measures to be taken around the flight hardware.

NOTE: Personnel shall be trained in flight hardware handling, connector mate/de-mate and must have a current ESD certificate.

## PERSONNEL PROTECTIVE EQUIPMENT

Protective equipment required during the execution of this test are worker ground straps, ESD approved protective lab coats/smocks and gloves.  The flight unit requires a protective container, wrapping materials, and ESD approved dust caps for each connector when delivered to the integration team.

## REQUIRED PERSONNEL

The following paragraphs delineate the personnel required to perform this test as a minimum to be:

- Spacecraft Test Conductor

- Spacecraft Electrical and Mechanical Integration Technicians

	Quality Representative

           Subsystem Engineer

### Spacecraft Test Conductor (TC)

The Spacecraft TC has the main responsibility in ensuring the integrity of the test.  The Test Conductor is the point of contact for the unit under test.  The TC is in charge of all operations required to perform this procedure.  Additional TC responsibilities are:

- To oversee the processes and methods used in the proper execution of the test.

- To perform troubleshooting during the test as required.

- To identify, report and initiate corrective action for the flight hardware, database or Ground Support Equipment (GSE).

- To disposition test anomalies.

### spacecraft  Electrical & mechanical Integration Technicians

The responsibility of the Electrical and Mechanical Integration Technicians are to support the Spacecraft TC as required in the execution of the test.

### Quality Representative

The responsibility of the Quality Representative is to be cognizant of the execution of the test and to perform test data review on a daily basis.  The Quality Assurance representative is responsible for witnessing the Integration and signoff of the procedure steps at designated points.

### SUBSYSTEM ENGINEER

The responsibilities of the Subsystem Engineers are to represent technical issues regarding operation and design of the sub-system.  The cognizant engineers shall support the TC in development and execution of this procedure.  The subsystem engineers shall consist of C&DH, Power, and Mechanical.

## SIGNOFF PERSONNEL

The following personnel are required to signoff indicating their approval of this procedure as it is written.  A subset of this group shall be required for approval of troubleshooting steps and redline changes to this procedure in the future. They shall be the Integration & Test Manager or his Deputy, the Cognizant Subsystem Engineer, the Lead TC, and the Quality representative.

## Additional Support Personnel

Additional support personnel required will consist of a mechanical integration technician for the installation and removal of flight components as required during integration activities.  Safety personnel support is not required for this test.

## PERSONNEL ASSIGNMENTS AND RESPONSIBILITY REVIEW

| NAME | FUNCTION | PHONE | PAGER |
| --- | --- | --- | --- |
| Tom Gostomski | ST5 I&T Engineering Lead | 301-286-3440 | 877-295-4976 |
| Kevin Blahut | I&T TC | 301-286-6571 |  |
| Marty Brown | I&T TC | 301-286-5703 | 877-295-2094 |
| Dave Hahn | I&T TC | 301-286-1325 | 977-295-6819 |
| Pete Young | I&T TC | 301-286-6483 |  |
| EJ Bickley | I&T TC | 301-286-6232 | 877-302-5459 |
| Pete Rossoni | Spacecraft Mech. Lead | 301-286-5854 |  |
| George Jackson | C&DH Lead | 301-286-3255 | 877-299-7750 |
| Mitch Coccia | C&DH Engineer | 301-286-4907 | 877-286-1465 |
| Karen Stewart | EPSE Lead | 301-286-2383 |  |
| Dave Raphael | C&DH Engineer | 301-286-0178 | 877-293-2774 |
| Roger Counts | QA | 301-286-6484 | 888-849-0088 |
| Joyce Manning | QA | 301-286-3239 | 888-424-9834 |
| Steve Williams | QA | 301-286-7966 | 877-311-3507 |
| Roberto Arocho | PSE Test Engineer | 301-286-1349 |  |
| Amri Hernandez-Pellerano | PSE Design Engineer | 301-286-1068 |  |
| Rene Zelaya | CGSE Engineer | 301-867-0061 |  |
| Sara Woo | CGSE Engineer | 301-286-3669 | 877-275-6152 |
| Mark Branch | EMI/EMC | 301-286-9948 |  |
| Ted Dyer | EMI/EMC | 301-286-6201 |  |
| Patrick Harris | EMI/EMC | 301-286-2526 |  |
| Amani Ginyard | Contamination Engineer | 301-286-4364 |  |
| Syed Ameen | PGSE Lead | 301-286-8886 |  |
| Sam Placanica | GN&C Lead | 301-286-8836 |  |
| Angie Russo | GN&C Hardware Lead | 301-286-2554 |  |
| Jason Badgley | Propulsion Engineer | 301-286-2593 |  |
| Mike Rhee | Propulsion Lead | 301-286-5788 |  |
| Donya Douglas | Thermal Lead | 301-286-6952 |  |
| Terry Sullivan | Electrical Systems Lead | 301-286-6720 |  |
| Evan Webb | Systems Lead | 301-286-2667 |  |
| Keith Chamberlin | Systems Engineer | 301-286-2580 | 301-207-1817 |

# HAZARDS

This procedure contains hazardous operations pertaining to the use of the spacecraft RF subsystem. Standard Laboratory practices should be followed to ensure safe operation and utilization of test equipment and setups.

# SAFETY

## SAFETY SUPPORT

Safety Support is not required for this test but should be notified before any activities.

## SAFETY PRECAUTIONS

### TEMPORARY TEST CONFIGURATIONS

When connecting temporary test interface hardware to the unit under test, always insulate bare wires and leads to preclude the accidental shorting of leads, pins, etc.

### WARNINGS DURING TEST

While performing powered operations always an amber light or equivalent will be set up and visible from anywhere in the work area to advise personnel of the Spacecraft powered condition.  The Test Conductor prior to power up of the Spacecraft shall issue a verbal warning.

The test area where flight hardware resides is a no food or drink zone and a sign must be posted indicating that the work area is an ESD work area.

Storm Code Conditions are to be monitored during the operation of this test. Operations are to be shut down whenever Storm Code conditions 3 (RED) occurs, testing shall be stopped and all systems secured. No testing is allowed while Storm Code 3 conditions are in effect.

STORM CODE CONNDITIONS:

| Number | Color | Response |
| --- | --- | --- |
| 1 | White | Alert |
| 2 | Yellow | Warning |
| 3 | Red | Action |
| 4 | Green | All Clear |
| 5 | Blue | High Winds |

If any of the following occurs,

o Storm Code Condition 3 in effect.

o Environmental Test Facility power loss.

o Environmental Test Facility evacuation.

The Spacecraft and EGSE will be powered OFF.

# ELECTROSTATIC DISCHARGE (ESD)

## Electrostatic Discharge (ESD) Prevention

This hardware contains ESD-sensitive components.  Specific ESD preventative 	measures must be invoked prior to and during I&T in order to prevent potentially serious damage to the ST-5 Spacecraft and/or its subsystems.  This calls for strict adherence to the provisions established in the ST-5 ESD Control Plan or Mission Assurance Plan, Document ST5-495-010.  Prior to beginning this I&T procedure, perform the following:

1.  Scan personnel, tools, fixtures, and other articles in the work area to verify the articles have less than a 500-volt static potential.  Eliminate any unneeded articles that exceed a 500-volt potential, or dissipate the charge found on the article. Avoid the use of non-conductive materials near the Spacecraft.  Be especially aware of any type of adhesive tape.

2.  Verify all personnel coming in contact with any ST-5 component are clothed in electrostatic dissipating bunny suits or smocks, and wearing gloves or booties that are made of an approved material.

3.  Verify all personnel coming in contact with any ST-5 component (or within 1 meter of its proximity) are wearing a grounding wrist strap connected to an approved ESD ground through a series resistance >/= 250 KOHM and < 1 MOHM located within the wrist strap ground cord.  This must be verified by use of an approved wrist strap checker prior to use.

4.  When making electrical measurements with external or hand held meters, remove probes from contact with the breakout box (BOB), Spacecraft, or component prior to changing meter scales.  At all times set the meter range prior to making contact with the meter probes to the breakout box.  Do NOT use auto ranging on any meter unless directed by the cognizant engineer.

5.  Ground meter probes simultaneously prior to and between measurements in order to drain off electrostatic charges arising from friction or meter range changes.

WARNING:  The unrolling of any tape creates a large amount of static potential.  Tape shall be unrolled slowly and discharged before adhering to surface.

## Meter Measurement Considerations

Prior to the use of meter probes, ensure the correct scale/range is selected on the meter. Ensure the electrostatic potential is discharged from the meter by grounding the probes prior to use. During the course of taking measurements, the probe leads must be simultaneously grounded after any scale/range change. Note DO NOT USE AUTO RANGING scale on any digital multimeters. Scales should be manually selected to as to carry out the display to 1/10 scale (example XX.X )

## BOB and Harness Installation Considerations

Prior to instructions to mate a BOB or harness, perform the following:

In all cases, personnel shall avoid contact with BOB connector pins or contacts.

Ensure the BOB or harness connector body is discharged to an approved ESD ground connector prior to mating of the BOB or harness connector.

Ensure the personnel performing the mating of connectors have grounded the wrist strap to an approved ESD ground.

# GSE, TOOLS AND EQUIPMENT

## EQUIPMENT CALIBRATION

All electronic measuring equipment must have a currently valid calibration and must be maintained on periodic calibration cycles.  Ensure that all equipment used during the test is listed in the equipment table found in section 9.

## GSE

### EGSE

The EGSE listed in Table 8.2.1.1 is required to perform this test.

| ITEM | SIZE & WT (LxWxH, lbs.) | POWER (V, I, Ph, NEMA ID) | PURPOSE |
| --- | --- | --- | --- |
| PGSE | 33”X221/2”651/2” | STD 115VAC Plug / NEMA 5-15 | Battery Simulator / Charger |
| FEDS Rack | 33”X22”X59” | STD 115VAC Plug / NEMA 5-15 | Front End Data System |
| ASIST Workstation | 24”X30”X24” | STD 115VAC Plug / NEMA 5-15 | Operations Ground System |
| Voice Box System | 23X23X25-1/5” | STD 115VAC Plug / NEMA 5-15 | TELEX Voice System |
| OpteleCom System | 21X21X12” | STD 115VAC Plug / NEMA 5-15 | RS422 to Fiber Converter |
| RF Rack | 30X23”x69” | TL-120VAC 20APlug / NEMA L5-20 | RF Interface to S/C |
| S/C Umbilical Rack | 36”X24”X79-1/2” | TL-120VAC 20APlug / NEMA L5-20 | Spacecraft power |
| UPS Umbilical FEDS | 21 “ x 16” x 19 “ | TL-120VAC 20APlug / NEMA L5-20 | Uninterrupted Power Source |
| UPS ASIST | 21 “ x 16” x 19 “ | TL-120VAC 20APlug / NEMA L5-20 | Uninterrupted Power Source |

| ITEM | S/N | Software/Data Base Version |
| --- | --- | --- |
| PGSE |  |  |
| FEDS Rack |  |  |
| ASIST Workstation |  |  |
| RF Rack |  |  |
| S/C Umbilical Rack |  |  |

Table 9.2.1.1

Telemetry signal assignment for the Umbilical Rack strip chart recorder is listed in Table 9.2.1.2:

| Channel | Description | UMBILICAL TLM |
| --- | --- | --- |
| CH1 | ST5-S/C # SA VDC | UMBSC#SAV |
| CH2 | ST5-S/C # SA I | UMBSC#SAI |
| CH3 | ST5-S/C # BUS VDC | UMBSC#BUSV |
| CH4 | ST5-S/C # BUS I | UMBSC#BUSI |
| CH5 | ST5-S/C # BAT VDC | UMBSC#BATV |
| CH6 | ST5-S/C # BAT I | UMBSC#BATI |
| CH7 | ST5-S/C # BAT I POL | UMBSC#BATIP |
| CH8 | ST5-S/C # BAT CELL 1 VDC | UMBSC#BATCV1 |
| CH9 | ST5-S/C # BAT CELL 5 VDC | UMBSC#BATCV5 |

Table 9.2.1.2

### EMI/EMC EGSE

The test article and any fixtures shall be installed in accordance with established EMI/EMC facility procedures. All Scope Ref. Leads will be to Chassis/Quiet Ground.

### MGSE

Three 11” legs are used to hold the Spacecraft off of the EMI facility ground plane table during this test.

Z

Y

X

ST5 SC with 11” standoffs on Top Deck

Side View

Top View

## TOOLS AND EQUIPMENT LIST

The following tools and equipment are required to perform this test.  When a specific tool, instrument or piece of test equipment is not available an equivalent or more precise instrument may be used in its place. Prior approval by the test conductor and the Subsystem Engineer is required when making a substitution.

All tools, GSE, etc., coming near or into contact with the Spacecraft shall be degaussed prior to use.  Non-magnetic tools shall be used whenever possible.

| ITEM | MAKE/MN | S/N | NEMS | CAL DUE |
| --- | --- | --- | --- | --- |

Table 9.3. 1

## COMMUNICATIONS DISCIPLINE

### Voice

Voice communication will use the telex or similar system.

## BOB INSTALLATION

Follow the steps outlined in the following prior to the connection of all BOBs to any flight articles.

1) Ensure that all the bottle plugs or jumpers are removed from all the BOBs prior to installing them.

2) Connect the ends of the BOB together and verify all pins are shorted. Failure of a BOB to meet this criterion may be due to a broken or missing wire and must be corrected prior to its use during integration. The BOB may be used as is, if the damaged wire or missing connection is not used in the harness connector that the BOB was to be mated to. Do Not use any BOBs that are reversed wired.

3) Prior to disconnecting the BOB connectors from each other, short each pin to chassis ground to dissipate any accumulated charge in the BOB harness.

4) Observe the layout of the BOB. Check what Pins on the BOB go to what side of the BOB (S/C or harness side). Mate the BOBs to the S/C Harness as shown in the following table.  Mate connectors as practical, taking into account factors such as S/C harness length and BOB availability.

# EGSE CONFIGURATION

## NORMAL SPACECRAFT EGSE TEST CONFIGURATION

The Spacecraft to EGSE test configuration is shown in Figure 10.0. This is the Spacecraft to EGSE configuration during most Spacecraft testing. It does not include any additional harnessing or GSE associated with special testing, such as the various environmental test configurations. This figure is shown for Reference only.

FIGURE 10.1 Normal Spacecraft EGSE Configuration

# PRE-ENVIRONMENTAL TEST ACTIVITIES / VERIFICATIONS

## HARDWARE S/N RECORDING

Record the Spacecraft and Spacecraft component S/N’s below.

| COMPONENT | S/N |
| --- | --- |
| Spacecraft |  |
| CCA |  |
| Magnetometer Electronics |  |
| Magnetometer Sensor Head |  |
| Transponder |  |
| HPA |  |
| VEC 1 |  |
| VEC 2 |  |
| Nutation Damper |  |
| TCE |  |
| CGMT |  |
| PTE |  |
| BATTERY |  |
| MSSS |  |

Verify __________

## HARDWARE GSE GROUND VERIFICATION

Ensure all EGSE are ground (</= 1.0 ohm) from chassis to Quiet Ground.

Verify __________

Ensure the Spacecraft structure is tied via a ground strap to Quiet Ground (</= 1.0 ohm).

Verify __________

# TEST SEQUENCE

## CONFIGURE GSE AND SPACECRAFT

### Configure Spacecraft EGSE NORMAL Setup per Figure 12.1.1

### Safe To Mate

Verify that the Umbilical Console Integration Verification procedure was performed per ST5-495-305 (S/C#1), ST5-495-414 (S/C#2) or ST5-495-415 (S/C#3).

Verify __________

### Verify the Configuration of Spacecraft EGSE Setup per Figure 12.1.1

I&T Verification _______, Systems Verification _______

Verify __________

### Spacecraft Pre-EMI Functional Test

Perform the Spacecraft Functional Test per ST5-495-TBD09 at High 8.4V Bus Voltage. Using the Battery Simulator SOC100  (High 8.4V Bus Voltage) when asked during power on.

Verify __________

## CONDUCTED EMMISSIONS & COMMON MODE NOISE TESTS

NOTE: Limits are to be enabled and the TC will Monitor the ST5UMB# (S/C#) page on ASIST.

### Configure Spacecraft EGSE for CE01 Test (7.2V Bus) per Figure 11.2.1

FIGURE 12.2.1

CE01, CE03 (Conducted Narrowband Emissions Limits), & CMN 5V & 7.2V Bus Setup

Verify __________

### Verify that BUS Cable adaptor (GD 2059236-1 P2) is connected to Flight Harness P22 (GD 2054900).	Record in the Mate/De-Mate Log if Connector Saver is not used.

Verify __________

### Verify that BUS Cable adaptor (GD2059236-1) pigtail Lugs are connected to the Oscilloscopes as follows.

P1-13 to P2-13 (7V (+))

P1-43 to P2-43 (7V (-))

P1-18 to P2-18 (5V (+))

P1-3 to P2-3 (5V (-))

Verify __________

### Connect an Unpopulated 44 pin BOB to BUS Cable adaptor (GD 2059236-1) P1 looking into the Flight Harness. (Ref. 8.5)

Verify __________

### Measure the Resistance P22 looking into the Flight Harness side of the BOB between the following pins: (Ref. 8.5)

| C&DH J22 Continuity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| From (+) | Signal Name | To (-) | Signal Name | Expected | Measured |
| 13 | MBOOM_PWR_A | 14 | MBOOM_PWR_B | <1 |  |
| 43 | MBOOM_PWR_RTN_A | 44 | MBOOM_PWR_RTN_B | <1 |  |
| 18 | VEC1_PWR_A | 19 | VEC1_PWR_B | <1 |  |
| 03 | VEC1_PWR_RTN_A | 33 | VEC1_PWR_RTN_B | <1 |  |
| 43 | MBOOM_PWR_RTN_A |  | Backshell | >1M |  |
| 03 | VEC1_PWR_RTN_A |  | Backshell | >1M |  |

| C&DH J-22 Isolation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| From (+) | Signal Name | To (-) | Signal Name | Expected | Measured |
| 13 | MBOOM_PWR_A |  | chassis | >1M |  |
| 18 | VEC1_PWR_A |  | chassis | >1M |  |

Verify __________

### Remove the BOB from BUS Cable adaptor GD 2059236-1 P1. (Ref. 8.5)

Verify __________

### Connect BUS Cable adaptor GD 2059236 P1 to C&DH J22

Record in the Mate/De-Mate Log if Connector Saver is not used.

Verify __________

### Verify EMI/EMC Test Equipment is isolated from AC ground.

Verify with the EMC personnel that all EMI/EMC Test Equipment used for these tests is isolated from AC ground.

Verify __________

### Ambient Room Scan

Inform the EMI personnel that the EGSE is OFF for the Ambient Scan.

Verify __________

### Ambient Room “EGSE ON” Scan

Power on all of the GSE and Inform the EMI personnel that the EGSE is ready for the Ambient Scan.

Verify __________

### Power ON the Spacecraft

Execute the STOL procedure st5intgpwron.prc at the ASIST Primary workstation.

Verify __________

#### Record the Battery Voltage and Current.	 (Page ST5POWERSTAT)

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Configure the Spacecraft for Worst Case Emissions Mode

| S/C Configuration per Figure 12.2.1 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | On Umbilical rack ensure switches configuration at power on: C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | Not connected |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 Not installed | ETU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats (2) installed |  |
| RF GSE to Antenna Hats (2) |  |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom not installed Mag sensor head integrated | Mag Boom not available |
| MSSS Stim installed |  |
| Mag Stim installed |  |
| Battery Enable Plug (#1) (DA-P05) to S/C DA-J05 installed | Battery being used |

TABLE 12.2.1a. Conducted Emissions Hardware Configuration (CE01, CE03, & CMN)

| Subsystem | Worst Case Emissions Mode | Conducted Science Data Collection Mode |
| --- | --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery at +8.4 V (SOC 100%) SAS ON 1st Run: PWM ON: (Duty Cycle 100%) (HPA OFF) 2nd Run: PWM OFF: (HPA ON) Battery Power used | Battery at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) Battery Power used |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in Lo Field/Hi Sensitivity | ON in Lo Field/Hi Sensitivity |
| MSSS | ON, +8.4V bus | ON |
| PTE | ON | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse | OFF |
| Transponder | Receiver ON Uplink Established | Receiver ON Uplink N/A |
| HPA | 1st Run: OFF (PWM ON) 2nd Run: ON (PWM OFF): transmitting | OFF |
| VEC #1 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘11’ (Heater ON) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| VEC #2 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘10’ (Heater OFF) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| Mag Boom | Deployed (ON) | Deployed (ON) |

TABLE 12.2.1.b Conducted Emissions Configuration (CE01, CE03, & CMN)

Execute the STOL procedure st5_emi_cfg (s/c#,“CE”) at the ASIST Primary workstation and verify the above configuration.

Where s/c# = spacecraft id at umbilical

Verify __________

### Verify that the HPA is OFF.

Verify __________

### Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (+) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE01 (+) Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current CE01 (+) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE01 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE01(-) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus

Inform the EMI personnel that the Spacecraft is configured for the CE CMN Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE CMN test is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Turn ON the HPA

Verify __________

### Repeat with HPA ON +5V & +7.2V Bus (+) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE01 +7.2V Bus (+) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

### Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE01 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE01(+) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE01 (Conducted Narrowband Emissions Limits) +5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE01 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE01(-) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus

Inform the EMI personnel that the Spacecraft is configured for the CE CMN Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE CMN test is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Turn OFF the HPA

Verify __________

### CE01 Test Results

Record results 	Met ________		Exceeded __________

If the results exceeded the profile in the EMI Plan, reconfigure the Spacecraft as necessary.

Verify __________

### CMN Test Results

Record results 	Met ________		Exceeded __________

If the results exceeded the profile in the EMI Plan, reconfigure the Spacecraft as necessary.

Verify __________

### Recharge Battery if voltage is <= 7.5V.

If the Battery voltage is <=7.5 V recharge the Battery prior to continuing to the next step.

Verify __________

### Record the Battery Voltage and Current.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE03 +5V & +7.2 V Bus (+) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE03 Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current CE03 (+) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE03 +5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE03 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE03(-) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus

Inform the EMI personnel that the Spacecraft is configured for the CE CMN Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE CMN test is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Turn ON the HPA

Verify __________

### Repeat with HPA ON +5V & +7.2V Bus (+) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE03 +7.2V Bus (+) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

### Perform CE03+5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE03 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE03(-) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE03 +5V & +7.2 V Bus (-) SIDE

Inform the EMI personnel that the Spacecraft is configured for the CE03 (-) side Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE03(-) side is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Perform CE CMN (Common Mode Noise Test) 5V & 7.2V Bus

Inform the EMI personnel that the Spacecraft is configured for the CE CMN Test.

“Monitor the Battery Voltage and Current during the test”.

Verify __________

#### Record the Battery Voltage and Current when CE CMN test is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Turn OFF the HPA

Verify __________

### CE01 Test Results

Record results 	Met ________		Exceeded __________

If the results exceeded the profile in the EMI Plan, reconfigure the Spacecraft as necessary.

Verify __________

### CMN Test Results

Record results 	Met ________		Exceeded __________

If the results exceeded the profile in the EMI Plan, reconfigure the Spacecraft as necessary.

Verify __________

### Recharge Battery if voltage is <= 7.5V.

If the Battery voltage is <=7.5 V recharge the Battery prior to continuing to the next step.

Verify __________

### Record the Battery Voltage and Current.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Power OFF the Spacecraft

Execute the STOL procedure st5intgpwroff at the ASIST Primary workstation.

Verify __________

## Self Compatibility

NOTE: This test requires the use of the PGSE Battery Simulator.

### Reconfigure the Spacecraft test setup Self Compatibility test per Tables & Figure 12.3.1

| Configure S/C per Figure 12.4.1 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | S/C Enable/Safe Plug (#2) On Umbilical rack ensure switches configuration at power on: C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | PGSE to S/C |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 installed | ETU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats (2) installed |  |
| RF GSE to Antenna Hats (2) |  |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom not installed Mag sensor head integrated | Mag Boom not available |
| MSSS Stim installed |  |
| Mag Stim installed |  |
| Batt Sim connected to DA-J05 | Battery Simulator used |

TABLE 12.3.1.a Self Compatibility Hardware Configuration (Self Compatibility)

| Subsystem | Most Susceptible Mode | Conducted Science Data Collection Mode |
| --- | --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +6.5 V (SOC 12.5%) SAS ON PWM ON (Duty Cycle <100%) | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) |
| GN&C | Science Mode (See Mag, MSSS, PTE, and TCE for details) | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in Lo Field/Hi Sensitivity | ON in Lo Field/Hi Sensitivity |
| MSSS | ON, +6.5V bus | ON |
| PTE | ON | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse | OFF |
| Transponder | Receiver ON Uplink Established | Receiver ON Uplink N/A |
| HPA | ON: transmitting. | OFF |
| VEC #1 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘11’ (Heater ON) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| VEC #2 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘10’ (Heater OFF) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| Mag Boom | Deployed (ON) | Deployed (ON) |

TABLE 12.3.1.b Self Compatibility Configuration (Self Compatibility)

FIGURE 12.4.1 Spacecraft Self Compatibility, Setup (Self Compatibility)

Verify __________

#### Verify that BUS Cable adaptor (GD2059236) is installed between PSE J22 and Flight Harness P12.

Verify __________

### Verify the Configuration of Spacecraft EGSE Setup per Figure 9.4.1

I&T Verification _______, Systems Verification _______

Verify __________

### Power ON the Spacecraft by execution of the STOL procedure st5intgpwron at the ASIST Primary workstation.

When configuring the PGSE, set the  BSOC to 12.5% for Battery at 6.5V.

Verify __________

### Configure the Spacecraft for Most Susceptible Mode

Execute the STOL procedure st5_emi_cfg(s/c#,”SC”) at the ASIST Primary workstation.

Where s/c# = spacecraft id at umbilical

Verify __________

### Verify the Spacecraft Thruster is in Delta-V mode.

Verify __________

### Perform Self Compatibility test

Inform the EMI personnel that the Spacecraft is ready for the CS01 Test.

Verify __________

### Power OFF the Spacecraft

Execute the STOL procedure st5intgpwroff at the ASIST Primary workstation.

Verify __________

## RADIATED EMISSIONS

NOTE: Limits are to be enabled and the TC will Monitor the ST5UMB# (S/C#) page on ASIST.

### Reconfigure for Radiated Emissions (RE02) according to Tables & Figure 12.5.1

| Configure S/C per Section 12.5.1 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | On Umbilical rack ensure switches configuration at power on: C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | Not connected |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 installed | ETU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats (2) removed |  |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom Deployed | Use Mag Boom Deployment Procedure |
| MSSS Stim not installed. MSSS Dust Covers installed. |  |
| Mag Stim not installed |  |
| Battery Enable Plug (#1) (DA-P05) to S/C DA-J05 installed | Battery being used |
| Position GSE antenna at location identified in Step X, 1 m from S/C. | Performed by EMI/EMC Group |

TABLE 12.5.1.a Radiated Emissions Hardware Configuration (RE02)

| Subsystem | Worst Case Emissions Mode | Radiated Science Data Collection Mode |
| --- | --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery at +8.4 V (SOC 100%) SAS ON 1st Run: PWM ON: (Duty Cycle 100%) (HPA OFF) 2nd Run: PWM OFF: (HPA ON) Battery Power used | Battery at +8.4 V (SOC 100%) SAS ON PWM ON (Duty cycle 100%) |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in Full Field/Lo Sensitivity | ON in Full Field/Lo Sensitivity |
| MSSS | ON, +8.4V bus | ON |
| PTE | ON | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse | OFF |
| Transponder | Receiver ON Uplink Established | Receiver ON Uplink N/A |
| HPA | 1stON, Not transmitting | OFF |
| VEC #1 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘11’ (Heater ON) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| VEC #2 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘10’ (Heater OFF) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| Mag Boom | Deployed (ON) | OFF |

TABLE 12.5.1.b Radiated Emissions Configuration (RE02)

FIGURE 12.5.1 Radiated Emissions Test Setup (RE02)

Verify __________

#### Verify that Flight Harness P/J-22 has been reconnected.

Record in the Mate/De-Mate Log if Connector Saver is not used.

Verify __________

### Verify the Configuration of Spacecraft EGSE Setup per Figure 9.4.1

PSE Verification _______, C&DH Verification _______, Code 549 Verification ________,

Electrical Systems Verification _________, 	QA Verification _________

Verify __________

### Power ON the Spacecraft by execution of the STOL procedure

Execute st5obspwron.PRC at the ASIST Primary workstation.

Turn on the Strip Chart Recorder set at 5mm/m when prompted by procedure.

Verify that the Red & Yellow Limits checking is on.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

### Configure the Spacecraft for Worst Case Emissions Mode

by execution of the STOL procedure TBD_WORST_EM.PRC at the ASIST Primary workstation.

Verify __________

### Perform RE02 (Radiated Narrowband Electric Field)

Inform the EMI personnel that the Spacecraft is ready for the RE02 Test.

“Monitor the ST5UMB# (S/C#) page during the test”.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack when RE02 is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Power OFF the Spacecraft

Execute the STOL procedure st5obspwroff.PRC at the ASIST Primary workstation. Also Turn off the Strip Chart Recorder.

Verify __________

### Recharge Battery if voltage is <= 6.5V.

If the Battery voltage is <=6.5 V recharge the Battery prior to continuing to the next step.

Verify __________

### Record (RE02) Noise Locations

Mark the nosiest locations on the following Figure 11.6.16.

Record Distance and Height of antenna from Spacecraft.

Verify __________

### Reconfigure for Radiated Emissions RE04 Test according to setup per Table & Figure 12.5.9

Note: No wiring configuration change is required for this test, EMI test setup only.

| Configure S/C per Section 6.5.4 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | On Umbilical rack ensure switches configuration at power on:C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | Not connected |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 installed | ETU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats (2) removed |  |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom Deployed | Use Mag Boom Deployment Procedure |
| MSSS Stim not installed. MSSS Dust Covers installed. |  |
| Mag Stim not installed |  |
| Battery Enable Plug (#1) (DA-P05) to S/C DA-J05 installed | Battery being used |
| Position GSE antenna at location identified in Step X, 1 m from S/C. | Performed by EMI/EMC Group |

Table 12.5.9.a Radiated AC Magnetic Field Emissions Spacecraft Hardware Configuration (RE04)

| Subsystem | Maximum Magnetic Emissions Mode | Radiated Science Data Collection Mode |
| --- | --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) Battery Power used | Battery at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) Battery Power used |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in Full Field/Lo Sensitivity | ON in Full Field/Lo Sensitivity |
| MSSS | ON,+8.4V bus | ON |
| PTE | ON | ON |
| TCE | ON 2 Hz rate, 50 msec firing pulse | OFF |
| Transponder | Receiver ON Uplink Established | Receiver ON Uplink N/A |
| HPA | ON, Not transmitting | OFF |
| VEC #1 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘11’ (Heater ON) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| VEC #2 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘10’ (Heater OFF) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| Mag Boom | Deployed (ON) | OFF |

Table 12.5.9.b Radiated AC Magnetic Field Emissions Spacecraft Configuration (RE04)

FIGURE 12.5.9 Radiated Emissions Test Setup (RE04)

Verify __________

### Power ON the Spacecraft by execution of the STOL procedure

St5obspwron.PRC at the ASIST Primary workstation. Using the Battery when asked in the procedure. Turn on the Strip Chart Recorder set at 5mm/m when prompted by procedure.

Verify that the Red & Yellow Limits checking is on.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Configure the Spacecraft for Maximum Magnetic Emissions Mode

Execute the STOL procedure TBD_WORST_EM.PRC at the ASIST Primary workstation.

Verify __________

### Perform RE04 (Radiated AC Magnetic Field Emissions –Maximum Magnetic Emissions Mode)

Inform the EMI personnel that the Spacecraft is ready for the RE04 Test.

“Monitor the ST5UMB# (S/C#) page during the test”.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack when RE04 is completed.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Configure the Spacecraft for Maximum Magnetic Emissions Mode.

Execute the STOL procedure TBD_SCI_COL.PRC at the ASIST Primary workstation.

Verify __________

### Perform RE04 (Radiated AC Magnetic Field Emissions –Science Data Collection Mode)

Inform the EMI personnel that the Spacecraft is ready for the RE04 Test.

“Monitor the ST5UMB# (S/C#) page during the test”.

Verify __________

### Power OFF the Spacecraft

Execute the STOL procedure st5obspwroff.PRC at the ASIST Primary workstation. Also Turn off the Strip Chart Recorder.

Verify __________

### Recharge Battery if voltage is <= 6.5V.

If the Battery voltage is <=6.5 V recharge the Battery prior to continuing to the next step.

Verify __________

### Record (RE04) Noise Locations

Mark the nosiest locations on the following Figure 11.6.15.

Record Distance and Height of antenna from Spacecraft.

RE02 (Worst Case Emissions Mode):

Location #1	Distance __________		Height __________

Location #2	Distance __________		Height __________

Location #3	Distance __________		Height __________

RE04 (Most Susceptible Mode):

Location #1	Distance __________		Height __________

Location #2	Distance __________		Height __________

Location #3	Distance __________		Height __________

Z

Y

X

ST5 SC with 11” standoffs on Bottom Deck

Verify __________

### Radiated Emissions (RE04) Setup Photos.

Take Photos of the antenna setups with relation to Spacecraft.

Verify __________

## RADIATED SUSCEPTIBILITY

NOTE: Limits are to be enabled and the TC will Monitor the ST5UMB# (S/C#)  page on ASIST.

### Reconfigure for Radiated Susceptibility RS03 according to Tables & Figure 12.6.1

Note: No wiring configuration change is required for this test, EMI test setup only.

| Configure S/C per Section 12.6.1 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | On Umbilical rack ensure switches configuration at power on: C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | Not connected |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 not installed | ETU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats (2) removed |  |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom Deployed | Use Mag Boom Deployment Procedure |
| MSSS Stim not installed. MSSS Dust Covers installed. |  |
| Mag Stim not installed |  |
| Batt Sim connected to DA-J05 | Battery Simulator used |
| Locate GSE pick-up antenna | Antenna provided by and activity performed by EMI/EMC Group |
| Locate ST5 RF GSE to pick-up antenna (2) | Antenna provided by EMI/EMC Group |

TABLE 12.6.1.a	 Spacecraft Radiated Susceptibly Hardware Configuration (RS03)

| Subsystem | Most Susceptible Mode | Radiated Science Data Collection Mode |
| --- | --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +6.5 V (SOC 11.6%) SAS ON PWM ON (Duty Cycle <100%) | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in Full Field/Lo Sensitivity | ON in Full Field/Lo Sensitivity |
| MSSS | ON, +6.5V bus | ON |
| PTE | ON | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse | OFF |
| Transponder | Receiver ON Uplink Established | Receiver ON Uplink N/A |
| HPA | ON, transmitting | OFF |
| VEC #1 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘11’ (Heater ON) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| VEC #2 | S/C 1 ON in Mode ‘10’ (Heater OFF) S/C 2 ON in Mode ‘10’ (Heater OFF) S/C 3 ON in Mode ‘11’ (Heater ON) | S/C 1 No VECs S/C 2 VEC 1 only S/C 3 Both VEC 1&2 |
| Mag Boom | Deployed (ON) | OFF |

TABLE 12.6.1.b Spacecraft Radiated Susceptibly Configuration (RS03)

FIGURE 12.6.1 Radiated Susceptibility Electric Field (RS03) Setup

Verify __________

### Verify the Configuration of Spacecraft EGSE Setup per Figure 12.6.1

PSE Verification _______, C&DH Verification _______, Code 549 Verification ________,

Electrical Systems Verification _________, 	QA Verification _________

Verify __________

### Power ON the Spacecraft by execution of the STOL procedure

St5obspwron.PRC at the ASIST Primary workstation.

Turn on the Strip Chart Recorder set at 5mm/m when prompted by procedure.

Verify that the Red & Yellow Limits checking is on.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Configure the Spacecraft for Most Susceptible Mode.

Execute the STOL procedure TBD_MOST_SUS.PRC at the ASIST Primary workstation.

Verify __________

### Perform RS03 (Radiated Susceptibility Electric Field)

Inform the EMI personnel that the Spacecraft is ready for the RS03 Test.

“Monitor the ST5UMB# (S/C#) page during the test”.

Verify __________

### RS03 Test Results

Record results 	Met ________		Exceeded __________

If the results exceeded the profile in the EMI Plan, reconfigure the Spacecraft using TBD_RAD_SCI_ COL.PRC and repeat RS03

### Power OFF the Spacecraft

Execute the STOL procedure st5obspwron.PRC at the ASIST Primary workstation. Also Turn off the Strip Chart Recorder.

Verify __________

### Recharge Battery if voltage is <= 6.5V.

If the Battery voltage is <=6.5 V recharge the Battery prior to continuing to the next step.

Verify __________

### Configure Spacecraft EGSE NORMAL Setup per Figure 9.0.1

PSE Verification _______, C&DH Verification _______, Code 549 Verification ________,

Electrical Systems Verification _________, 	QA Verification _________

Verify __________

### Spacecraft Aliveness Test

Perform the Spacecraft Aliveness Test per ST5-495-TBD07 at High 8.4V Bus Voltage. Using the Battery Simulator SOC100  (High 8.4V Bus Voltage) when asked during power on.

Turn on the Strip Chart Recorder set at 5mm/m when prompted by procedure.

Power OFF when asked at end of functional test. Also Turn off the Strip Chart Recorder.

Verify __________

## RF Self Compatibility

NOTE: Limits are to be enabled and the TC will Monitor the ST5UMB# (S/C#)  page on ASIST.

### Reconfigure for RF Self Compatibility according to Tables & Figure 12.7.1

| Configure S/C per Figure 12.7.1 | NOTES |
| --- | --- |
| Umbilical DA-P06 to S/C DA-J06 installed (In lieu of S/C Enable / Safe Plug #2) | S/C Enable/Safe Plug (#2) On Umbilical rack ensure switches configuration at power on: C&DH Sep A & Sep B “OPEN” PSE Sep A & Sep B “CLOSED” |
| Umbilical DA-P02A to S/C DA-J02A | Umbilical C&DH to S/C |
| Umbilical DA-P01A to S/C DA-J01A | Umbilical Power to S/C |
| PGSE DA-P01B to S/C DA-J01B | PGSE to S/C |
| CGMT Enable Plug (#4) (DA-P04) to S/C DA-J04 installed | EDU thruster being used in lieu of TCE GSE |
| PTE Enable Plug (#3) (GAP5-DA-P53) to S/C GAP5-DA-J53 installed | Tank Pressure and Temp being read by C&DH |
| Antenna Hats Removed |  |
| RF GSE to Antenna | EMI branch to supply |
| Limit Switch (GAP4-P42-LSW-PSE) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Fly Away (GAP4-P43-SEP) removed | Umbilical DA-P06 to S/C DA-J06 used instead |
| Mag Boom Deployed | Use Mag Boom Deployment Procedure Mag sensor head installed |
| MSSS Stim not installed |  |
| Mag Stim not installed |  |
| Batt Sim connected to DA-J05 | Battery Simulator used |

TABLE 12.7.1.a RF Self Compatibility Hardware Configuration (RF Self Compatibility)

| Subsystem | RF Self Compatibility (minimal operation) |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON (send NOOP every 5 seconds) 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle 100%) |
| GN&C | STBY Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | OFF |
| MSSS | ON, +8.4V bus |
| PTE | ON |
| TCE | OFF |
| Transponder | Receiver ON Uplink N/A |
| HPA | OFF |
| VEC #1 | OFF |
| VEC #2 | OFF |
| Mag Boom | Deployed (ON) |

TABLE 12.7.1.b RF Self Compatibility Configuration (RF Self Compatibility)

FIGURE 12.7.1 Spacecraft RF Self Compatibility Test Configuration

### Power ON the Spacecraft

Execute the STOL procedure st5obspwron.prc at the ASIST Primary workstation.

Turn on the Strip Chart Recorder set at 5mm/m when asked by procedure.

Verify __________

#### Record the Battery Voltage and Current from the Umbilical rack.

Bat Voltage _______V (6.0-8.4) 	 Bat Current _________A (>-3.0-+3.0)

Verify __________

### Turn ON the HPA

Verify __________

RF Steps TBD

### Turn OFF the HPA

Verify __________

### Power OFF the Spacecraft

Execute the STOL procedure st5obspwroff.PRC at the ASIST Primary workstation. Also Turn off the Strip Chart Recorder.

Verify __________

## POST EMI FUNCTIONAL TEST

### Spacecraft Post EMI Functional Test

Perform the Spacecraft Functional Test per ST5-495-TBD09 at High 8.4V Bus Voltage. Using the Battery Simulator SOC100  (High 8.4V Bus Voltage) when asked during power on.

Turn on the Strip Chart Recorder set at 5mm/m when prompted by procedure.

Power OFF when asked at end of functional test. Also Turn off the Strip Chart Recorder.

Verify __________

# TEST COMPLETION

## Problem Record Disposition

Has a disposition been made for all known problems that have occurred?

YES __________			No __________

Record the W.O.A. number and PR number of any open problem records in the space below:

W.O.A. # __________

| PR No. | PR Description |
| --- | --- |

## Test Completion Report/ WOA Closeout

Provide a test summary for submittal to the Project Office.

Q.A.Verify __________

# TEST SEQUENCE / SUMMARY

The test sequence / summary is shown in Table 13.0.1.

| Test # | Test | Specifications |
| --- | --- | --- |
| 1 | PRE EMI FUNCTIONAL |  |
| 2 | SPACECRAFT ALIVENESS |  |
| 3 | CE01 | 114dBuA@30Hz, 114dBuA@300Hz, 44dBuA@20kHz, 20dBuA@2.5MHz, 20dBuA@50MHz |
| 4 | CE03 | 114dBuA@30Hz, 114dBuA@300Hz, 44dBuA@20kHz, 20dBuA@2.5MHz, 20dBuA@50MHz |
| 5 | CE CMN | 34dBuA@30Hz - 50Hz |
| 7 | SPACECRAFT ALIVENESS |  |
| 8 | Self Compatibility | 0.5Vpp @ 30Hz – 50 MHz on 7.2V bus |
| 9 | SPACECRAFT ALIVENESS |  |
| 10 | RE02 | 35 dBuV/m @ 18 kHz, decreasing 5 dB per decade 20 dBuV/m @ 30 MHz, decreasing 18 dB per decade 16 dBuV/m @ 7 GHz – 7.4GHz 58 dBuV/m @ 7.4 GHz, increasing 18dB per decade 65 dBuV/m @ 18 GHz |
| 11 | RE04 (worst case em.) | 43 dBpT @ 10 kHz – 50 kHz |
| 15 | RE04 (sci. data col.) | 43 dBpT @ 10 kHz – 50 kHz |
| 16 | RS03 | 2 V/m @ 1 MHz – 2.2 GHz 45 V/m @ 2.2 GHz – 2.5 GHz, LV S-band comm. TX to TDRSS 5 V/m @ 2.5 GHz – 7.0 GHz (Transponder range 7.0 –7.4GHz) 5 V/m @ 7.4 GHz – 8.45 GHz 33 V/m @ 8.450 GHz – 8.500 GHz, ST5 TX self-compatibility 5 V/m @ 8.5 GHz – 12 GHz 10 V/m @ 12 GHz – 18 GHz |
| 17 | RF Self Compatibility |  |
| 18 | POST EMI FUNCTIONAL |  |

Table 13.0.1

Spacecraft EMI Test Sequence/Summary

# TEST SCRIPT SUMMARY

## TBD_WORST_EM.PRC

The Spacecraft will be configured per Table 14.1

| Subsystem | Worst Case Emissions Mode |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle <100%) Battery Power used |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in HI Gain/Sensitivity |
| MSSS | ON in TBS o off sun, +8.4V bus |
| PTE | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse |
| Transponder | Receiver ON Uplink Established |
| HPA | Transmitter OFF |
| VEC #1 | ON in Mode ‘11’ (Heater ON) |
| VEC #2 | ON in Mode ’11’ (Heater ON) |
| Mag Boom | Deployed (ON) |

Table 14.1

## TBD_MOST_SUS.PRC

The Spacecraft will be configured per Table 14.2

| Subsystem | Most Susceptible Mode |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +6.5 V (SOC 12.5%) SAS ON PWM ON (Duty Cycle =100%) BattSim used |
| GN&C | Science Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in HI Gain/Sensitivity |
| MSSS | ON in TBSo off sun, +6.5V bus |
| PTE | ON |
| TCE | ON at 2 Hz rate, 50 msec firing pulse |
| Transponder | Receiver ON Uplink Established |
| HPA | Transmitter ON |
| VEC #1 | ON in Mode ‘11’ (Heater ON) |
| VEC #2 | ON in Mode ‘11’ (Heater ON) |

- Table 14.2

## TBD_RAD_SCI_ COL.PRC

The Spacecraft will be configured per Table 14.3

| Subsystem | Radiated Science Data Collection Mode |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle <100%) Battery Power used |
| GN&C | Science Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in HI Gain/Sensitivity |
| MSSS | ON in TBSo off sun |
| PTE | ON |
| TCE | OFF |
| Transponder | Receiver ON Uplink N/A |
| HPA | Transmitter OFF |
| VEC #1 | OFF |
| VEC #2 | OFF |
| Mag Boom | Deployed (OFF) |

Table 14.3

## TBD_CON_SCI_ COL.PRC

The Spacecraft will be configured per Table 14.4

| Subsystem | Conducted Science Data Collection Mode |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle <100%) Battery Power used |
| GN&C | Science Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in HI Gain/Sensitivity |
| MSSS | ON in TBSo off sun |
| PTE | ON |
| TCE | OFF |
| Transponder | Receiver ON Uplink N/A |
| HPA | Transmitter OFF |
| VEC #1 | ON |
| VEC #2 | OFF |
| Mag Boom | Deployed (On) |

Table 14.4

## TBD_MAX_MAG.PRC

The Spacecraft will be configured per Table 14.5

| Subsystem | Maximum Magnetic Emissions Mode |
| --- | --- |
| C&DH | CULPRiT ON Continuous Playback ON Analog Data Collection ON Uplink ON 100 kbps ½ Rate Conv. Encoding ON R-S Encoding ON PRN Generator ON |
| Power | Battery Sim. at +8.4 V (SOC 100%) SAS ON PWM ON (Duty Cycle <100%) Battery Power used |
| GN&C | Delta V Mode (See Mag, MSSS, PTE, and TCE for details) |
| Magnetometer | ON in HI Gain/Sensitivity |
| MSSS | ON in TBS o off sun |
| PTE | ON |
| TCE |  |
| Transponder | Receiver ON Uplink Established |
| HPA | Transmitter ON |
| VEC #1 | ON in Mode ‘11’ (Heater ON) |
| VEC #2 | ON in Mode ‘11’ (Heater ON) |

Table 14.5

## TBD_HPA_LOOP.PRC

The Spacecraft will be configured per Table 14.6

| Subsystem | HPA Switching |
| --- | --- |
| HPA | ON 2sec. OFF 2sec. For 2 Minutes |

APPENDIX A

ABBREVIATIONS

ASIST		The Advanced Spacecraft Integration and Systems Test Workstation

BOB		Break Out Box

CCB 		Configuration Control Board

CMD		Command

DMM 		Digital Multimeter

ESD 		Electrostatic Discharge

GSE		Ground Support Equipment

FEDS 		Front End Data System

I&T 		Integration and Test

ICD 		Interface Control Document

J 		designates connector on flight component

k		kilo

msec		Millisecond

M 		Mega-

m		milli-

P 		designates connector on flight harness

PR.		Problem Record

QA		Quality Assurance

Rtn		return

S/C		spacecraft

SDS		Spacecraft Data System

SGSE 	Spacecraft Ground Support Equipment

S/N		serial number

TBD	 	To Be Determined, (e.g. Component to be tested)

TBR	 	To Be Reviewed

TBS	 	To Be Supplied

TBV	 	To Be Verified

TLM 		Telemetry

APPENDIX B

ST5 Emergency Shutdown Procedure

- Start STOL Procedure ST5EOFF.

IF REQUIRED TO EVACUATE, THEN:

- On the front of Umbilical panel Manually Turn OFF the key switch to the ‘UMBILICAL INTERFACE UNIT’.

2.	If using BATSIM, select the BATT ON/OFF in LabView to terminate the ability to apply Battery Simulator.

APPENDIX C

ST5 Spacecraft EMI Test Flow

Spacecraft Level EMI/EMC Test Flow

Common Mode Noise

(CE CMN)

I&T Area

Functional Test

Self Compatibility

I&T Area

Conducted Emissions

(CE01/CE03)

I&T Area

Functional Test

Ambient Room

Scan

GSE On

Scan

ST5

I&T

EMI/EMC

Facility

Radiated Susceptibility

(RS03)

EMI Lab

Functional Test

Radiated Emissions

(RE02, RE04)

EMI Lab

Functional Test

Ambient Room

Scan

GSE On

Scan

Safe to Mate

RF Self Compatibility

APPENDIX D	Spacecraft EMI/EMC Data Sheet (Self-Compatibility)

WOA# ______________ DATE: _________  	S/C #  __________   	TC: __________

Self-Compatibility							QA: __________

| TEST | TIME ON UTC | TIME OFF UTC | Voltage Ripple 7.2V Bus | Voltage Ripple 5V Bus | RESULTS / COMMENTS |
| --- | --- | --- | --- | --- | --- |
| SAS |  |  |  |  |  |
| CULPRiT |  |  |  |  |  |
| MSS |  |  |  |  |  |
| PTE |  |  |  |  |  |
| TCE |  |  |  |  |  |
| Transponder |  |  |  |  |  |
| Mag Boom |  |  |  |  |  |
| Continuous Playback |  |  |  |  |  |
| Analog Data Collection |  |  |  |  |  |
| VEC 1 |  |  |  |  |  |
| VEC 2 |  |  |  |  |  |
| HPA |  |  |  |  |  |

APPENDIX E	Spacecraft EMI/EMC Data Sheet (Radiated Susceptibility)

WOA# ______________ DATE: _________  S/C #  _____________   	TC: __________

Radiated Susceptibility  (RS03)	 				QA: __________

| RS03 | START TIME UTC | STOP TIME UTC | START FREQ | STOP FREQ | COMMENTS |
| --- | --- | --- | --- | --- | --- |