QUALIFICATION TEST PROCEDURE

FAST-PROC-005

REVISION C

Used for

FAST MUE CUSTOM TRANSFORMER

and

INDUCTOR ASSEMBLIES

Prepared by

MARTIN MARIETTA SERVICES, INC.

9701 PHILADELPHIA WAY, SUITE E

LANHAM, MARYLAND 20706

July 30, 2026

Revision History

| Symbol | Release Date | Description | Approval |
| --- | --- | --- | --- |
| ---- | 8-12-93 | Qualification Test Procedure |  |
| A | 11-23-93 | Deleted Seal Test, para. 5.2. Updated Data Forms 1, 4-9. | T.D.G. |
| B | 01-12-94 | Incorporated per CCR No. FAST-274 | T.D.G. |
| C | 06-14-94 | Incorporated per CCR No. FAST-323 |  |

Approval Signatures

Prepared by: Thomas  MacDonald                                                        Date: 10-04-93

Thomas MacDonald

Senior Engineer, Martin Marietta Services, Inc.

Approved by: Willie O. Jones                                                               Date: 10-07-93

Martin Marietta Services Quality Assurance

Approved by: Timothy D. Gruner                                                         Date: 10-08-93

Timothy Gruner

FAST MUE Lead Engineer, Goddard Space Flight Center, Code 745.2

Approved by: Steven C. Manning                                                         Date: 10-08-93

Ron Kolecki

SMEX Quality Assurance, Goddard Space Flight Center, Code 303.0

Contents

1.0 Objective	  1

2.0 Scope	  1

3.0 Applicable Documents	  1

4.0 General Requirements	  1

4.1 Test Data 	  1

4.2 Test and Inspection Log	  1

4.3 Test Equipment	  1

5.0 Detail Requirements	  2

5.1 Visual Examination, Internal	  2

5.2 Electrical Measurements	  2

5.2.1 Dielectric Withstanding Voltage Test	  3

5.2.2 Insulation Resistance Test	  3

5.2.3 DC Winding Resistance	  3

5.2.4 Magnetizing Inductance	  4

5.2.5 Turns Ratio and Polarity	  4

5.3 Thermal Shock	  4

5.4 External Visual Examination	  4

5.5 Post Thermal Shock Electrical Tests	  5

5.6 Final Inspection	  5

6.0 Test Procedure Forms	 11

Form 1.	Transformer, Power, Coil-Driver	 11

Form 2.	Transformer, Current-Sense	 13

Form 3.	Transformer, Voltage-Sense	 14

Form 4.	Transformer, Power, DC-DC Converter	 16

Form 5.	Inductor, Common-Mode, Input	 19

Form 6.	Inductor, Differential-Mode	 21

Form 7.	Inductor, Filter, 360 µH	 23

Form 8.	Inductor, Filter, Coupled	 25

Form 9.	Inductor, Output, Common-Mode	 27

Tables

Table 1.   Dielectric Strength/Insulation Resistance	  6

Table 2.   Dielectric Strength/Insulation Resistance	  7

Table 3.   GD1534097 Transformer, Power, Coil Driver	  8

Table 4.   GD1534098 Transformer, Current-Sense	  8

Table 5.   GD1534096 Transformer, Voltage-Sense	  8

Table 6.   GD1534095 Transformer, Power, DC-DC Converter	  9

Table 7.   GD1534091 Inductor, Common-Mode, Input	  9

Table 8.   GD1534090 Inductor, Differential-Mode	  9

Table 9.   GD1534092 Inductor, Filter, 360 µH	 10

Table 10.  GD1534094 Inductor, Filter, Coupled	 10

Table 11.  GD1534093 Inductor, Common-Mode, Output	 10

1.0 Objective

This document provides the guidelines and procedures for the testing of custom-wound, in-house electromagnetic parts. The test procedures are aimed at identifying potential failures, due to both obvious and latent defects, prior to installing the parts into the next higher assembly.

2.0 Scope

This document defines the test procedures that are required in order to qualify in-house manufactured electromagnetic parts for use on the FAST MUE DC to DC Converter and the FAST MUE Coil Driver subassemblies.

3.0 Applicable Documents

The following documents form a part of this procedure to the extent specified herein.  In the event of a discrepancy, this procedure shall take precedence.

MIL-T-27		Transformers and Inductors, General Specification for

MIL-STD-202	Test Methods for Electronic and Electrical Component Parts

MIL-STD-981	Design, Manufacturing, and Quality Standards for Custom Electromagnetic Devices for Space Applications

4.0 General Requirements

The general requirements for documentation and data determination are specified herein.

4.1 Test Data

The results of all tests and measurements shall be retained for a period of five years after the task is completed (NAS-5-30963, Sec E, Para E-4).

4.2 Test and Inspection Log

A test and inspection log of the appropriate type for each part shall be used to record the results of all tests and examinations and shall be retained as part of the test data. In addition, QA shall monitor the results of all test activities.

4.3 Test Equipment

All test equipment used for testing electromagnetic parts shall be calibrated per

MIL-STD-45662.

5.0 Detail Requirements

Required Tests and Sequence:  All electromagnetic parts shall be subjected to the following tests and examinations in the sequence shown.

A.	Visual Examination, Internal

B.	Electrical Measurements

C.	Thermal Shock

D.	Visual Examination, External

E.	Post Thermal Shock Tests

5.1 Visual Examination, Internal

An internal visual examination shall be made on each part during the manufacturing procedure just prior to potting. Parts that fail to meet the following requirements, when examined under the magnification specified herein, shall be removed from the lot. Rejected parts shall be handled in accordance with the Martin Marietta Services QAP manual.

A.	Assembly of internal parts shall indicate that proper manufacturing techniques have been used (1X magnification).

B.	Wire shall be uniformly wound with no damaged or broken parts (10X).

C.	Internal connections shall be free of imperfections (10X).

D.	Lead wire shall be the correct length and shall contain no kinks (5X).

E.	Wire insulation shall contain no cracks, breaks, or cuts (10X).

F.	Parts shall contain no foreign particles (10X).

5.2 Electrical Measurements

Electrical measurements shall consist of the following tests in the sequence shown.  Enter all test data onto the data sheets (Forms 1-9) in Section 6.0.

A.	Dielectric Strength

B.	Insulation Resistance

C.	DC Resistance

D.	Magnetizing Inductance

E.	Turns Ratio and Polarity

5.2.1 Dielectric Withstanding Voltage Test

Test Method:  The Dielectric Withstanding Voltage Test shall be performed in accordance with MIL-T-27, para. 4.8.8.1, using method 301 of MIL-STD-202. The following details and exceptions shall apply:

A.	Mounting: The device under test shall be placed on a metal plate.

B.	Test Voltage:  As specified in Tables 1 and 2 of this section.

C.	Duration of Applied Voltage:  60 seconds minimum.

D.	Points of Application of Test Voltage:  Between all mutually insulated windings and between these windings and the metal plate.

Accept-Reject Criteria:  Electromagnetic parts that have leakage currents that exceed the maximum specified in Tables 1 and 2, or that show any evidence of arcing, flashover, insulation breakdown, or other damage shall be removed from the lot.

5.2.2 Insulation Resistance Test

Test Method: Insulation resistance tests shall be performed in accordance with MIL-T-27, para. 4.8.10, using method 302 of MIL-STD-202.  The following details and exceptions shall apply:

A.	Mounting:  The device under test shall be placed on a metal plate.

B.	Test Voltage:  As specified in Tables 1 and 2.

C.	Points of Measurement:  The measurements shall be made between each winding and the metal plate with all other windings connected to the plate.

D.	Electrification Time: 2 minutes minimum (Resistance readings shall be taken at the end of the electrification time).

Accept-Reject Criteria: Measured insulation resistance values shall exceed the minimum

insulation resistance as specified in Tables 1 and 2.  Failed units shall be removed from the lot.

5.2.3 DC Winding Resistance

The DC Winding Resistance shall be measured in accordance with MIL-T-27, para. 4.8.11.3. The following details shall apply:

A.	Resistance readings shall be measured at or corrected to 20 C.

B.	For resistances less than 1 ohm, measurements shall be made using a 4-wire measurement technique and a precision digital multimeter.

Accept-Reject Criteria: Measured resistance values for each unit under test shall be as specified in Tables 3-11 in this section, as applicable.

5.2.4 Magnetizing Inductance

The self inductance of each inductor under test and the primary magnetizing inductance of each transformer under test shall be measured in accordance with MIL-T-27, para. 4.8.11.4 at the specified test voltage and frequency. Measured inductance values for each unit under test shall be as specified in Tables 3-11.

5.2.5 Turns Ratio and Polarity

The turns ratio and polarity of each transformer and multi-winding inductor under test shall be measured in accordance with MIL-T-27, para. 4.8.11.14 and 4.8.11.17. Turn ratios shall be determined by the voltmeter method or any other suitable means. The turns ratio and polarity for each unit under test shall be as specified in Tables 3-11. The test shall be performed at 125 KHz.

5.3 Thermal Shock

Test Method:  Electromagnetic parts shall be tested in accordance with MIL-T-27, para. 4.8.4.2, and method 107 of MIL-STD-202.  The following details and exceptions shall apply:

A.	Test Condition Letter: A, except 10 cycles.

B.	The temperature for Step 3 shall be the maximum operating temperature for the class (130 C).

C.	Exposure time at temperature extremes (Steps 1 and 3) shall be 30 +0/-5 minutes.

"Dwell time between temperature extremes at room temperature shall be 5 minute 	maximum."

D.	Mounting:  Parts shall be mounted to minimize direct heat conduction between the specimen and the mounting apparatus used.

Accept/Reject Criteria: Any parts that show evidence of cracking or crazing

(10X magnification) shall be removed from the lot.

5.4 External Visual Examination

Parts shall be examined in accordance with MIL-T-27, para. 4.8.1.1, to verify that the materials, external design and construction, physical dimensions, and workmanship comply with the applicable drawing. Parts that fail to meet the following requirements, when examined under the magnification specified, shall be removed from the lot.

A.	Marking and dimensions shall be as specified in the manufacturing procedure (1X).

B.	Wire, if used, shall be free of imperfections (10X).

C.	Potting shall show no evidence of cracking, crazing, or other imperfections that would render the part unsuitable for its intended purpose (10X).

5.5 Post Thermal Shock Electrical Tests

Following thermal shock testing, transformers and inductors shall be subject to the following electrical tests:

A.	Dielectric Withstanding Voltage, para. 5.2.1.

B.	Insulation Resistance, para. 5.2.2.

C.	DC Winding Resistance, para. 5.2.3.

D.	Magnetizing Inductance, para. 5.2.4.

E.	Turns Ratio and Polarity, para. 5.2.5.

5.6 Final Inspection

Perform final inspection and data package review. Rejected parts shall be handled in accordance with the Martin Marietta Services QAP manual.

Table 1.   Dielectric Strength/Insulation Resistance

| Part Number | GD1534097 | GD1534098 | GD1534096 | GD1534095 |
| --- | --- | --- | --- | --- |
| Symbol | T101, T201 | T3, 5, 100, 200 | T2, 6, 102, 202 | T1, 4 |
| Data Sheet | Form 1 | Form 2 | Form 3 | Form 4 |
|  |  | Dielectric | Strength |  |
| Test Voltage | 300 VRMS | N/A | N/A | 300 VRMS |
| Maximum Leakage (amps) | .63  10-7 | N/A | N/A | .63  10-7 |
| Test Duration | 60 Seconds | N/A | N/A | 60 Seconds |
|  |  | Insulation | Resistance |  |
| Test Voltage | 100 VDC | N/A | 100 VDC | 100 VDC |
| Minimum Resistance (megohms) | 1000 | N/A | 1000 | 1000 |
| Test Duration | 2 Minutes | N/A | 2 Minutes | 2 Minutes |

Table 2.   Dielectric Strength/Insulation Resistance

| Part Number | GD1534091 | GD1534090 | GD1534092 | GD1534094 | GD1534093 |
| --- | --- | --- | --- | --- | --- |
| Symbol | L1 | L2 | L3, L5 | L4, L6 | L7, L8, L9 |
| Data Sheet | Form 5 | Form 6 | Form 7 | Form 8 | Form 9 |
|  |  | Dielectric | Strength |  |  |
| Test Voltage | 300 VRMS | 300 VRMS | 300 VRMS | 300 VRMS | 300 VRMS |
| Maximum Leakage (amps) | .63  10-7 | .63  10-7 | .63  10-7 | .63  10-7 | .63  10-7 |
| Test Duration | 60 Seconds | 60 Seconds | 60 Seconds | 60 Seconds | 60 Seconds |
|  |  | Insulation | Resistance |  |  |
| Test Voltage | 100 VDC | 100 VDC | 100 VDC | 100 VDC | 100 VDC |
| Minimum Resistance (megohms) | 1000 | 1000 | 1000 | 1000 | 1000 |
| Test Duration | 2 Minutes | 2 Minutes | 2 Minutes | 2 Minutes | 2 Minutes |

Table 3.   GD1534097 Transformer, Power, Coil Driver

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | 5-6 | --- | --- | --- |
| DC Resistance (ohms) ±20% | .050 | .120 | .106 | --- | --- | --- |
| Inductance ±25% | 250 µH | N/A | N/A | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *2 | 3 Vpp *4 | 7.4 Vpp *6 | --- | --- | --- |

Table 4.   GD1534098 Transformer, Current-Sense

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 3-4 | 1-2 | --- | --- | --- | --- |
| DC Resistance (ohms) ±20% | N/A | .930 | --- | --- | --- | --- |
| Inductance ±25% | N/A | 12 mH | --- | --- | --- | --- |
| Turns Ratio with 10 Turn Primary | 3 Vpp *4 | 30 Vpp *2 | --- | --- | --- | --- |

Table 5.   GD1534096 Transformer, Voltage-Sense

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | --- | --- | --- | --- |
| DC Resistance (ohms) ±20% | .245 | .245 | --- | --- | --- | --- |
| Inductance ±25% | 400 µH | 400 µH | --- | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *2 | 3 Vpp *4 | --- | --- | --- | --- |

* With Respect to Lead Number _______

Table 6.   GD1534095 Transformer, Power, DC-DC Converter

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | 5-6 | 7-8 | 9-10 | 11-12 |
| DC Resistance (ohms) ±20% | .055 | .181 | .405 | .065 | .214 | .214 |
| Inductance ±25% | 215 µH | N/A | N/A | N/A | N/A | N/A |
| Turns Ratio and Polarity | 3 Vpp *2 | 3 Vpp *4 | 5.14 Vpp *6 | 2.14 Vpp *8 | 5.57 Vpp *10 | 5.57 Vpp *12 |

Table 7.   GD1534091 Inductor, Common-Mode, Input

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | --- | --- | --- | --- |
| DC Resistance (ohms) ±20% | .065 | .065 | --- | --- | --- | --- |
| Inductance ±25% | 675 µH | 675 µH | --- | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *2 | 3 Vpp *4 | --- | --- | --- | --- |

Table 8.   GD1534090 Inductor, Differential-Mode

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | 1-3** | --- | --- | --- |
| DC Resistance (ohms) ±20% | .094 | .094 | --- | --- | --- | --- |
| Inductance ±10% | --- | --- | 320 µH | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *1 | 3 Vpp *4 | --- | --- | --- | --- |

* With Respect to Lead Number _______

** With Terminals 2 and 4 Tied Together

Table 9.   GD1534092 Inductor, Filter, 360 µH

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | --- | --- | --- | --- | --- |
| DC Resistance (ohms) ±20% | .115 | --- | --- | --- | --- | --- |
| Inductance ±10% | 360 µH | --- | --- | --- | --- | --- |

Table 10.  GD1534094 Inductor, Filter, Coupled

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | 5-6 | --- | --- | --- |
| DC Resistance (ohms) 20% | .080 | .62 | .62 | --- | --- | --- |
| Inductance ±10% | 100 µH | 665 µH | 665 µH | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *2 | 7.80 Vpp *4 | 7.80 Vpp *6 | --- | --- | --- |

Table 11.  GD1534093 Inductor, Common-Mode, Output

| Winding Number | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Terminal Numbers | 1-2 | 3-4 | --- | --- | --- | --- |
| DC Resistance (ohms) ±20% | .055 | .055 | --- | --- | --- | --- |
| Inductance ±25% | 100 µH | 100 µH | --- | --- | --- | --- |
| Turns Ratio and Polarity | 3 Vpp *2 | 3 Vpp *4 | --- | --- | --- | --- |

* With Respect to Lead Number _______

6.0 Test Procedure Forms

Form 1.	Transformer, Power, Coil-Driver			P/N  GD1534097		S/N_______________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

1.0 	Qualification Tests

1.1 	Internal Visual Test:  Test per 5.1.  Record results.					Pass/Fail_____________

1.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1	__________  amps	Lead 4	__________  amps

Lead 2	__________  amps	Lead 5	__________  amps

Lead 3	__________  amps	Lead 6	__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1	__________  ohms	Lead 4	__________  ohms

Lead 2	__________  ohms	Lead 5	__________  ohms

Lead 3	__________  ohms	Lead 6	__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		250 µH ±25%	__________	.050 ohms ±20%	__________

3 to 4		N/A			__________	.120 ohms ±20%	__________

5 to 6		N/A			__________	.106 ohms ±20%	__________

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding Number	Terminal Numbers		Pass/Fail

2				3 - 4			__________

3				5 - 6			__________

1.6	Polarity Test:  Test per 5.2.5.  Record results below.

Winding Number	Terminal Numbers		Pass/Fail

2				3 - 4			__________

3				5 - 6			__________

1.7	Thermal Shock: Test per 5.3. Record results. 					Pass/Fail___________

2.0	Post Thermal Shock Tests

2.1	External Visual Test:  Test per 5.4. Record results.				Pass/Fail___________

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1	__________  amps	Lead 4	__________  amps

Lead 2	__________  amps	Lead 5	__________  amps

Lead 3	__________  amps	Lead 6	__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1	__________  ohms	Lead 4	__________  ohms

Lead 2	__________  ohms	Lead 5	__________  ohms

Lead 3	__________  ohms	Lead 6	__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		250 µH ±25%	__________	.050 ohms ±20%	__________

3 to 4		N/A			__________	.120 ohms ±20%	__________

5 to 6		N/A			__________	.106 ohms ±20%	__________

2.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding Number	Terminal Numbers		Pass/Fail

2				3 - 4			__________

3				5 - 6			__________

Performed by:  __________________________		Date:  ___/___/___

Q/A Review:     __________________________		Date:  ___/___/___

Form 2.	Transformer, Current-Sense	                	P/N  GD1534098		S/N  _______________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

LCR Meter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

Oscilloscope    	_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1	Internal Visual Test:  Test per 5.2.1.  Record results.					Pass/Fail ________

1.2	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

3 to 4		12 mH ± 25%	__________	.93 ohms  ± 20%	__________

1.3	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding Number	Terminal Numbers		Pass/Fail

2				3 - 4			__________

1.4	Thermal Shock:  Test per 5.3.  Record results.					Pass/Fail ___________

2.0	Post Thermal Shock Tests

2.1	External Visual Examination:   Test per 5.4.  Record results.				Pass/Fail___________

2.2	Inductance and Resistance Tests:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

3 to 4		12 mH  ± 25%	__________	.93 ohms  ± 20%	__________

2.3	Turns Ratio Test:  Test per 5.2.5  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 		__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 3.	Transformer, Voltage-Sense			P/N  GD1534096	S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

LCR Meter		_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

Oscilloscope		_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1.	Internal Visual Test:  Test per 5.1.  Record results below.			Pass/Fail ______________

1.2	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

Lead 3		__________  ohms

Lead 4		__________  ohms

1.3	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		400 µH  ± 25%	__________	.245 ohms ± 20%	__________

3 to 4		400 µH  ± 25%	__________	.245 ohms ± 20%	__________

1.4	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 		__________

1.5	Polarity Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 		__________

1.6	Thermal Shock Test:  Test per 5.3.  Record results.				Pass/Fail

2.0	Post Thermal Shock Tests

2.1	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

Lead 3		__________  ohms

Lead 4		__________  ohms

2.2	Inductance and Resistance Tests:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		500 µH  ± 20%	__________	.245 ohms ± 20%	__________

3 to 4		500 µH  ± 20%	__________	.245 ohms ± 20%	__________

2.3	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 		__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 4.	Transformer, Power, DC-DC Converter		P/N  GD1534095		S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1	Internal Visual Examination:  Test per 5.2.1.  Record results.				Pass/Fail __________

1.2Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 7		__________  amps

Lead 2		__________  amps	Lead 8		__________  amps

Lead 3		__________  amps	Lead 9		__________  amps

Lead 4		__________  amps	Lead 10	__________  amps

Lead 5		__________  amps	Lead 11	__________  amps

Lead 6		__________  amps	Lead 12	__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 7		__________  ohms

Lead 2		__________  ohms	Lead 8		__________  ohms

Lead 3		__________  ohms	Lead 9		__________  ohms

Lead 4		__________  ohms	Lead 10	__________  ohms

Lead 5		__________  ohms	Lead 11	__________  ohms

Lead 6		__________  ohms	Lead 12	__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		215 µH ± 25%	__________	.055 ohms  ± 20%	__________

3 to 4		N/A			__________	.181 ohms  ± 20%	__________

5 to 6		N/A			__________	.405 ohms  ± 20%	__________

7 to 8		N/A			__________	.065 ohms  ± 20%	__________

9 to 10	N/A			__________	.214 ohms  ± 20%	__________

11 to 12	N/A			__________	.214 ohms  ± 20%	__________

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

3		    5-6 	__________

4		    7-8 	__________

5		    9-10	__________

6		    11-12 	__________

1.6	Polarity Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		3-4 		__________

3		5-6 		__________

4		7-8 		__________

5		9-10		__________

6		11-12 		__________

1.7	Thermal Test:  Test per 5.3.  Record results.						Pass/Fail _________

2.0	Post Thermal Shock Tests

2.1	External Visual Test:  Test per 5.4.  Record results.					Pass/Fail _________

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 7		__________  amps

Lead 2		__________  amps	Lead 8		__________  amps

Lead 3		__________  amps	Lead 9		__________  amps

Lead 4		__________  amps	Lead 10	__________  amps

Lead 5		__________  amps	Lead 11	__________  amps

Lead 6		__________  amps	Lead 12	__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 7		__________  ohms

Lead 2		__________  ohms	Lead 8		__________  ohms

Lead 3		__________  ohms	Lead 9		__________  ohms

Lead 4		__________  ohms	Lead 10	__________  ohms

Lead 5		__________  ohms	Lead 11	__________  ohms

Lead 6		__________  ohms	Lead 12	__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		215 µH ± 25%	__________	.055 ohms  ± 20%	__________

3 to 4		N/A			__________	.181 ohms  ± 20%	__________

5 to 6		N/A			__________	.405 ohms  ± 20%	__________

7 to 8		N/A			__________	.065 ohms  ± 20%	__________

9 to 10	N/A			__________	.214 ohms  ± 20%	__________

11 to 12	N/A			__________	.214 ohms  ± 20%	__________

2.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal		Pass/Fail

Number	Numbers

2		    3-4 		__________

3		    5-6 		__________

4		    7-8 		__________

5		    9-10		__________

6		    11-12 		__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 5.	Inductor, Common-Mode, Input			P/N  GD1534091		S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

1.0	Qualifications Tests

1.1	Internal Visual Test:  Test per 5.1.  Record results.					Pass/Fail _______

1.2	Dielectrical Strength:  Test per 5.2.1. Record results below.

Lead 1			__________  amps

Lead 2			__________  amps

Lead 3			__________  amps

Lead 4			__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1			__________  ohms

Lead 2			__________  ohms

Lead 3			__________  ohms

Lead 4			__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		675 µH  ± 25%	__________	.065 ohms  ± 20%	__________

3 to 4		675 µH  ± 25%	__________	.065 ohms  ± 20%	__________

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.6	Polarity Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.7Thermal Shock:  Test per 5.3.  Record results.			Pass/Fail _______

2.0	Post Thermal Shock Tests

2.1External Visual Test:  Test per 5.4.  Record results.			Pass/Fail _______

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 3		__________  amps

Lead 2		__________  amps	Lead 4		__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 3		__________  ohms

Lead 2		__________  ohms	Lead 4		__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		675 µH  ± 25%	__________	.065 ohms  ± 20%	__________

3 to 4		675 µH  ± 25%	__________	.065 ohms  ± 20%	__________

2.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 6.	Inductor, Differential-Mode				P/N  GD1534090		S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1	Internal Visual Test:  Test per 5.1.  Record results.					Pass/Fail _______

1.2	Dielectrical Strength:  Test per 5.2.1. Record results below.

Lead 1			__________  amps

Lead 2			__________  amps

Lead 3			__________  amps

Lead 4			__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1			__________  ohms

Lead 2			__________  ohms

Lead 3			__________  ohms

Lead 4			__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		N/A			__________	.094 ohms  ± 20%	__________

3 to 4		N/A			__________	.094 ohms  ± 20%	__________

1 to 3		320 µH  ± 10%	__________	N/A

w/2 and 4 connected

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results Below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.6	Polarity Test:  Test per 5.2.5.  Record results.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.7Thermal Shock:  Test per 5.3.  Record results.			Pass/Fail _______

2.0	Post Thermal Shock Tests

2.1External Visual Test:  Test per 5.4.  Record results.			Pass/Fail _______

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 3		__________  amps

Lead 2		__________  amps	Lead 4		__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 3		__________  ohms

Lead 2		__________  ohms	Lead 4		__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		N/A			__________	.094 ohms  ± 20%	__________

3 to 4		N/A			__________	.094 ohms  ± 20%	__________

1 to 3		320 µH  ± 10%	__________	N/A

w/2 and 4 connected

2.5	Turns Ratio Test:  Test per 5.2.5.  Record results.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 7.	Inductor, Filter, 360 µH				P/N  GD1534092		S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Insulation Tester	_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1	Internal Visual Test:  Test per 5.1.  Record results.					Pass/Fail _________

1.2	Dielectrical Strength:  Test per 5.2.1. Record results below.

Lead 1		__________  amps

Lead 2		__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance		Measured	Resistance		Measured

Number	Limits			Inductance	Limits			Resistance

1 to 2		360 µH  ± 10%	__________	.115 ohms ± 20%	__________

1.5	Thermal Shock:  Test per 5.3.  Record results. 					Pass/Fail _________

2.0	Post Thermal Shock Tests

2.1	External Visual Test:  Test per 5.4.  Record results.					Pass/Fail _________

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps

Lead 2		__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		360 µH  ± 10%	__________	.115 ohms  ± 20%	__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 8.	Inductor, Filter, Coupled			P/N  GD1534094	S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	________________

Digital Multimeter	_________________	_______________	_______________	________________

1.0	Qualification Tests

1.1	Internal Visual Test:  Test per 5.1.  Record results.				Pass/Fail _______

1.2	Dielectrical Strength:  Test per 5.2.1. Record results below.

Lead 1		__________  amps

Lead 2		__________  amps

Lead 3		__________  amps

Lead 4		__________  amps

Lead 5		__________  amps

Lead 6		__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

Lead 3		__________  ohms

Lead 4		__________  ohms

Lead 5		__________  ohms

Lead 6		__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		100 µH  ± 10%	__________	.080 ohms  ± 20%	__________

3 to 4		665 µH  ± 10%	__________	.62 ohms  ± 20%	__________

5 to 6		665 µH  ± 10%	__________	.62 ohms  ± 20%	__________

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

3		    5-6 	__________

1.6	Polarity Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

3		    5-6 	__________

1.7	Thermal Shock:  Test per 5.3.  Record results.			Pass/Fail _______

2.0	Post Thermal Shock Tests

2.1	External Visual Test:  Test per 5.4.  Record results.			Pass/Fail _______

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 4		__________  amps

Lead 2		__________  amps	Lead 5		__________  amps

Lead 3		__________  amps	Lead 6		__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 4		__________  ohms

Lead 2		__________  ohms	Lead 5		__________  ohms

Lead 3		__________  ohms	Lead 6		__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		100 µH  ± 10%	__________	.080 ohms  ± 20%	__________

3 to 4		665 µH  ± 10%	__________	.62 ohms  ± 20%	__________

5 to 6		665 µH  ± 10%	__________	.62 ohms  ± 20%	__________

2.5	Turns Ratio Test: Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

3		    5-6 	__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___

Form 9.	Inductor, Output, Common-Mode		P/N  GD1534093     	S/N  ___________

Test Equipment	Manufacturer		Model Number		Cal. Due Date		NASA ID Number

Oscilloscope		_________________	_______________	_______________	________________

Insulation Tester	_________________	_______________	_______________	________________

Megohmmeter		_________________	_______________	_______________	________________

Function Generator	_________________	_______________	_______________	________________

LCR Meter		_________________	_______________	_______________	_________________

Digital Multimeter	_________________	_______________	_______________	_________________

1.0	Qualification Tests

1.1	Internal Visual Test:  Test per 5.1.  Record results.			Pass/Fail _______

1.2	Dielectrical Strength:  Test per 5.2.1. Record results below.

Lead 1		__________  amps

Lead 2		__________  amps

Lead 3		__________  amps

Lead 4		__________  amps

1.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms

Lead 2		__________  ohms

Lead 3		__________  ohms

Lead 4		__________  ohms

1.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Number	Limits			Inductance	Limits			Resistance

1 to 2		100 µH ± 25%	__________	.055 ohms  ± 20%	__________

3 to 4		100 µH ± 25%	__________	.055 ohms  ± 20%	__________

1.5	Turns Ratio Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.6	Polarity Test:  Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

1.7Thermal Shock:  Test per 5.3.  Record results.			Pass/Fail _______

2.0	Post Thermal Shock Tests

2.1External Visual Test:  Test per 5.4.  Record results.			Pass/Fail _______

2.2	Dielectrical Strength:  Test per 5.2.1.  Record results below.

Lead 1		__________  amps	Lead 3		__________  amps

Lead 2		__________  amps	Lead 4		__________  amps

2.3	Insulation Resistance:  Test per 5.2.2.  Record results below.

Lead 1		__________  ohms	Lead 3		__________  ohms

Lead 2		__________  ohms	Lead 4		__________  ohms

2.4	Inductance and Resistance Measurements:  Measure per 5.2.3 and 5.2.4.  Record results below.

Terminal	Inductance 		Measured	Resistance		Measured

Numbers	Limits			Inductance	Limits			Resistance

1 to 2		100 µH  ± 25%	__________	.055 ohms  ± 20%	__________

3 to 4		100 µH  ± 25%	__________	.055 ohms  ± 20%	__________

2.5	Turns Ratio Test: Test per 5.2.5.  Record results below.

Winding	Terminal	Pass/Fail

Number	Numbers

2		    3-4 	__________

Performed by:  __________________________		Date:  ___/___/___

QA Review:     __________________________		Date:  ___/___/___