FAST-PROC-095

FAST RF HARNESS

ELECTRICAL INTEGRATION PROCEDURE

P/N 85S01S01040.0

S/N 894093

Prepared by:

Ken Hersey, Code 737.1			Date

Antenna Engineer

Approved by:

Dave Everett, Code 743.1			Date

FAST Electrical Systems Engineer

Jim Watzin, Code 704.1			Date

SMEX Chief Systems Engineer

Ron Kolecki, Code 303.0			Date

SMEX Flight Assurance Manager

1.0	SCOPE

This document describes the electrical integration of the RF Harness to the flight antenna and the flight transponder. After integration has been completed, a short functional of the antenna subsystem will be conducted.

All steps that require torquing of hardware shall be witnessed by SMEX QA.

2.0	HARDWARE/EQUIPMENT

The following hardware is required to execute this procedure:

RF Harness, GORE P/N 85S01S01040.0, S/N 0894093

SMA Torque Wrench

The following test equipment is required to execute this procedure:

Narda Radiation Monitor

Narda Isotropic Probe

3.0	PREPARATION

1.	Clean the outer surface of the RF harness with isopropyl alcohol. _____

2.	Using a swab, clean the inside of the SMA connectors with isopropyl alcohol. _____

3.	Visually inspect the connectors to verify cleanliness. _____

4.0	DETAIL OF CONNECTOR OPERATIONS

This reference section describes the procedure for connecting SMA connectors. It should be followed whenever the procedure calls for mating SMA connectors.

Only personnel who have been trained in the connection of GORE coaxial assemblies shall be involved in their integration and de-integration.

1.	Align the center lines of each connector.

2.	By hand, mate the coupling nut until it bottoms.

3.	Torque the coupling nut to 8 in-lbs.

4.	At all times:  the harness and female jack shall remain stationary during the connection operation – only the coupling nut shall be rotated. A 5/16  inch wrench should be used to prevent the harness from rotating.

5.0	INTEGRATION OF RF HARNESS

At all times, the 1.0 inch minimum bend radius of the harness must be observed.

1.	Route one end of the harness through the radiation disk connector hole, through the hole in the transponder radiation disk support, to the diplexer output connector. _____

2.	Mate the harness to the diplexer output connector. _____            _____ (QA)

Torque Wrench ID: _____	Cal Due: _____

3.	Route the other end of the harness along the upper axial boom to the antenna connector. _____

4.	Mate the harness to the antenna connector. _____     _____ (QA)

Torque Wrench ID: _____	Cal Due: _____

6.0	SHORT FUNCTIONAL

All personnel shall be at least 6.25 inches from the antenna during this test.

1.	Turn the transmitter on (XMTRPWR ON). _____

2.	Verify that the antenna is radiating with the radiation monitor. _____

Radiation Monitor ID: _____	Cal Due: _____

Isotropic Probe ID: _____	Cal Due: _____

3.	Turn the transmitter off (XMTRPWR OFF). _____