SMEX FAST

SPACECRAFT FUNCTIONAL TEST PLAN

FAST-TEV-035

David Everett

October 12, 1993

This document describes the spacecraft functional test.  This test verifies that the major functions of the spacecraft are working within nominal parameters.

This test will take about eight hours.  It will be run after every environmental test and at each plateau during thermal vacuum.  The test is a single STOL procedure (which calls other procedures).  The test flow is arranged to exercise groups of spacecraft functions in a manner similar to normal operations during the campaign.  For example, the power system test discharges the battery by powering on the transmitter and instruments.  The transponder and instruments are checked out by dumping science data while the battery is being discharged.

The subsystem leads are responsible for developing STOL procedures which will be called by the Spacecraft Functional Test.  The electrical system engineer will merge the individual procedures into a cohesive test.

The following is an incomplete list of testing by subsystem:

ACS:  Mary Walker

--vary pulse periods

--send bad pulses (extra, too wide, missing)

--vary torque coil settings (strength and polarity)

--run precession and spin coils simultaneously

--stimulate magnetometer

--stimulate HCI and sun sensor

--run initial acquisition from three different initial conditions

--run spin control from HCI, sun sensor, and magnetometer inputs

C&DH:  Tim Gruner

--operate absolute and relative time sequences

--swap processors

--receive telemetry via Pegasus interface

--check clock stability

--check continuity and integrity of VC1 in safehold recorder

--perform memory writes to and reads from various locations

--send many commands in rapid succession

Many of the C&DH tests will come from the software acceptance test procedures.

Instrument:  Henry Heetderks

--verify continuity and integrity of VC1 in engineering recorder

--operate at all data rates

--collect science data while stimulating instruments

--verify operation of the spin-phase clock

--perform memory writes to and reads from various locations

--perform PROM swap and reboot

--check communication on main and redundant of port A

--verify rate limiting of VC1, VC3, and VC5

--operate instruments in different modes

Power:  Tom Spitzer

Similar to the power portion of the SAMPEX functional test.

Pyro:  Tim Gruner

--send pyro commands with GSE connected to the arming plug

RF:  Haydee Maldonado

Similar to the RF portion of the SAMPEX functional test.