FAST FLIGHT STRUCTURE ALIGNMENT PLAN

OBJECTIVE

The objectives for the FAST alignment are as follows:

-Establish  the spacecraft X,Y,Z coordinate system, with origin     at center of interface plane bolt circle.

-Determine concentricity and parallelism of the axial boom          mounting holes on the instrument deck with respect to the          origin.

-Determine location of ESA/wire boom mounting holes with respect    to the origin.

-Determine the location of the mag boom base mounting holes with    respect to the origin.

The measurements will be performed in two configurations the first is with the structure in its free standing configuration and the second is with masses loading down the structure.  The mass sizes and locations will be determined during the alignment process.

LOGISTICS AND COORDINATE SYSTEM

The FAST structure is to be placed on a level/flat surface table.  This will establish the XY plane.  The X axis will be established as the line going through the two 0.251" dia. holes located at the bottom of the structures interface flange (see fig. 1).  Two modified spherical targets will be used to establish the X axis.  The Z axis is to be established as the line passing  vertically perpendicular through the center of the two 0.251" dia. holes.  The Y axis will be established by conventional right-hand rule method.

REFERENCE TARGETS

Nine reference targets are located on the edge of the instrument deck (see figure 2).  These targets are visible when the FAST spacecraft is fully built-up and are to be used as transfer axes for the established origin.  The target drawings are shown in figure 3.  The reference targets are to be designated RFT-1, thru. RFT-9.

MEASUREMENT TARGETS

The alignment targets are to be placed into aluminum plugs which are threaded into each of the interface holes.  The plug drawings are shown in fig. 4.  The additional height of the plugs should be taken into account when the measurements are made.

LOCATIONS TO BE MEASURED

The locations to be measured are designated as follows and as shown in figure 5:

AXIAL BOOM: AXBM1, AXBM2, AXBM3, AXBM4, AXBM5, AXBM6, AXBM7,                   AXBM8 (8-32 threaded holes)

ELECTROSTATIC ANALYZER -A-: ESA-A1, ESA-A2, ESA-A3, ESA-A4,

ESA-A5(8-32 threaded holes)

ELECTROSTATIC ANALYZER -B-: ESA-B1, ESA-B2, ESA-B3, ESA-B4,

ESA-B5(8-32 threaded holes)

ELECTROSTATIC ANALYZER -C-: ESA-C1, ESA-C2, ESA-C3, ESA-C4,                                    ESA-C5(8-32 threaded holes)

ELECTROSTATIC ANALYZER -D-: ESA-D1, ESA-D2, ESA-D3, ESA-D4,

ESA-D5(8-32 threaded holes)

SEARCH COIL MAG. BOOM: SCMB1, SCMB2, SCMB3, SCMB4                                         (10-32 threaded holes)

FLUXGATE MAG. BOOM: FGMB1, FGMB2, FGMB3, FGMB4

(10-32 threaded holes)

FAST FLIGHT STRUCTURE ALIGNMENT PLAN

Prepared by: Giulio Rosanova/ Code 741         October 1993