FAST-TEV-054

THERMAL VACUUM TEST PLAN

FOR THE

FAST AURORAL SNAPSHOT EXPLORER (FAST)

PROTOFLIGHT ANTENNA

Prepared by:

Ken Hersey			Date

Antenna Engineer

Approved by:

Giulio Rosanova		Date

Mechanical Lead Engineer

Approved by:

Keith Parrish		Date

Thermal Lead Engineer

Approved by:

Pete Mule			Date

Verification Engineer

Approved by:

Peter Rossoni		Date

Test Facility Engineer

Approved by:

Ron Kolecki		Date

Flight Assurance Manager

1.0	Introduction

1.1	Purpose

Fast Auroral Snapshot Explorer (FAST) protoflight antenna thermal vacuum verification testing, consisting of eight cycles from -100°C to +55°C, will be performed to qualify the antenna design and accept the items for flight.

1.2	Test Objectives

The antenna testing will be performed to accept the items for flight by subjecting the test items to operating temperatures predicted by thermal analyses.

1.3	Pass/Fail Criteria

Structural degradation or failure short functional testing or post RF functional testing will constitute a test failure.

1.4	Test Item Description

The test items will be one protoflight FAST antenna, with the serial number denoted in the certification log. The antennas have a weight of 1.6 lbs, a diameter of 13", and a length of 5.5". The antenna will be mounted to the ETU upper axial boom during the test. The boom has a weight of 5 lbs, a diameter of 4", and a length (which includes the antenna) of 32". Drawings GE1524452 and 26302-FAXB-ICD illustrate the antenna and the assembly to the upper axial boom.

1.5	Test Facility

Building 7, Small Vacuum Chamber

1.6	Desired Start Test Date

March 15, 1993

1.7	Work Directive Number (assigned after completion of plan)

1.8	Fiscal and Labor JONs

FJON:  740-864-02-01-69

LJON:  754-864-02-01-69

1.9	Applicable Documents

a.	General Environmental Verification Specification for STS and ELV Payloads, Subsystems, and Components (GEVS-SE)

b.	Requirements Document for SMEX FAST Mission (FAST-SPEC-005)

2.0	Test Organization

Project Manager			Tim Gehringer/740.4	x6831

Test Item Representative	Ken Hersey/737.1		x4939

Verification Manager		Peter Mule/750.2		x8281

Mechanical Lead			Giulio Rosanova/741.3	x5907

Thermal Lead			Keith Parrish/724.1	x3104

Quality Assurance		Ron Kolecki/740.4		x9399

Contamination Engineer	Mike Rodiriquez/724.4	x9296

3.0	Special Precautions/Instructions

3.1	Safety

If the test must be aborted, the cable at the bulkhead outside the chamber must be disconnected before opening the chamber. Once the test is restarted, the cable should be reconnected, and the times of non-operation should be recorded.

3.2	Precautions

Due to the porous nature of the antenna cylinder and the paint existing on the aluminum, only water can be used, and sparingly, if necessary to clean the antenna. Care should be taken not to excessively scratch the tin/lead plated copper on the cylinder, nor to damage the single solder joint on the bottom of the antenna cylinder.

No ESD precautions are necessary.

3.3	Contamination

The antenna shall be handled with clean gloves at all times.

3.4	Emergencies

See Section 3.1 in the event of a facility shutdown.

4.0	Test Requirements

4.1	Test Operations

The test item representative will be responsible for overseeing the handling of the test item. The QA Representative shall be notified before the test is started.

4.2	Procedure Redlining

Redlines to this document must be approved by the mechanical lead, verification manager, test item representative, and QA representative and coordinated with the 754.4 test engineer.

5.0	Data Acquisition

5.1	Acquisition

Standard printouts of hourly thermocouple temperature readings are sufficient for this test.

6.0	Support Equipment

6.1	Thermocouples

Thermocouples shall be located as shown in the GE1524452 drawing portion at the end of this document.

6.2	Support Equipment

A Vector Network Analyzer, two cables, and an 8 in-lb torque wrench (provided by Code 737.1) are required.

7.0	Pre-test Operations

NOTE:

1.	Clean room gloves must be worn when handling the antenna.

2. 	Notify QA before beginning this test.

3. 	Clean all materials going into the chamber with alcohol and 			approved clean room wipes.

1. 	Mount antenna to ETU upper axial boom per FAST-PROC-092 and 		drawing 26302-FAXB-ICD.

2. 	Mount assembly inside chamber with 8 #8-32 bolts, 1" long. Torque to 		42 in-lbs. _____     _____ (QA)

Torque Wrench ID: _____	Cal Due: _____

3.	Install thermocouples as shown in the GE1524452 drawing portion at 		the end of this document.

4.	Connect inside cable to bulkhead. Torque to 8 in-lbs. _____     _____ 		(QA)

5.	Connect outside cable from bulkhead to test equipment. Torque to 8 in-		lbs. _____     _____ (QA)

6.	Setup and calibrate Vector Network Analyzer to measure VSWR from 		2.0-2.3 GHz.

7.	Connect inside cable to antenna. Torque to 8 in-lbs. _____     _____ 		(QA)

Torque Wrench ID: _____	Cal Due: _____

8.0	Test Operations

Each of eight cycles from -100°C to +55°C shall be performed under vacuum with a temperature transition rate of approximately 40°C/hour. Between each extrema, a four hour soak shall be performed.

A low level RF signal will be sent to the test item during functional tests. The test engineer need not be present during this operation is remote operations are in place.

As a minimum, a VSWR short functional shall be conducted at the first, last, and an intermediate soak at each extrema and during transition. It is planned that an automated test will be performed every hour, on the hour, coinciding with the thermal chamber thermocouple printout events.

9.0  Post Test Operations

Test item and instrumentation removal will occur immediately following test completion.