# Magnetic Test Procedure for

the FAST Spacecraft

June 13, 1994

Goddard Space Flight Center

Greenbelt, Maryland

Prepared by:

Darryl Mitchell, Code 754.2			Date

Magnetic Test Manager

Reviewed by:

John Lyons, GSFC Code 734			Date

Reviewed by:

Keith Parrish, GSFC Code 724			Date

Reviewed by:

Ron Kolecki, GSFC Code 303			Date

SMEX Flight Assurance Manager

Reviewed by:

Pete Mulè, GSFC Code 750.2			Date

Verification Manager for FAST

Approved by:

Edward Dyer, GSFC Code 754.			Date

Head Electromagnetic Test Engineering Section

Approved by:

David Everett, GSFC Code 743			Date

FAST Electrical Systems Engineer

Table of Contents

1.0  Purpose					1

2.0  Applicable Documents			1

3.0  Test Facilities				1

4.0  General Requirements			1

4.1  Contamination Control		1

4.2  ESD Control			2

4.3  Changes to Test Procedure		2

4.4  Responsibilities			2

5.0  Test Sequence				2

6.0  Test Procedure				3

6.1  Test Set-up				3

6.2  Permanent Magnetization		3

6.2.1  Initial Perm			3

6.2.2  5 Gauss Deperm		3

7.0  Test Limits					4

8.0  Additional Testing				4

1.0 PURPOSE

The purpose of this test is to characterize the magnetic field structure of the Fast Auroral SnapshoT (FAST) Spacecraft. Data collected will be used to determine spacecraft compliance with the SMEX-FAST " Magnetic Contamination and EMI/EMC Control and Implementation Plan" (FAST-SPEC-012).

2.0 APPLICABLE DOCUMENTS

The following documents form part of this procedure to the extent specified herein. In the case of conflict, the information presented in this procedure shall take precedence.

- SPACECRAFT MAGNETIC TEST FACILITY(ATTITUDE CONTROL TEST FACILITY) SMTF (ACTF), NASA Document X-754-83-9

- SPACECRAFT MAGNETIC TEST FACILITY OPERATING PROCEDURE, NSI Document 12-01-403

- SMEX-FAST EMI/EMC AND MAGNETIC TEST PLAN

- SMEX-FAST MAGNETIC CONTAMINATION AND EMI/EMC CONTROL AND IMPLEMENTATION PLAN, FAST-SPEC-012 Rev. A

The measurements will be conducted at the GSFC Spacecraft Magnetic Test Facility (SMTF), building 305. The SMTF contains a 13 meter coil system capable of nulling the ambient geomagnetic field to zero, with a uniformity of 0.001% within a spherical volume of two meters in diameter. The facility also contains the following additional equipment used for testing:

- Helmholtz perm-deperm coil system, 2.9 meter diameter

1-   Helmholtz perm-deperm coil system, 1.5 meter diameter

- Fluxgate magnetometers

1-  Dynamic Signal Analyzer, Hewlett Packard model HP 3562A

1-   Proton magnetometer, Varian model 4931

1-  Data Acquisition & Analysis System, Hewlett Packard 87 microcomputer

- Ea. dolly, turntable, & S/C mounting fixtures

- Power supplies for perming & deperming (DC. & 60 Hz )

4.0 GENERAL REQUIREMENTS

4.1 Contamination Control

Class 100,000 Clean Room conditions shall be maintained in the Spacecraft Magnetic Test Facility for the duration of the FAST Spacecraft magnetics test. The SMTF HEPA filter system fans shall be operated for a minimum of three days prior to the arrival of the FAST Spacecraft at the SMTF.

4.2 ESD Control

The FAST Spacecraft contains ESD sensitive components. ESD preventative measures will therefore be used before and during the magnetic test in order to prevent any damage to spacecraft systems.

4.3 Changes to Test Procedure

The Magnetic Test Procedure presented herein may be modified as necessary at the discretion of the cognizant test engineer. All changes to this procedure shall be annotated in the margins of the master copy. All changes shall be initialed and dated by the code 754 Test Engineer and an authorized Project Representative.

4.4 Responsibilities

4.4.1 Code 754.2/NSI Responsibility

Code 754.2/NSI shall provide the equipment and personnel needed to conduct the magnetic test of the FAST SPACECRAFT. Code 754.2/NSI shall be responsible for operating and monitoring all test equipment needed for this test. Code 754.2 shall provide documentation of the tests including photographs of the test setups and a test report. The test conductor shall be a designated representative of Code 754.2.

4.4.2 Code 740 Responsibilities

Code 740 is responsible for the handling and operation of the FAST Spacecraft during the Magnetics Test. Code 740 shall designate a Test Director who shall be the single point of contact with code 754.

5.0 TEST SEQUENCE

The FAST Spacecraft magnetic test sequence shall be performed as indicated in the table below.

| Test | Spacecraft Operating Mode | Spacecraft Power ### Source |
| --- | --- | --- |
| Initial perm | off | none |
| post deperm | off | none |
| post deperm | normal mode w/ instruments on | solar array connectors |
| post deperm | normal mode w/ instruments & transmitter on | solar array connectors |
| post deperm | normal mode w/ instruments on | battery _ |
| post deperm | normal mode w/ instruments on & battery charging | solar array connectors |
| post deperm | normal mode w/ instruments & ACS sensors off | battery |
| pc ;t derP m | normal mode w/ instruments & ACS sensors off | battery & solar array connectors |

An array of 60 compact florescent lamps will be used for tests where spacecraft power will be supplied by the spacecraft solar arrays.

6.0 TEST PROCEDURE

6.1 TEST SETUP

6.1.1 Set up three triaxial magnetometers on a north horizontal line passing through the center of the facility coils. Position the three magnetometer probes 1.0 meters, 1.5 meters and 2.0 meters from the center of the facility coils respectively.

6.1.2 Energize all test instrumentation and adjust the coil currents until zero field (<1.0 nT) is established inside the facility coil system.

6.1.3 Align the magnetometer probes to the coil system axes. 6.1.4 Perform an end-to-end data system calibration.

6.1.5 Perform a dry run magnetic mapping of the facility test dolly to ensure that no magnetic materials are present on the facility test dolly.

6.2 PERMANENT MAGNETIZATION

6.2.1 Initial Perm

6.2.1.1 Position the spacecraft outside the coil system on the facility test dolly. 6.2.1.2 Perform the initial background measurements.

6.2.1.3 Roll the spacecraft into the center of the test facility coils and note spacecraft orientation with respect to facility axes.

6.2.1.4 Rotate the table 360 degrees in azimuth and acquire the triaxial

magnetic field data at each 10 degree increment of rotation.

6.2.1.6 Roll the spacecraft to the outside of the test facility coil system. 6.2.1.7 Perform the final background measurement.

Note: At this time the data collection and analysis program will start reducing the data and calculating the three axial dipole moments.

6.2.2 5 Gauss Deperm

After completing the initial perm measurement sequence, the item is demagnetized by exposure to a 5 Gauss diminishing sinusoidal magnetic field. Once the deperm sequence is completed, post deperm measurements are performed in an identical manner to those of the initial perm measurements (steps 6.2.1.1 to 6.2.1.7).

7.0 TEST LIMITS

The magnetic test limits for the FAST Spacecraft are : Initial Perm, - 4 nT maximum field @ 1 meter

Post Deperm - 2 nT maximum field @ 1 meter

8.0 ADDITIONAL TESTING

Trimming and calibration of the ACS magnetometer and alignment of the science fluxgate magnetometer will also take place at the Spacecraft Magnetic Test Facility. A separate procedure will be developed by Code 740 to cover these tests.

NSI Technology Services Corporation

#### A Subsidiary of

ManTech International Corporation

To:	D. R. Mitchell/754.2    From:	P. K. Harris

Subject:	FAST Spacecraft Magnetic Date:	July 15, 1994

##### Measurements

Copies:	T. Cesko/NSI	In reply refer to:

C.A. Harris/NSI

D.H. Orbock/NSI

Introduction

Magnetic testing was performed on the Fast Auroral SnapshoT (FAST) Spacecraft from June 20 through July 1, 1994, in accordance with GSFC Document 7542-87-94, "Magnetic Test Procedure for the FAST Spacecraft."

Test Description

On June 22-25, magnetic field mappings were performed on the FAST Spacecraft in the Spacecraft Magnetic Test Facility (SMTF) at the Magnetic Test Site to characterize the magnetic field structure of the spacecraft. The initial perm, post 5 Gauss (5.0x105 nT) deperm, and DC stray magnetic field states of the spacecraft were measured at magnetometer distances of 1.0, 1.5, and 2.0 meters. Figure 1 shows the spacecraft on the spacecraft dolly in the center of the facility coils with the facility magnetometer probes visible.	Figure 2 shows the spacecraft inside the facility deperming coils.

The SMTF was also utilized for trimming and calibration of the FAST ACS magnetometer and alignment of the FAST science fluxgate magnetometer. Figure 3 shows the FAST Spacecraft in the center of the coils with one of the magnetometer booms extended. In addition, calibrations were performed on the FAST Torque Coils.

Test Results

The resulting dipole moments measured in mA-m2 (Gauss-cm3) for the FAST Spacecraft are presented,in Table 1. Table 2 gives a brief description of the conditions under which each test run was performed. Figure 4 shows the solar simulator setup for test run FSS/1.

Since calibration and alignment of the FAST magnetometers were performed by Code 740, the data from these tests are not provided in this report.

Table 3 shows the dipole moments measured in A-m2 (103 Gauss-cm3) for the calibration of the two FAST Torque Coils. Test runs F/C-2 through F/C-13 are from the first torque coil with F/C-2 through F/C-7 showing decreasing field strength and F/C-8 through F/C-13 showing decreasing field strength with the field in the reverse direction. Test runs F/C-14.1 through F/C-25 are from the second torque coil with F/C-14.1 through F/C-19 showing decreasing field strength and F/C-20 through F/C-25 showing decreasing field strength with the field in the reverse direction. The dipole moments measured were in agreement with the torque coil specifications provided by the FAST project personnel.

The complete test data (except for the torque coils calibration) are provided in the attached computer printouts (Attachments 1-13). Attachments 1 and 2 show that the FAST Spacecraft exceeds the magnetic test limits of 4 nT at 1 meter for the initial perm measurement and 2 nT at 1 meter for the post deperm measurement. These test limits are specified in GSFC 7542-87-94, which is included as Attachment 14.

P. K. Harris, Magnetics Engineer

PKH/pkh

Table 1. FAST S/C Magnetic Dipole Moments in mA-m2

| Run I | Magnetization I | Mx | My | Mz I | Mr |
| --- | --- | --- | --- | --- | --- |
| F/1 | Initial Perm | -139.6 | 82.3 | -73.4 | 177.9 |
| F/2 | Post Deperm | -72.3 | 51.7 | -27.9 | 93.1 |
| F/3 | Perm - Stray | -109.5 | 64.1 | 25.2 | 129.4 |
| F#1 | Perm + Stray | -135.1 | 109.2 | 70.2 | 187.4 |
| F#2 | Perm + Stray | -147.5 | 9.1 | 50.0 | 156.0 |
| F#3 | Perm + Stray | -147.4 | 121.9 | 23.4 | 192.7 |
| F#4 | Perm + Stray | -137.2 | 108.1 | 54.9 | 183.1 |
| F#5 | Perm + Stray | -115.3 | 102.1 | 65.3 | 167.3 |
| F#6 | Perm + Stray | -123.2 | 135.6 | 36.5 | 186.8 |
| F#7 | Perm + Stray | -157.9 | 131.1 | 59.1 | 213.5 |
| F#8 | Perm + Stray | -144.4 | 117.7 | 51.7 | 193.3 |
| F#9 | Perm + Stray | -143.3 | 118.5 | 44.2 | 191.1 |
| FSS/1 | Perm + Stray | -75.3 | 80.2 | 8.1 | 110.3 |

Table 2. FAST S/C Test Description

|  | Run 'Magnetization | Description |
| --- | --- | --- |
| F/1 | Initial Perm | Spacecraft power off. |
| F/2 | Post Deperm | Spacecraft power off. |
| F/3 | Perm - Stray | Spacecraft power off w/ GSE connected. |
| F#1 | Perm + Stray | Spacecraft in normal mode w/ instruments on (1.4 A lower shunt current). |
| F#2 | Perm + Stray | Spacecraft in normal mode w/ instruments & transmitter on.(4.2 A umbilical current w/ same shunt current). |
| F#3 | Perm + Stray | Spacecraft in normal mode w/ instruments & transmitter on (3.2 A umbilical current w/ drop in shunt current). |
| F#4 | Perm + Stray | Spacecraft in normal mode w/ instruments on (side B upper shunt current). |
| F#5 | Perm + Stray | Spacecraft in normal mode w/ instruments on (battery power -1.8 A). |
| F#6 | Perm + Stray | Spacecraft in normal mode w/ instruments on (battery charging 1.4 A). |
| F#7 | Perm + Stray | Post TQ spin coil on/off (battery charging). |
| F#8 | Perm + Stray | Post TQ spin coil on/off (reverse direction & half field strength). |
| F#9 | Perm + Stray | Post precession coil (vertical) on/off. |
| FSS/l | Perm + Stray | Spacecraft in normal mode w/ instruments & ACS sensors off (solar simulator illuminated). |

Note:	For stray field measurements, background taken with S/C in center of coils.

Table 3. FAST S/C Torque Coils Calibration Dipole Moments in A-m2

| Run | Mx | Mr | Mz | Mr |
| --- | --- | --- | --- | --- |
| F/C-1 | -8.4 | 10.6 | 3.2 | 13.9 |
| F/C-2 | -77.5 | 54.3 | -3.5 | 94.7 |
| F/C-3 | -20.7 | 78.1 | -3.0 | 80.8 |
| F/C-4 | -28.3 | 61.1 | -2.5 | 67.4 |
| F/C-5 | -17.0 | 36.6 | -1.5 | 40.4 |
| F/C-6 | -5.7 | 12.2 | -0.5 | 13.5 |
| F/C-7 | -0.1 | 0.1 | 0.0 | 0.1 |
| F/C-8 | 39.4 | -85.7 | 3.6 | 94.4 |
| F/C-9 | 33.7 | -73.4 | 3.1 | 80.8 |
| F/C-10 | 28.1 | -61.1 | 2.5 | 67.3 |
| P/C-11 | 16.8 | -36.6 | 1.5 | 40.3 |
| F/C-12 | 5.5 | -12.1 | 0.5 | 13.3 |
| F/C-13 | -0.1 | 0.1 | 0.0 | 0.1 |
| F/C-14.1 | -0.2 | 0.4 | 85.1 | 85.1 |
| F/C-15 | -0.1 | 0.5 | 72.6 | 72.6 |
| F/C-16 | 0.1 | 0.3 | 60.3 | 60.3 |
| F/C-17 | 0.0 | 0.1 | 37.7 | 37.7 |
| F/C-18 | -0.1 | 0.1 | 12.3 | 12.4 |
| F/C-19 | 0.0 | 0.2 | -0.6 | 0.6 |
| F/C-20 | 0.1 | -0.1 | -83.4 | 83.4 |
| F/C-21 | 0.2 | -0.2 | -71.0 | 71.0 |
| F/C-22 | 0.2 | -0.2 | -61.9 | 61.9 |
| F/C-23 | -0.1 | -0.1 | -36.2 | 36.2 |
| F/C-24 | 0.1 | 0.2 | -12.1 | 12.1 |
| F/C-25 | -0.3 | 0.2 | -0.5 | 0.6 |